15.4 MW wind farm at Satara District, Maharashtra in India

UNFCCC registration reference number: 0593

Date of registration of project: 24th December 2006

MONITORING REPORT

From – 01/04/2002 to 31/03/2007

Version 04

Date: 11th December 2007

1. Title of the project activity

15.4 MW wind farm at Satara District, Maharashtra.

PDD Version IV

Dated: October 18, 2006

Date of registration of project: 			24th December 2006

UNFCCC registration reference number: 	0593

2. Introduction

The project activity has been undertaken to harness the available wind power potential at Satara vis-à-vis development of local economy. The project activity has established 44 number of sophisticated, Wind Energy Generators (WEG) of 350 kW capacities aggregating to a total installed capacity of 15.4 MW in Satara District. The generated electricity is wheeled to the nearest grid sub-station through a 33 kV supply line and fed into the grid after stepping up to 132 kV.

Name of Investor

Ellora Time Limited

3. Methodology used

Project Type: I			Renewable energy project

Project Category: 		ACM0002, Grid connected renewable electricity generation

Reference: 	UNFCCC CDM website (http://cdm.unfccc.int/methodologies/PAmethodologies/approved.html)

The project activity is considered under “Zero emissions grid-connected electricity generation from renewable sources”, the project activity has a capacity more than 15 MW. Therefore, as per the scope of the project activity enlisted in the ‘list of sectoral scopes and related approved baseline and monitoring methodologies’, the project activity may principally be categorized in:

Scope Number 1

Sectoral Scope – Energy Industries (renewable/non-renewable sources).

4.  Abbreviations in the report

CDM – Clean Development Mechanism

PDD – Project Design Document

GHG – Green House Gas / Gases

IPCC – Intergovernmental Panel on Climate Change

SEB – State Electricity Board

MSEB – Maharashtra State Electricity Board

5.  General description of the project

5.1 Project Activity

The candidate CDM project will generate electricity from WEGs in Chikhali, Nivkhane and Bhambe villages in the district of Satara, Maharashtra. The generated electricity is sold to a third party; DGP Hinoday industries Limited. Since wind power is GHG emissions free, the electricity, which is fed into the regional electricity grid (through local grid sub-station) replaces the fossil fuel intensive generation supplying the electricity to the grid.

The project activity started in May 2000 and was estimated to generate approximately 25.96 million kWh per year, contributing an estimated reduction of 223,570 tCO2e over the ten year crediting period of the project from 2002-2012. This reduction is the result of displacement of fossil fuel fired power plants that would otherwise have delivered the electricity to Western Region Grid in the absence of the project activity.

5.2 Technical description of the project

Location of project activity

The wind electricity generators have been installed at three different sites in the Satara District as follows:

Site I: 	Village Chikhali: 	14 WEGs of capacity 0.350 MW

Site II: 	Village Nivkhane: 	15 WEGs of capacity 0.350 MW

Site III: 	Village Bhambe:	15 WEGs of capacity 0.350 MW

District:		Satara

State:			Maharashtra

Latitude:		170 27’ N

Longitude:		730 50’ E

Height			1100 m above mean sea level

Technology employed by the project activity

The project activity leads to the promotion of 350 kW Wind Electric Generators (WEGs) into the region, demonstrating the success of these small sized wind turbines, which feed the generated power into the nearest sub-station, thus increasing energy availability and improving quality of power under the service area of the substation. Hence the project leads to technological well being.

The technology to be employed, converts wind energy to electricity using a Wind Electric Generator.  The product that is employed is of 0.35 MW capacity.

Suzlon 350 kW Wind Turbine is a stall-regulated turbine with a three-bladed high efficiency rotor. The rotor is coupled to the generator through flange. This unique integrated power-train design incorporates torsionally flexible coupling to avoid problems of misalignment and vibration. The salient features of this technology are as follows:

- Specially designed for tropical climates and remote operations

- A High Coefficient of Power for ensuring optimum harnessing capacities

- Integrated power transmission mechanism to ensure high efficiency

- Carefully devised electrical system to withstand weak grid conditions

- Microprocessor-based fully automatic control system deploying user-friendly operation and remote monitoring

- Polyamide slide bearings for yawing

- Unique soft braking logic

- ISO-certified vendors confirm high quality components

- ISO 9001:2000 for Design, Development, Manufacture and Supply of Wind Turbines

- ISO 9001:2000 certification for Installation, Commissioning, Operation and Maintenance

- Type certification by Germanischer Lloyd, Germany

- Approved by the Ministry of Non-Conventional Energy Sources (MNES)

SAFETY SYSTEM

- Safety System consists of four levels of independent systems:

- Electronic sensing of faults by the computer for immediate action.

- Independent electrical circuitry to act when over-speed is detected.

- Hydraulic sensing and active device to prevent over-speeding.

- Mechanical flexible couplings with shearing studs.

Technology Transfer

No technology transfer from other countries is involved in this project activity.

5.3 Crediting Period

The project activity has sought a 10 year fixed crediting period starting from 1/4/2002.

- Monitoring methodology & Plan:

Approved consolidated monitoring methodology ACM0002/Version 06, 19th May 2006 Sector Scope: 1, “Consolidated monitoring methodology for zero-emissions grid connected electricity generation from renewable sources” by CDM Meth Panel.

The parameter to be monitored is Electricity Supplied to the Western Regional Grid of India (As per D2.1.1 of the PDD of the registered project activity)

| ID number | Data variable | Source of data | Data unit | Measured (m), calculated (c) or estimated (e) | Recording frequency | Proportion of data to be monitored | How will the data be archived? (electronic/ paper) | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A. | Generation of electricity from wind farm | Electronic meter installed by MSEB. | kWh | M | Monthly | 100% | Electronic & Paper | Three meters have been set up by the MSEB namely: Meter 1 (HTSC No. 4038) metering 14 * 0.35 kW WEGs, Meter 2 (HTSC N0. 4064) metering 15 * 0.35 kW WEGs, Meter 3 (HTSC No. 4237) Metering 15 * 0.35 kW WEGs. The total of these three meters gives the electricity generated by the 15.4 MW wind farm. The detailed data monitored has been presented in Annex 1. |
| 1B. | Generation of electricity from individual wind turbine | Electronic meter integrated within the wind turbine | kWh | M | Daily | 100% | Electronic | To monitor the generation & performance of individual turbines. Refer Annex 2 for details. |

6.1 Calibration / Maintenance of Measuring and Analytical Equipments

- The proposed project activity requires evacuation facilities for sale to grid and the evacuation facility is essentially maintained by the state power utility (MSEB).

- The electricity generation measurements are required by the utility and the investors to assess electricity sales revenue and / or wheeling charges.

- The project activity has therefore envisaged two independent measurements of generated electricity from the wind turbines.

- The primary recording of the electricity fed to the state utility grid will be carried out jointly at the incoming feeder of the state power utility (MSEB).

- The joint measurement is being carried out once in a month in presence of both parties (the developer’s representative and officials of the state power utility). This reading is then translated into a monthly generation report which is forwarded by MSEB to the project participant. Based on this report MSEB gives credit to the project proponent for the generation. The double check for the monthly generation reports is done by comparing them with the invoices raised by the project proponent towards the third party and MSEB (which buys the surplus electricity after consumption by third party at the end of the year).

- Metering equipment - Metering is carried out through electronic tri vector meters. The main meter is installed and owned by MSEB, whereas the check meters are owned by the project promoters and the metering equipments is maintained in accordance with electricity standards. The responsibility of calibration for both main & check meters is with the state government electricity utility (MSEB). The calibration and other maintenance of the meters is done by MSEB as per the standard procedures.

- There are 3 meters in the project. Two meters are of 0.5 accuracy class while one meter of 0.5 accuracy class recently got replaced by a meter of 0.2 accuracy class as per MSEDCL letter dated 28th December 2005 which recommends change of meters from class 0.5 to 0.2 for the entire state of Maharashtra in a phased manner. The documental proof for the same has been submitted to the verifier for reference.

- The secondary monitoring, which will provide a backup (fail-safe measure) in case the primary monitoring is not carried out, would be done at the individual WEGs. Each WEG is equipped with an integrated electronic meter. These meters are connected to the Central Monitoring Station (CMS) of the entire wind farm. The generation data of individual machine can be monitored as a real-time entity at CMS. The snapshot of generation on the last day of every calendar month will be kept as a record both in electronic as well as printed (paper) form.

6.2 Environmental Impact

- The electricity from wind electricity generator has no negative environmental impacts.

- As per the Schedule 1 of Ministry of Environment and Forests (Government of India) notification dated January 27, 1994, - 30 activities are required to undertake environmental impact assessment studies. The details of these activities are available at: http://envfor.nic.in/divisions/iass/notif/eia.htm

- The proposed project doesn’t fall under the list of activities requiring EIA as it will not involve any negative environmental impacts, because the WEGs installed for generation of power use wind (cleanest possible source of renewable energy).

7. GHG Emission Reduction Calculations

7.1 Project Activity Emissions: Nil

Emissions by sources of GHGs due to the project activity within the project boundary are zero since wind power is a GHG emission free source of energy.

7.2 Leakage: Nil

This is not applicable as the renewable energy technology used is not equipment transferred from another activity. Therefore, as per the simplified procedures for SSC project activities, no leakage calculation is required.

There is no alternate fuel which can generate electricity from the installed plant and machinery in absence of wind.

7.3 Baseline Emissions

As per the registered PDD, the wind power project uses the Combined Margin methodology as stated in ACM 0002.

The total baseline emissions BEy (tCO2/yr) = EGy * EFy

Where

BEy = Baseline emissions in year y (tCO2).

EGy (MWh/yr) = Electricity generated by the project in year y;

EFy = Combined margin emissions factor (Baseline emission factor) for the year y (tCO2/MWh).

The emission factor EFy of the Western Region Grid is calculated as a combination of the Operating Margin emission factor (EFOM, y) and the Build Margin emission factor (EFBM, y):

EFy = wOM EFOM,y + wBM .EFBM,y

Where the weights wOM and wBM, are 50% and 50%, and EFOM, y and EFBM, y, the Operating Margin and Build Margin emission factors respectively are calculated as given below.

Operating Margin Emission Factor

The Operating Margin is the weighted average emissions of all generating sources serving the Western Grid excluding hydro, geothermal, wind, low-cost biomass, nuclear and solar generation. It is derived from the following equation:

EFOM, simple, y =   Fi, j, y COEFi, j

GENj, y

where

Fi, j, y is the amount of fuel i (in a mass or volume unit) consumed by relevant power sources j in year(s) y,

j refers to the power sources delivering electricity to the grid, not including low-operating cost and must-run power plants, and including imports to the grid.

COEFi,j y is the CO2 emission coefficient of fuel i (tCO2 / mass or volume unit of the fuel), taking into account the carbon content of the fuels used by relevant power sources j and the percent oxidation of the fuel in year(s) y, and

GEN j, y is the electricity (MWh) delivered to the grid by source j.

The CO2 emission coefficient COEFi is obtained as

COEFi  = NCVi * EFCO2, i * OXIDi

where:

NCVi is the net calorific value (energy content) per mass or volume unit of a fuel i,

OXIDi is the oxidation factor of the fuel (see page 1.29 in the 1996 Revised IPCC Guidelines for default values),

EFCO2,i is the CO2 emission factor per unit of energy of the fuel i.

The Operating Margin for the purpose of this project has been updated based on ex-post monitoring, using the data vintage for the year in which the project generation occurs (as mentioned in the monitoring plan of the registered PDD).

Data of Operating Margin for the three financial years from 2002-03 to 2005-06 has been obtained from -

‘The CO2 Baseline Database for the Indian Power Sector’, Central Electricity Authority (CEA), Ministry of Power, Govt. of India.

Version 2

Dated: 21st June 2007

This database is prepared as per ACM0002 version 6.

Key baseline information is furnished as Annex 3. The detailed excel sheet is available at:

http://www.cea.nic.in/planning/c%20and%20e/Government%20of%20India%20website.htm

The EFOM,Y  for the year 2002-03 is 0.9830 tCO2/MWh

The EFOM,Y  for the year 2003-04 is 0.9903 tCO2/MWh

The EFOM,Y  for the year 2004-05 is 1.0120 tCO2/MWh

The EFOM,Y  for the year 2005-06 is 0.9934 tCO2/MWh

The EFOM,Y  for the year 2006-07 is 0.9934 tCO2/MWh

Build Margin Emission Factor

The Build Margin emission factor (EFBM, y) is calculated as the generation weighted average emission factor (tCO2/MWh) of a sample of power plants m, as follows:

EFBM, y =  Fi, m, y .COEFi, m

GENm, y

Where

Fi, m,y = quantity of fuel i used in plant m (kt/yr) in year y

COEFi, m = carbon emissions factor for fuel i in plant m (tCO2/kt), taking into account the carbon content of the fuels by power sources and the percent oxidation of the fuel

GENm, y = annual generation from plant j (MWh/yr) in year y

The build margin emission factor has also been calculated ex-post based on the values available at the time of monitoring of project generation (as mentioned in the monitoring plan of the registered PDD).

Baseline Emission Factor (EFy)

The emission factor EFy of the Western Region Grid is calculated as a combination of the ex-post updated Operating Margin emission factor (EFOM, y) and the ex-post updated Build Margin emission factor (EFBM, y):

EFy = wOM EFOM, y + wBM .EFBM, y

Where the weights wOM and wBM, are 50% and 50%, and EFOM, y and EFBM, y are the Operating Margin and Build Margin emission factors respectively are calculated as given below (as per the registered PDD).

| Year | Operating Margin Emission Factor (tCO2/MWh) | Built Margin Emission Factor (tCO2/MWh) | Baseline Emission Factor (tCO2/MWh) |
| --- | --- | --- | --- |
| 2002-03 | 0.9830 | 0.7700 | 0.8765 |
| 2003-04 | 0.9903 | 0.7700 | 0.8802 |
| 2004-05 | 1.0120 | 0.7700 | 0.8910 |
| 2005-06 | 0.9934 | 0.6300 | 0.8117 |
| 2006-07 | 0.9934 | 0.6300 | 0.8117 |

The baseline emissions are estimated as the product of the electricity generated by the project activity and the Emission factor of the regional electricity grid as calculated above.

| Name of Item | GEN |
| --- | --- |
| Description | Electricity Quantity – Electricity fed into the western grid |
| Crediting Period | 01/04/2002 – 31/03/2012 |
| Method of monitoring | Measured using energy meter |
| Recording frequency | Continuous |
| Reporting frequency | Monthly (Jointly by SEB and representative of the investor) |
| Background data | Joint meter reading available at the site |
| Archiving mode | Electronic & paper |

The project started on 01/05/2000 and the crediting period starts from 01/04/2002.

Report has been prepared by:

SenergyGlobal Private Limited,

9th Floor, Eros Corporate Tower, Nehru Place,

New Delhi - 110 019. India

Tel.: +91- 11- 4650 6028, 4180 5501

Fax: +91- 11- 4180 5504

E.mail: mail@senergyglobal.com

Url: www.senergyglobal.com

Annex 1

1 A – Generation of electricity from wind farm

Three meters have been set up by the MSEB namely:

Meter 1 (HTSC No. 4038) metering 14 *  0.35 kW WEGs

Meter 2 (HTSC N0. 4064) metering 15 *  0.35 kW WEGs

Meter 3 (HTSC No. 4237) Metering 15 *  0.35 kW WEGs

The total of these three meters gives the electricity generated by the 15.4 MW wind farm.

| Crediting period | Net electricity generated (in kWh) |  |  | Total electricity generated (in kWh) | Baseline Emission Factor (tCO2e/MWh) | Total emissions reductions (tCO2e) |
| --- | --- | --- | --- | --- | --- | --- |
|  | MSEB Meter 1 (4038) (14*350) kW | MSEB Meter 2 (4064) (15*350) kW | MSEB Meter 3 (4237) (15*350) kW |  |  |  |
| 01/04/2002 - 30/04/2002 | 458177 | 500945 | 196147 | 1155269 | 0.8765 | 1012.62 |
| 01/05/2002 - 31/05/2002 | 909702 | 904554 | 525809 | 2340065 | 0.8765 | 2051.12 |
| 01/06/2002 - 30/06/2002 | 896248 | 870862 | 1121229 | 2888339 | 0.8765 | 2531.70 |
| 01/07/2002 - 31/07/2002 | 1401449 | 1484446 | 1593562 | 4479457 | 0.8765 | 3926.35 |
| 01/08/2002 - 31/08/2002 | 1518929 | 1555113 | 1422569 | 4496611 | 0.8765 | 3941.39 |
| 01/09/2002 - 30/09/2002 | 707304 | 730562 | 857900 | 2295766 | 0.8765 | 2012.29 |
| 01/10/2002 - 31/10/2002 | 189821 | 183443 | 168010 | 541274 | 0.8765 | 474.44 |
| 01/11/2002 - 31/12/2002 | 242093 | 211348 | 205318 | 658759 | 0.8765 | 577.42 |
| 01/01/2003 - 31/01/2003 | 294323 | 250393 | 251212 | 795928 | 0.8765 | 697.65 |
| 01/02/2003 - 28/02/2003 | 252806 | 274333 | 286899 | 814038 | 0.8765 | 713.52 |
| 01/03/2003 - 31/03/2003 | 306256 | 321795 | 348802 | 976853 | 0.8765 | 856.24 |
| 01/04/2003 - 30/04/2003 | 441846 | 435869 | 692631 | 1570346 | 0.8802 | 1382.18 |
| 01/05/2003 - 31/05/2003 | 761719 | 565990 | 971061 | 2298770 | 0.8802 | 2023.32 |
| 01/06/2003 - 30/06/2003 | 1001289 | 1302792 | 1114671 | 3418752 | 0.8802 | 3009.09 |
| 01/07/2003 - 31/07/2003 | 1534352 | 1447053 | 806346 | 3787751 | 0.8802 | 3333.88 |
| 01/08/2003 - 31/08/2003 | 1531960 | 1503924 | 1497176 | 4533060 | 0.8802 | 3989.88 |
| 01/09/2003 - 30/09/2003 | 1090431 | 1018897 | 907870 | 3017198 | 0.8802 | 2655.66 |
| 01/10/2003 - 31/10/2003 | 324076 | 350080 | 427263 | 1101419 | 0.8802 | 969.44 |
| 01/11/2003 - 30/11/2003 | 208428 | 196458 | 179874 | 584760 | 0.8802 | 514.69 |
| 01/12/2003 - 31/12/2003 | 270990 | 228120 | 223008 | 722118 | 0.8802 | 635.59 |
| 01/01/2004 - 31/01/2004 | 187722 | 154104 | 161298 | 503124 | 0.8802 | 442.84 |
| 01/02/2004 - 29/02/2004 | 288030 | 311934 | 253560 | 853524 | 0.8802 | 751.25 |
| 01/03/2004 - 31/03/2004 | 262260 | 263460 | 483900 | 1009620 | 0.8802 | 888.64 |
| 01/04/2004 - 30/04/2004 | 858540 | 899460 | 786240 | 2544240 | 0.8910 | 2266.92 |
| 01/05/2004 - 31/05/2004 | 777120 | 800580 | 766500 | 2344200 | 0.8910 | 2088.68 |
| 01/06/2004 - 30/06/2004 | 1146840 | 1359660 | 1424460 | 3930960 | 0.8910 | 3502.49 |
| 01/07/2004 - 31/07/2004 | 1775880 | 1686126 | 1302420 | 4764426 | 0.8910 | 4245.11 |
| 01/08/2004 - 31/08/2004 | 1475040 | 1685994 | 1673580 | 4834614 | 0.8910 | 4307.65 |
| 01/09/2004 - 30/09/2004 | 995280 | 1012440 | 885720 | 2893440 | 0.8910 | 2578.06 |
| 01/10/2004 - 31/10/2004 | 236340 | 277920 | 236760 | 751020 | 0.8910 | 669.16 |
| 01/11/2004 - 30/11/2004 | 434880 | 323340 | 341460 | 1099680 | 0.8910 | 979.82 |
| 01/12/2004 - 31/12/2004 | 676200 | 554820 | 504360 | 1735380 | 0.8910 | 1546.23 |
| 01/01/2005 - 31/01/2005 | 128760 | 125160 | 148260 | 402180 | 0.8910 | 358.34 |
| 01/02/2005 - 28/02/2005 | 162720 | 175680 | 356940 | 695340 | 0.8910 | 619.55 |
| 01/03/2005 - 31/03/2005 | 397980 | 456000 | 337860 | 1191840 | 0.8910 | 1061.93 |
| 01/04/2005 - 30/04/2005 | 423900 | 442320 | 364380 | 1230600 | 0.8117 | 998.86 |
| 01/05/2005 - 31/05/2005 | 388620 | 462480 | 575220 | 1426320 | 0.8117 | 1157.73 |
| 01/06/2005 - 30/06/2005 | 932100 | 1182912 | 952200 | 3067212 | 0.8117 | 2489.62 |
| 01/07/2005 - 31/07/2005 | 961560 | 1422330 | 1596762 | 3980652 | 0.8117 | 3231.05 |
| 01/08/2005 - 31/08/2005 | 908628 | 1517754 | 2125632 | 4552014 | 0.8117 | 3694.82 |
| 01/09/2005 - 30/09/2005 | 725106 | 650592 | 1114344 | 2490042 | 0.8117 | 2021.14 |
| 01/10/2005 - 31/10/2005 | 664800 | 651162 | 272352 | 1588314 | 0.8117 | 1289.22 |
| 01/11/2005 - 30/11/2005 | 198300 | 179526 | 254652 | 632478 | 0.8117 | 513.37 |
| 01/12/2005 - 31/12/2005 | 362742 | 300882 | 272238 | 935862 | 0.8117 | 759.63 |
| 01/01/2006 - 31/01/2006 | 122808 | 144048 | 120204 | 387060 | 0.8117 | 314.17 |
| 01/02/2006 - 28/02/2006 | 222108 | 166710 | 224226 | 613044 | 0.8117 | 497.60 |
| 01/03/2006 - 31/03/2006 | 1054746 | 427482 | 355074 | 1837302 | 0.8117 | 1491.32 |
| 01/04/2006 - 30/04/2006 | 440608 | 473787 | 439007 | 1353402 | 0.8117 | 1098.54 |
| 01/05/2006 - 31/05/2006 | 696203 | 752251 | 556856 | 2005310 | 0.8117 | 1627.69 |
| 01/06/2006 - 30/06/2006 | 1072549 | 932290 | 1114845 | 3119684 | 0.8117 | 2532.21 |
| 01/07/2006 - 31/07/2006 | 1002391 | 1251845 | 1410758 | 3664994 | 0.8117 | 2974.83 |
| 01/08/2006 - 31/08/2006 | 1587309 | 1801715 | 1880793 | 5269817 | 0.8117 | 4277.45 |
| 01/09/2006 - 30/09/2006 | 500972 | 564339 | 481956 | 1547267 | 0.8117 | 1255.90 |
| 01/10/2006 - 31/10/2006 | 316743 | 293396 | 338131 | 948270 | 0.8117 | 769.70 |
| 01/11/2006 - 30/11/2006 | 306309 | 164839 | 254108 | 725256 | 0.8117 | 588.68 |
| 01/12/2006 - 31/12/2006 | 318445 | 283280 | 174576 | 776301 | 0.8117 | 630.11 |
| 01/01/2007 - 31/01/2007 | 86306 | 103492 | 203045 | 392843 | 0.8117 | 318.87 |
| 01/02/2007 - 28/02/2007 | 217224 | 146626 | 249978 | 613828 | 0.8117 | 498.24 |
| 01/03/2007 - 31/03/2007 | 358348 | 293408 | 295751 | 947507 | 0.8117 | 769.08 |
|  |  |  |  |  | TOTAL | 99,414 |

Yearly Emission Reductions

| S.No | Crediting period | Emissions reductions |
| --- | --- | --- |
| 1 | 01/04/2002 - 31/03/2003 | 18794.75 |
| 2 | 01/04/2003 - 31/03/2004 | 20596.44 |
| 3 | 01/04/2004 - 31/03/2005 | 24223.93 |
| 4 | 01/04/2005 - 31/03/2006 | 18458.52 |
| 5 | 01/04/2006 - 31/03/2007 | 17341.29 |
|  | TOTAL CERs | 99,414 |

Annex 2

I B – Generation of electricity from individual wind turbine

Each WEG is equipped with an integrated electronic meter. These meters are connected to the Central Monitoring Station (CMS) of the entire wind farm. The generation data of individual machine can be monitored as a real-time entity at CMS. The snapshot of generation will be kept as a record in electronic form and this is maintained at the Satara site office. The detailed values of electricity generated from each WEG have not been presented in this report as the values do not take into account the transmission losses incurred and hence would not present a true picture of the net electricity delivered to grid. However, this data is available at the site office in Satara, and the same will be made available to the verifier during verification.

Annex 3

| CENTRAL ELECTRICITY AUTHORITY: CO2 BASELINE DATABASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VERSION |  |  | 2.0 |  |  |  |  |  |  |  |  |  |  |  |
| DATE |  |  | 21 June 2007 |  |  |  |  |  |  |  |  |  |  |  |
| BASELINE METHODOLOGY |  |  | ACM0002 / Ver 06 |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION FACTORS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Weighted Average Emission Rate (tCO2/MWh) (excl. Imports) |  |  |  |  |  |  |  | Weighted Average Emission Rate (tCO2/MWh) (incl. Imports) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 0.7237 | 0.7265 | 0.7379 | 0.7095 | 0.7134 | 0.7137 |  | North | 0.7237 | 0.7265 | 0.7401 | 0.7098 | 0.7203 | 0.7240 |
| East | 1.0876 | 1.0573 | 1.1128 | 1.1035 | 1.0768 | 1.0756 |  | East | 1.0876 | 1.0320 | 1.0852 | 1.0758 | 1.0534 | 1.0473 |
| South | 0.7347 | 0.7451 | 0.8231 | 0.8417 | 0.7837 | 0.7353 |  | South | 0.7446 | 0.7478 | 0.8235 | 0.8417 | 0.7843 | 0.7355 |
| West | 0.8988 | 0.9247 | 0.9034 | 0.9020 | 0.9242 | 0.8746 |  | West | 0.8984 | 0.9242 | 0.9026 | 0.9009 | 0.9234 | 0.8829 |
| North-East | 0.4239 | 0.4140 | 0.4020 | 0.4281 | 0.3180 | 0.3309 |  | North-East | 0.4239 | 0.4140 | 0.4020 | 0.4281 | 0.4799 | 0.3309 |
| India | 0.8216 | 0.8309 | 0.8546 | 0.8507 | 0.8423 | 0.8170 |  | India | 0.8191 | 0.8284 | 0.8520 | 0.8479 | 0.8397 | 0.8145 |
| Simple Operating Margin (tCO2/MWh) (excl. Imports) |  |  |  |  |  |  |  | Simple Operating Margin (tCO2/MWh) (incl. Imports) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 0.9768 | 0.9778 | 0.9988 | 0.9870 | 0.9745 | 0.9933 |  | North | 0.9768 | 0.9778 | 0.9996 | 0.9871 | 0.9758 | 0.9946 |
| East | 1.2198 | 1.2210 | 1.2025 | 1.2303 | 1.2037 | 1.1585 |  | East | 1.2198 | 1.1874 | 1.1703 | 1.1959 | 1.1745 | 1.1255 |
| South | 1.0219 | 1.0001 | 1.0071 | 1.0041 | 0.9997 | 1.0073 |  | South | 1.0292 | 1.0001 | 1.0065 | 1.0041 | 0.9999 | 1.0073 |
| West | 0.9791 | 1.0102 | 0.9845 | 0.9922 | 1.0132 | 0.9936 |  | West | 0.9784 | 1.0094 | 0.9830 | 0.9903 | 1.0120 | 0.9934 |
| North-East | 0.7335 | 0.7101 | 0.7418 | 0.7366 | 0.7143 | 0.6994 |  | North-East | 0.7335 | 0.7101 | 0.7418 | 0.7366 | 0.8402 | 0.6994 |
| India | 1.0169 | 1.0239 | 1.0207 | 1.0259 | 1.0274 | 1.0228 |  | India | 1.0132 | 1.0202 | 1.0170 | 1.0217 | 1.0234 | 1.0189 |
| Build Margin (tCO2/MWh) (excl. Imports) |  |  |  |  |  |  |  | Build Margin (tCO2/MWh) (not adjusted for imports) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North |  |  |  |  | 0.5336 | 0.6005 |  | North |  |  |  |  | 0.5336 | 0.6005 |
| East |  |  |  |  | 0.9043 | 0.9672 |  | East |  |  |  |  | 0.9043 | 0.9672 |
| South |  |  |  |  | 0.7089 | 0.7113 |  | South |  |  |  |  | 0.7089 | 0.7113 |
| West |  |  |  |  | 0.7700 | 0.6300 |  | West |  |  |  |  | 0.7700 | 0.6300 |
| North-East |  |  |  |  | 0.1456 | 0.1489 |  | North-East |  |  |  |  | 0.1456 | 0.1489 |
| India |  |  |  |  | 0.6953 | 0.6841 |  | India |  |  |  |  | 0.6953 | 0.6841 |
| Combined Margin (tCO2/MWh) (excl. Imports) |  |  |  |  |  |  |  | Combined Margin in tCO2/MWh (incl. Imports) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 0.7552 | 0.7557 | 0.7662 | 0.7603 | 0.7540 | 0.7969 |  | North | 0.7552 | 0.7557 | 0.7666 | 0.7603 | 0.7547 | 0.7976 |
| East | 1.0620 | 1.0626 | 1.0534 | 1.0673 | 1.0540 | 1.0628 |  | East | 1.0620 | 1.0458 | 1.0373 | 1.0501 | 1.0394 | 1.0463 |
| South | 0.8654 | 0.8545 | 0.8580 | 0.8565 | 0.8543 | 0.8593 |  | South | 0.8690 | 0.8545 | 0.8577 | 0.8565 | 0.8544 | 0.8593 |
| West | 0.8746 | 0.8901 | 0.8773 | 0.8811 | 0.8916 | 0.8118 |  | West | 0.8742 | 0.8897 | 0.8765 | 0.8802 | 0.8910 | 0.8117 |
| North-East | 0.4395 | 0.4278 | 0.4437 | 0.4411 | 0.4299 | 0.4242 |  | North-East | 0.4395 | 0.4278 | 0.4437 | 0.4411 | 0.4929 | 0.4242 |
| India | 0.8561 | 0.8596 | 0.8580 | 0.8606 | 0.8614 | 0.8534 |  | India | 0.8543 | 0.8578 | 0.8562 | 0.8585 | 0.8594 | 0.8515 |
| GENERATION DATA |  |  |  |  |  |  |  | EMISSION DATA |  |  |  |  |  |  |
| Gross Generation Total (GWh) |  |  |  |  |  |  |  | Absolute Emissions Total (tCO2) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 144,292 | 151,185 | 155,385 | 165,735 | 168,438 | 179,751 |  | North | 97,866,565 | 102,743,113 | 106,808,582 | 109,996,544 | 112,212,597 | 120,056,079 |
| East | 58,936 | 64,048 | 66,257 | 75,374 | 85,776 | 93,902 |  | East | 58,026,488 | 61,427,499 | 66,593,200 | 75,512,010 | 83,956,860 | 92,517,515 |
| South | 129,035 | 131,902 | 136,916 | 138,517 | 144,086 | 147,355 |  | South | 89,019,263 | 92,112,060 | 105,187,726 | 108,049,156 | 105,539,862 | 101,712,149 |
| West | 162,329 | 165,805 | 177,399 | 172,682 | 183,955 | 188,606 |  | West | 135,192,153 | 141,597,621 | 148,557,341 | 144,127,175 | 157,781,065 | 153,933,199 |
| North-East | 5,319 | 5,332 | 5,808 | 5,867 | 7,883 | 7,778 |  | North-East | 2,202,108 | 2,158,348 | 2,280,049 | 2,462,796 | 2,468,463 | 2,532,819 |
| India | 499,911 | 518,272 | 541,764 | 558,175 | 590,138 | 617,392 |  | India | 382,306,576 | 400,038,640 | 429,426,898 | 440,147,681 | 461,958,846 | 470,751,761 |
| Net Generation Total (GWh) |  |  |  |  |  |  |  | Absolute Emissions OM (tCO2) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 135,230 | 141,415 | 144,743 | 155,043 | 157,291 | 168,206 |  | North | 97,866,565 | 102,743,113 | 106,808,582 | 109,996,544 | 112,212,597 | 120,056,079 |
| East | 53,350 | 58,097 | 59,841 | 68,428 | 77,968 | 86,014 |  | East | 58,026,488 | 61,427,499 | 66,593,200 | 75,512,010 | 83,956,860 | 92,517,515 |
| South | 121,158 | 123,630 | 127,789 | 128,373 | 134,676 | 138,329 |  | South | 89,019,263 | 92,112,060 | 105,187,726 | 108,049,156 | 105,539,862 | 101,712,149 |
| West | 150,412 | 153,125 | 164,448 | 159,780 | 170,726 | 176,003 |  | West | 135,192,153 | 141,597,621 | 148,557,341 | 144,127,175 | 157,781,065 | 153,933,199 |
| North-East | 5,195 | 5,213 | 5,671 | 5,752 | 7,762 | 7,655 |  | North-East | 2,202,108 | 2,158,348 | 2,280,049 | 2,462,796 | 2,468,463 | 2,532,819 |
| India | 465,345 | 481,479 | 502,492 | 517,376 | 548,423 | 576,206 |  | India | 382,306,576 | 400,038,640 | 429,426,898 | 440,147,681 | 461,958,846 | 470,751,761 |
| Share of Must-Run (Hydro/Nuclear) (% of Net Generation) |  |  |  |  |  |  |  | Absolute Emissions BM (tCO2) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 25.9% | 25.7% | 26.1% | 28.1% | 26.8% | 28.1% |  | North |  |  |  |  | 17,108,583 | 20,622,114 |
| East | 10.8% | 13.4% | 7.5% | 10.3% | 10.5% | 7.2% |  | East |  |  |  |  | 14,303,611 | 16,990,438 |
| South | 28.1% | 25.5% | 18.3% | 16.2% | 21.6% | 27.0% |  | South |  |  |  |  | 19,839,024 | 20,029,713 |
| West | 8.2% | 8.5% | 8.2% | 9.1% | 8.8% | 12.0% |  | West |  |  |  |  | 27,148,870 | 22,318,133 |
| North-East | 42.2% | 41.7% | 45.8% | 41.9% | 55.5% | 52.7% |  | North-East |  |  |  |  | 299,121 | 266,981 |
| India | 19.2% | 18.9% | 16.3% | 17.1% | 18.0% | 20.1% |  | India |  |  |  |  | 78,699,210 | 80,227,378 |
| Net Generation in Operating Margin (GWh) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  |  |  |  |  |  |  |
| North | 100,189 | 105,076 | 106,942 | 111,450 | 115,151 | 120,869 |  |  |  |  |  |  |  |  |
| East | 47,570 | 50,308 | 55,377 | 61,378 | 69,746 | 79,863 |  |  |  |  |  |  |  |  |
| South | 87,114 | 92,103 | 104,449 | 107,603 | 105,568 | 100,978 |  |  |  |  |  |  |  |  |
| West | 138,071 | 140,173 | 150,889 | 145,264 | 155,731 | 154,918 |  |  |  |  |  |  |  |  |
| North-East | 3,002 | 3,039 | 3,074 | 3,343 | 3,456 | 3,621 |  |  |  |  |  |  |  |  |
| India | 375,947 | 390,700 | 420,730 | 429,040 | 449,653 | 460,249 |  | IMPORT DATA |  |  |  |  |  |  |
| 20% of Net Generation (GWh) |  |  |  |  |  |  |  | Net Imports (GWh) - Net exporting grids are set to zero |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North | 27,046 | 28,283 | 28,949 | 31,009 | 31,458 | 33,641 |  | North | 0 | 0 | 0 | 0 | 3,616 | 5,748 |
| East | 10,670 | 11,619 | 11,968 | 13,686 | 15,594 | 17,203 |  | East | 489 | 555 | 357 | 1,689 | 0 | 0 |
| South | 24,232 | 24,726 | 25,558 | 25,675 | 26,935 | 27,666 |  | South | 1,162 | 1,357 | 518 | 0 | 0 | 0 |
| West | 30,082 | 30,625 | 32,890 | 31,956 | 34,145 | 35,201 |  | West | 321 | 0 | 797 | 962 | 285 | 11,982 |
| North-East | 1,039 | 1,043 | 1,134 | 1,150 | 1,552 | 1,531 |  | North-East | 0 | 0 | 0 | 0 | 2,099 | 0 |
| India | 93,069 | 96,296 | 100,498 | 103,475 | 109,685 | 115,241 |  |  |  |  |  |  |  |  |
| Net Generation in Build Margin (GWh) |  |  |  |  |  |  |  | Share of Net Imports (% of Net Generation) |  |  |  |  |  |  |
|  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |  |  | 2000-01 | 2001-02 | 2002-03 | 2003-04 | 2004-05 | 2005-06 |
| North |  |  |  |  | 32,064 | 34,340 |  | North | 0.0% | 0.0% | 0.0% | 0.0% | 2.3% | 3.4% |
| East |  |  |  |  | 15,818 | 17,567 |  | East | 0.9% | 1.0% | 0.6% | 2.5% | 0.0% | 0.0% |
| South |  |  |  |  | 27,987 | 28,158 |  | South | 1.0% | 1.1% | 0.4% | 0.0% | 0.0% | 0.0% |
| West |  |  |  |  | 35,257 | 35,425 |  | West | 0.2% | 0.0% | 0.5% | 0.6% | 0.2% | 6.8% |
| North-East |  |  |  |  | 2,055 | 1,793 |  | North-East | 0.0% | 0.0% | 0.0% | 0.0% | 27.0% | 0% |
| India |  |  |  |  | 113,181 | 117,283 |  |  |  |  |  |  |  |  |

----------------

Since data for the year 2006-07 is not available in the CEA database, the OM value of the year 2005-06 has been taken for the year 2006-07

The number of CERs have reduced due to ex-post calculations of both EFOM and EFBM in the current version of the Monitoring report (Version 3), which is based on the monitoring plan of the registered PDD; as compared to the calculations in the earlier version of Monitoring report (Version 1), which calculated ex-post EFOM but ex-ante EFBM, as was mentioned in Section B.2 of the registered PDD.