- CLEAN DEVELOPMENT MECHANISM

- PROJECT DESIGN DOCUMENT FORM (CDM-PDD)
Version 03 - in effect as of: 28 July 2006

# CONTENTS

A. 	General description of project activity

B. 	Application of a baseline and monitoring methodology

C. 	Duration of the project activity / crediting period

D. 	Environmental impacts

E. 	Stakeholders’ comments

## Annexes

Annex 1:  Contact information on participants in the project activity

Annex 2:  Information regarding public funding

Annex 3:  Baseline information

Annex 4:  Monitoring plan

SECTION A. 	General description of project activity

A.1 	Title of the project activity:

Yima Coal Industry (Group) Co., Ltd. CMM utilization project

Version 2.7

Completed on 10/06/2008

A.2.	Description of the project activity:

Yima Coal Industry Group (YCIG) is a state-owned industry group located in the western part of Henan Province, about 160 km from the capital Zhengzhou. Its annual coal production is expected to reach about 20 million tons by the end of 2007. During its mining activities, methane gas is released from the coal seam and needs to be discharged in order to comply with Chinese mining safety regulations. About 40% of gas is released in the form of coalmine methane (CMM), which is drained directly from YCIG’s coal seams. About 60% is released in the form of ventilation air methane (VAM), and is discharged through the mines’ ventilation shafts.

The project will include six coalmines and will utilize this available gas for the production of both electricity and heat. This will be done in two ways. Firstly, a set of 22 gas engines will be installed at the following mines belonging to YCIG: Qianqiu, Gengcun, Xinan, Xinyi, Yian. Total capacity will be 11 MW. The power produced will be used for YCIG’s own consumption, replacing electricity that would otherwise be consumed from the Central China Electricity Grid. Waste heat from some of the engines will also be utilized to supply hot water to nearby mining facilities. Secondly, a newly developed methane oxidation technology (known as the Vocsidizer) will be installed at Mengjin mine to destroy ventilation air methane (VAM) with low CH4 concentrations (below 1%). This technology will also produce thermal energy that can be substituted for coal-based heat. The following table shows the project’s total gas utilization and expected energy output.

Table 1: Estimated gas utilization and energy production of project activity

| CMM utilization (m3 CH4/year) | 15,555,556 |
| --- | --- |
| VAM utilization (m3 CH4/year) | 3,600,000 |
| Electricity production (MWh/year) | 53,900 |
| Heat production (GJ/year) | 36,303 |

Total emission reductions over a ten-year crediting period are estimated at 2,661,786 tCO2e.

The project will contribute to China’s sustainable development in the following ways:

- Avoid emissions of various pollutants such as SO2 and nitric oxides (NOx) as a result of the replacement of coal boilers by CMM-based heat.

- Decrease the need for additional power production in the China Central Grid because of power substitution taking place from the project.

- Introduce a new VAM oxidation technology to China with strong replicability effects.

- Create employment for the operation of the above-described technologies.

A.3. 	Project participants:

| Party Involved | Private or Public Entity | Project Participant |
| --- | --- | --- |
| People’s Republic of China (host) | Yima Coal Industry Group Co., Ltd. (public) | No |
| United Kingdom | EDF TRADING Limited | No |

A.4. 	Technical description of the project activity:

A.4.1.  Location of the project activity:

>>

A.4.1.1.		Host Party(ies):

People’s Republic of China

A.4.1.2.		Region/State/Province etc.:

Henan Province

A.4.1.3.		City/Town/Community etc:

Yima City

A.4.1.4.		Detail of physical location, including information allowing the unique identification of this project activity (maximum one page):

YCIG is located in Yima City, approximately 160 km west of Zhengzhou, the capital of Henan province. The project is carried out at Qianqiu, Gengcun, Xinan, Xinyi, Yian, and Mengjin mines. The geographic coordinates of these mines are as follows:

| Mine | Geographic coordinates |
| --- | --- |
| Qianqiu | 111° 45′ 11″ ～ 111° 51′ 05″ 34° 41′ 36″ ～ 34° 43′ 16″ |
| Gengcun | 111° 44′ 30″ ～ 111° 47′ 0″ 34° 42′ 30″ ～ 34° 44′ 0″ |
| Xinan | 112° 02′ 30″ ~ 112° 14′ 00″ 34° 45′ 00″ ~ 34° 54′ 30″ |
| Xinyi | 112° 04′ 09″ ～ 112° 11′ 58″ 34° 43′ 10″ ～ 34° 49′ 12″ |
| Yian | 112° 09’ 43″ ~ 112° 14’ 45″ 34° 47’ 30″ ~ 34° 51’ 41″ |
| Mengjin | 112° 11' 37" ~ 112° 19' 26" 34° 50' 01" ~ 34° 54' 36" |

Figure 1: Henan Province

People’s Republic of China

Henan Province

Yima City

Figure 2: Map of YCIG Coalfield

A.4.2.  Category(ies) of project activity:

8: 	Mining/mineral production

10. 	Fugitive emissions from fuels (solid, oil and gas)

A.4.3.  Technology to be employed by the project activity:

Gas Engines: Gas engines are Chinese-manufactured 500FG1-3RW models from Shengdong Oilfield Shengli Power Machinery Co., Inc. Engines and gas transport equipment are specifically designed to permit the safe combustion of low-concentration CMM. Engine capacity is 500kW and actual engine performance is estimated at 70-85% for electricity production (350-425kW). Other ancillary equipment associated with the gas engines includes cooling water towers, water pumps and heat exchangers.

Vocsidizers: One Vocsidizer unit will be installed at Mengjin mine. Each unit can process up to 125,000 m3 of VAM per hour, with methane concentrations typically ranging from 0.2% to 0.8%. The gas passes through a ceramic bed inside the Vocsidizer, and is gradually heated until the methane is oxidized, producing CO2 and thermal energy. Through embedded tubes, this energy can be recovered in the form of hot water. The hot water will be substituted for thermal energy from onsite coal boilers. The project will begin with the installation of one Vocsidizer module, and after a successful test period, additional modules may be added in a future phase of the project.

If the methane concentration in the VAM entering the Vocsidizer is considered to be too low to ensure a reliable performance of the Vocsidizer, additional CMM from a nearby extraction station may be added to the VAM stream. This would raise its methane concentration and make the methane oxidation more stable (mechanisms exist whereby CMM can be added automatically whenever the methane concentration of the VAM dips below a certain point). Any CMM added would otherwise have been vented into the atmosphere, so its addition to the VAM stream would not affect the performance of the rest of the project.

Table 2: Installed Equipment

| Mine | Equipment installed |
| --- | --- |
| Qianqiu | 4 x 500kW gas engines |
| Gengcun | 6 x 500kW gas engines |
| Xinan | 4 x 500kW gas engines |
| Xinyi | 4 x 500kW gas engines |
| Yian | 4 x 500kW gas engines |
| Mengjin | 1 Vocsidizer |

The project will promote the transfer of an advanced foreign technology to oxidize low methane concentration ventilation air to China and encourage its application in other projects.

A.4.4	Estimated amount of emission reductions over the chosen crediting period:

Table 3: Estimated Emission Reductions throughout crediting period

| Years | Annual estimation of emission reductions in tCO2e |
| --- | --- |
| 2008 | 68,732 |
| 2009 | 282,879 |
| 2010 | 282,879 |
| 2011 | 282,879 |
| 2012 | 282,879 |
| 2013 | 282,879 |
| 2014 | 282,879 |
| 2015 | 282,879 |
| 2016 | 282,879 |
| 2017 | 282,879 |
| 2018 | 47,146 |
| Total estimated reductions (tCO2e) | 2,661,786 |
| Total number of crediting years | 10 years |
| Annual average of emission reductions over the crediting period (tCO2e) | 266,179 |

A.4.5.  Public funding of the project activity:

No public funding is involved in the financing of this project.

SECTION B. 	Application of a baseline and monitoring methodology

B.1.	Title and reference of the approved baseline and monitoring methodology applied to the project activity:

The project uses the consolidated methodology ACM0008 for “coal bed methane and coal mine methane capture and use for power (electrical or motive) and heat and/or destruction by flaring” (Version 03). Based on ACM0008, the project also employs ACM0002 “Consolidated baseline methodology for grid-connected electricity generation from renewable sources” Version 06, to calculate the emission factor of the Central China Electricity Grid. Lastly, the project uses the “Tool for the demonstration and assessment of additionality” Version 03 for its demonstration of additionality.

Both ACM0008 and ACM0002 are available at:

http://cdm.unfccc.int/UserManagement/FileStorage/CDMWF_AM_UAHWQGQYAJVSO577503JFMY2OWJJMG

The Additionality Tool is available at:

http://cdm.unfccc.int/methodologies/PAmethodologies/AdditionalityTools/Additionality_tool.pdf

B.2	Justification of the choice of the methodology and why it is applicable to the project activity:

The following tables demonstrate that methodology ACM0008 is applicable to the project activity.

Table 4: CMM extraction techniques eligible under ACM0008

| CMM extraction techniques eligible under ACM0008 | Activities of Project |
| --- | --- |
| Underground boreholes to capture pre-mining CMM | Included. |
| Surface goaf wells, underground boreholes, gas drainage galleries or other goaf gas capture techniques | May be included in future. YCIG is working on improving and implementing goaf-capture techniques. |
| Ventilation CMM that would normally be vented | Included. |

Table 5: CMM utilization techniques eligible under ACM0008

| CMM utilization techniques eligible under ACM0008 | Activities of Project |
| --- | --- |
| Methane is captured and destroyed through utilization to produce electricity and thermal energy | Included. CMM is extracted and captured for power and heat generation in gas engines. A share of VAM is also destroyed for heat production. Emission reductions are claimed for displacing both electricity and heat. |
| The remaining share of methane, to be diluted for safety reasons, may still be vented | VAM that is not fed to the Vocsidizer for heat generation will be vented. |
| All the CBM or CMM captured by the project should either be used or destroyed, and cannot be vented. | All of the captured CMM is used to generate electricity and heat. No CBM is used in the project. |

Table 6: Other eligibility criteria of ACM0008

| Other eligibility criteria of ACM0008 | Activities of Project |
| --- | --- |
| The project cannot operate in open cast mines | All mines included in project are underground. |
| The project cannot capture methane from abandoned/decommissioned mines | All mines included in project are active. |
| The project should capture/use virgin coal-bed methane independently of any mining activities | No CBM will be used in project. |
| The project cannot use CO2 or any other fluid/gas to enhance CBM drainage before mining takes place | No CBM will be used in project. |
| Project participants must be able to supply the necessary data for ex-ante projections of methane demand as described in Sections 7 and 8 of ACM0008 | All necessary data is available. |

B.3.	Description of the sources and gases included in the project boundary

|  | Source | Gas |  | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Baseline | Emissions from vented CMM | CH4 | Included | All CMM and VAM captured in the baseline scenario is vented. This is the main emission source. |
|  | Emissions from destruction of methane in the baseline | CO2 | Excluded | No methane is destroyed in the baseline. |
|  |  | CH4 | Excluded | No methane is destroyed in the baseline. |
|  |  | N2O | Excluded | No methane is destroyed in the baseline. |
|  | Grid electricity generation | CO2 | Included | Emissions from Central China Electricity Grid equivalent to power generated by project activity. |
|  |  | CH4 | Excluded | Excluded for simplification. This is conservative. |
|  |  | N2O | Excluded | Excluded for simplification. This is conservative. |
|  | Captive heat use | CO2 | Included | Emissions from coal boilers that will be displaced by heat generated by gas engines and Vocsidizer. |
|  |  | CH4 | Excluded | Excluded for simplification. This is conservative. |
|  |  | N2O | Excluded | Excluded for simplification. This is conservative. |
| Project Activity | Emissions of methane as a result of continued venting | CH4 | Excluded | Only the change in CMM and VAM emissions will be taken into account, by monitoring the methane used or destroyed by the project activity. |
|  | On-site fuel (energy) consumption due to the project activity | CO2 | Included | Electricity consumed by the Central China Electricity Grid required to operate Vocsidizer fans and ancillary equipment for CMM utilization such as gas extraction stations and cooling water pumps. |
|  |  | CH4 | Excluded | Excluded for simplification as per ACM0008. |
|  |  | N2O | Excluded | Excluded for simplification as per ACM0008. |
|  | Emissions from methane destruction | CO2 | Included | Emissions from CMM utilized in gas engines and emissions from VAM utilized in Vocsidizer. |
|  | Emissions from NMHC destruction | CO2 | Included | NMHCs account for less than 1% of volume in extracted gas (gas analysis provided to validator). The level of NMHCs will be continuously monitored throughout the project. |
|  | Fugitive emissions of unburned methane | CH4 | Included | Unburned methane from gas engines and Vocsidizer |
|  | Fugitive methane emissions from on-site equipment | CH4 | Excluded | Excluded for simplification as per ACM0008 |
|  | Accidental methane release | CH4 | Excluded | Excluded for simplification as per ACM0008 |

Figure 3: Project Boundaries

B.4.	Description of how the baseline scenario is identified and description of the identified baseline scenario:

Identification of baseline scenario

Step 1. Identify technically feasible options for capturing and/or using CBM or CMM

Step 1a. Options for CBM and CMM extraction

These options could include:

- Ventilation air methane

- Pre-mining CMM extraction

- Post-mining CMM extraction

- Possible combinations of A, B and C. This is part of the project activity in the absence of the CDM.

CBM extraction is not considered to be a technically feasibly option primarily because the permeability of YCIG’s coal seam (0.05 millidarcy) is not at all suitable for this type of gas extraction.  As a result, YCIG has no intention to pursue efforts to extract CBM from its mines.

At YCIG’s mines, gas extraction is currently a combination of ventilation air methane and pre-mining CMM. Plans to supplement these gas capture techniques with post-mining drainage (goaf etc.) are currently being implemented. However, pre-mining and post-mining CMM will both be brought to the surface through the same drainage system, and it is therefore not possible to specify the shares of gas of each technique. For the purposes of the project, therefore, pre- and post-mining gas will be referred to as “CMM”.

The following tables show the share between CMM and VAM extracted from YCIG’s mines:

Table 7: Share of CMM and VAM gas extraction (2006)

| Mine | VAM (m3 CH4/yr) | CMM (m3 CH4/yr) | VAM % | CMM % |
| --- | --- | --- | --- | --- |
| Qianqiu | 8,409,600 | 6,832,800 | 55.17% | 44.83% |
| Gengcun | 16,819,200 | 7,884,000 | 68.09% | 31.91% |
| Xinan | 15,768,000 | 11,826,000 | 57.14% | 42.86% |
| Total | 40,996,800 | 26,542,800 | 60.70% | 39.30% |

Table 8: Share of CMM and VAM gas extraction (forecast 2007)

| Mine | VAM (m3 CH4/yr) | CMM (m3 CH4/yr) | VAM % | CMM % |
| --- | --- | --- | --- | --- |
| Xinyi | 8,409,600 | 6,307,200 | 57.14% | 42.86% |
| Yian | 8,409,600 | 6,307,200 | 57.14% | 42.86% |
| Mengjin | 8,409,600 | 5,256,000 | 61.54% | 38.46% |
| Total | 25,228,800 | 17,870,400 | 58.54% | 41.46% |

Step 1b. Options for extracted CBM and CMM treatment

- CMM Venting

- Using/destroying ventilation air methane rather than venting it

- Flaring of CBM/CMM

- Use for power generation

- Use for heat generation

- Supply to local users as cooking/heating fuel

- A combination of options ii, iv and v. This is part of the project activity in the absence of the CDM. About 3% of total methane extracted from YCIG’s mines would be utilized in option ii in the form of VAM, and about 14% would be utilized in option iv in the form of CMM.

Step 1c. Options for energy production (electricity and heat)

- Purchase of electricity from the Central China Electricity Grid.

- Electricity production from YCIG captive coal power plants.

- Heat generation from coal boilers.

- Electricity production using captured methane.

- A combination of options 1, 2, 3 and 4. This is the project activity in the absence of the CDM.

Step 2. Eliminate baseline options that do not comply with legal or regulatory requirements

Chinese regulations require that the maximum concentration of methane in a mine’s ventilation air remain below 1% (National Coalmine Safety Regulation of 2005). YCIG’s mines are too rich in gas to ensure that this regulation is met using any of the gas extraction options A, B or C individually. To comply with the safety requirements, these extraction techniques need to be used jointly. Therefore option D is the only gas extraction option that complies with legal requirements, and options A, B and C can be eliminated.

ACM0008 states: If an alternative does not comply with all applicable legislation and regulations, then show that, based on an examination of current practice in the country or region in which the law or regulation applies, those applicable legal or regulatory requirements are systematically not enforced and that non-compliance with those requirements is widespread in the country.

This situation applies to the gas engine technology designed to utilize low-concentration methane. China’s mining code requires that CMM which is utilized should have a minimum methane concentration of 30%. However in some regions in China (including Henan Province), geological factors prevent extracted CMM from reaching this concentration consistently despite technical efforts to do so. Specifically, some coal seams have a permeability that is too low to allow for gas extraction techniques that are used elsewhere. Evidence of CMM extraction with methane concentrations below 30% has been observed at all major coalmine groups of Henan province.

To address this issue, the engine manufacturer Shengdong has developed a technology that permits the safe utilization of CMM below 30% by introducing several safety features such as flame arresters and by injecting water vapour into the gas flow during transport above ground, which largely eliminates the risk of explosion. This technology has been successfully tested and the test results were verified by the scientific and technology committee of the Chinese Mining Safety Bureau in December 2005 (proof of this verification has been provided to validator).

Application of this engine technology for CDM projects is becoming more and more widespread in China, indicating that the requirements of the mining code are not being enforced. According to Shengdong, 260 such engines have already been sold to coalmines throughout China for CDM projects.  More importantly, the development of these projects is being officially approved by both the Chinese NDRC and provincial DRCs, who are required to give their authorization before project commissioning. In addition, the project received a confirmation letter from Coalmine Safety Supervision Bureau of Henan Province on December 11, 2007 which certifies that the project has been operating safely within a methane concentration range of 6% to 30% (copy of letter provided to validator). This demonstrates that Chinese authorities consider this technology to be safe and not in contradiction with the current mining code. As a result it can be concluded that regulations concerning the utilization of CMM with methane concentrations below 30% are not systematically enforced.

Step 3. Formulate baseline scenario alternatives

On the basis of the baseline options that are technically feasible and comply with regulatory requirements, the following baseline scenarios can be formulated. Note that only Option D under Step 1a was retained. Option D is therefore assumed to be part of all baseline scenarios.

Scenario I (business as usual scenario): Gas extraction is a combination of CMM ventilation, as well as pre- and post-mining extraction as defined in option D. All extracted CMM and VAM is vented into the atmosphere with no utilization taking place. YCIG’s electricity demand is met through a combination of grid purchases (88%) and captive coal power (12%). Heat demand is met by using onsite coal boilers fed with YCIG-produced coal.

Scenario II (utilization of VAM): A share of extracted VAM is utilized for heat production. This would involve a newly developed oxidation technology such as the Vocsidizer. One Vocsidizer twin unit can process about 125,000 m3 of VAM per hour with methane concentrations ranging from 0.2-0.8%. Heat generated by the oxidation process can be used to produce hot water at about 70-100 C. Given that this technology is new and still untried in China, it is reasonable to assume that YCIG would begin by installing one twin-unit. Depending on the VAM concentration, this unit could produce approximately 7-10% of YCIG’s annual heat demand, provided that it is located in proximity to the facilities requiring this heat. The remaining heat would continue to be produced in coal boilers, and electricity consumption would be met through purchases from the grid and captive coal power as in Scenario I.

Scenario III (flaring of CMM): The extracted CMM with a methane concentration above at least 25% is destroyed by flaring torches. No energy is produced in this process. YCIG’s energy needs continue to be met by coal boilers and electricity purchases from the grid and captive coal power.

Scenario IV (electricity generation from CMM): The extracted CMM is used to generate electricity from gas engines. A total of 22 x 500kW engines are installed, utilizing approximately 35% of available extracted CMM. Estimated electricity production would be about 53,900 MWh per year. This power would be used to meet YCIG’s own needs, and account for about 2-3% of the group’s annual electricity consumption. The remaining electricity would continue to be purchased from the grid and produced from captive coal power as previously. Waste heat from these engines could be transformed into hot water and supplied to nearby mining facilities. About 17% of YCIG’s total heat demand could be met this way. The remaining heat requirements would be met by coal boilers.

Scenario V (heat generation from CMM using converted coal boilers): Coal-fired boilers would be converted into gas-fired boilers and extracted methane could be used to produce heat for YCIG’s own consumption (showers, kitchens etc.). Yima could meet 100% of its heat requirements by using 21 million m3 CH4/year, accounting for about 45% of total CMM extracted (assuming an NCV of 37.8 MJ/m3 methane). The remaining share of the gas would be vented. Electricity demand would continue to be met by purchases from the grid and captive coal power.

Scenario VI (supply of CMM to local users as a cooking fuel): A gas distribution network for surrounding households (primarily for mine employees) would be constructed. Captured CMM would be supplied to end-users for cooking purposes. In theory, 100% of available extracted CMM could be supplied to households, although in practice, the extent of the distribution network would be limited by their distance to the mines and the costs of laying pipelines. Additional equipment required would be gas tanks, gas pumps and other ancillary equipment. YCIG’s electricity demand would continue to be met by purchases from the grid and captive coal power.

Scenario VII (project activity in absence of the CDM): This is a combination of Scenarios II and IV. It would involve the utilization of about 5% of total extracted VAM in a twin Vocsidizer unit, producing around 18,662 GJ/year of usable heat. A set of 22 x 500kW gas engines would utilize about 35% of available extracted CMM for electricity production (53,900 MWh/year). The remaining electricity would continue to be purchased from the grid and produced from captive coal power. The remaining heat requirements would be met by coal boilers.

Step 4. Eliminate baseline scenario alternatives that face prohibitive barriers

Scenario I (business as usual scenario): This is the business as usual scenario and faces no barriers.

Scenario II (utilization of VAM): The destruction of VAM in the Vocsidizer or a similar technology has taken place only on a few select sites in the form of pilot projects, and never before on Chinese soil. As a result, this technology is untried in China, the risks are perceived as being significant. Personnel trained to operate the Vocsidizer are not available, and coalmine groups in China are very reluctant to be the first to try a new technology. Access to funding to pay for this technology in Euros is therefore also very limited. These risks and technical barriers exclude this scenario from being a realistic baseline option.

Scenario III (flaring of CMM): No regulatory requirements in China exist to mandate flaring of captured CMM. As a result, Chinese coal mine groups are very unfamiliar with flaring equipment, and its usage in China is rare. Some mines interested in exploring this option were obliged to obtain special permission from the relevant safety authorities, creating additional barriers to its implementation. In addition, an important technical barrier would prevent YCIG from flaring extracted CMM. Flaring of CMM is generally considered to be possible only when methane concentrations are above 25%. At YCIG’s mineshafts, however, the methane concentration is generally too low, ranging from 10-25% most of the time. As a result, flaring is not a technically suitable option at YCIG and this scenario can be eliminated.

Scenario IV (electricity generation from CMM):

Technological barriers: YCIG has no experience with gas engines outside the scope of this project. The most important technical barrier to CMM utilization by gas engines has been the low concentration of YCIG’s CMM (ranging from 10-25% in almost all mines). These low concentrations require the application of the Shengdong gas engine technology which is able to utilize lower concentration methane safely. This technology is very recent (distributed only in 2006) and the perceived technology risk is therefore significantly higher than with conventional gas engine technology. It also involves training personnel for the operation of such equipment. Further technological barriers include the know-how and experience that is required to improve the gas extraction system in order to ensure a stable and consistent gas supply. Without such improvements, the project is at financial risk, as engine performance will suffer when the gas supply drops.

Barriers due to prevailing practices: These barriers are also confirmed by common practice in China and Henan Province. In the absence of the CDM, current CMM utilization rates in China are very low. In 2004, CMM drainage accounted for only 23% of total gas released in mining operations. Of this, only about 50% was actually utilized. Nearly 70% of utilization involves gas supply for residential use. As a result, overall utilization rates for electricity generation are extremely low. This is also the case in Henan province, where no other mine group has undertaken investments in CMM electricity production outside of the framework of the CDM. As a result, this scenario can be eliminated.

Scenario V (heat generation from CMM using converted coal boilers): The current system of heat generation using coal-fired boilers is common practice at all mines in Henan Province and is seen as the most economical and technically suitable option for heat generation. Its costs are relatively low due to the waste coal that can be used as a heating fuel. In addition to these barriers due to prevailing practice, YCIG faces the same technical challenge as with other options describe above. Its CMM quality is not high enough to guarantee the safe functioning of gas-fired boilers, which would require methane concentrations of above 30%. In view of these technical barriers, this scenario can be eliminated.

Scenario VI (supply of CMM to local users as a cooking fuel): The main technical barrier is as with Scenario V above. Methane concentrations of YCIG’s extracted CMM are generally below 25% and fluctuate strongly. This makes the gas unreliable and unsuitable as a cooking fuel. This scenario can therefore be eliminated.

Scenario VII (project activity in the absence of the CDM): This scenario is a combination of scenarios II and IV, and faces the same above-identified barriers. It can therefore be eliminated.

As a result, Scenario I (business as usual scenario) is the only scenario that does not face prohibitive barriers. It is therefore considered to be the baseline scenario.

B.5.	Description of how the anthropogenic emissions of GHG by sources are reduced below those that would have occurred in the absence of the registered CDM project activity (assessment and demonstration of additionality):

As required by methodology ACM0008, the latest version of the Tool for the demonstration and assessment of additionality (version 03) is applied to show how emission reductions under the project are additional to those that would have occurred in the absence of the registered CDM project activity.

Section C.1.1 below notes that the first project equipment was installed in December 2006, which was before the start of project validation. According to guidance provided by EB 38 (https://cdm.unfccc.int/EB/038/eb38rep.pdf), project participants must provide evidence that the CDM was considered prior to the start of the project activity. On March 1, 2005 YCIG signed a CDM services agreement with the project development company Eco-Carbone, based in Paris, France. This contract details the work to be carried out by Eco-Carbone in order to assess whether a coalmine utilization project at YCIG’s mines could be considered eligible under the CDM. It includes work on a Project Identification Note (PIN), and a PDD if the conclusions of the PIN are considered to support the possibility of a CDM project activity. This contract and the fact that it was signed more than a year and a half before any equipment was installed clearly provide evidence that the CDM was taken into account before the start of the project activity.

A scanned copy of the contract in English and Chinese language has been provided as evidence to the validator.

The additionality tool is applied as follows:

Step 1. Identification of alternatives to the project activity consistent with current laws and regulations.

As per ACM0008, this step can be ignored.

Step 2. Investment Analysis.

Sub-step 2a. Determine appropriate analysis method

The additionality tool gives the choice between a simple cost analysis (Option 1), an investment comparison (Option II), and a benchmark analysis (Option III). Because the proposed project has financial benefits other than CER revenue, a simple cost analysis is not appropriate. An investment comparison is not suitable either, because the alternative baseline scenario is the Central China Electricity Grid rather than another project. As a result, the benchmark analysis (Option III) has been selected.

Sub-step 2b – Option III. Apply Benchmark Analysis.

Various methods exist to determine the minimum rate of return of an investment. This evaluation must take into account both the cost of money and the investment’s perceived risk. The most common approach to selecting the appropriate discount rate or “hurdle rate” is to look at the investor’s weighted cost of capital (WACC), which is usually a combination of debt and equity. However, the WACC approach is generally considered to be appropriate only if the investment has the same risk profile as the average risk of the company (and therefore the same cost of capital). If this is not the case, an investment-specific risk adjusted discount rate must be used.

A survey conducted by the University of New York concluded that the average cost of capital for energy industries is about 6.8%. Another survey from the magazine Corporate Finance indicates that an appropriate risk premium for project investment in China is 5%, which would make the IRR hurdle rate 11.8%.

This number is confirmed by looking at other projects’ hurdle rates. A survey of the World Bank and the Asian Development Bank, for example, indicates that for hydro projects in China, the cost of capital is generally between 10-12%. For wind projects in China, this hurdle rate has been estimated at high as 20%. Another survey of companies active in Asia estimates the cost of capital in the energy sector of upper middle-income countries (which includes China) at about 10-11%, and the cost of equity at about 13%.

An IRR hurdle rate of 11.8% has therefore been assumed for this project.

Sub-step 2c. Calculation and comparison of financial indicators.

Table 9: Cost and Revenue Assumptions for Project Investment Analysis

| Parameter |  |
| --- | --- |
| Installed capacity (MW) | 11 |
| Total investment (RMB) | 169.5 million |
| Annual operating costs (RMB) | 9.4 million |
| Electricity cost (RMB/kWh, after tax) | 0.383 |
| Heat cost (RMB/GJ, after tax) | 20 |
| Electricity generation (MWh/Year) | 53,900 |
| Heat generation (GJ/Year) | 36,303 |
| Operational lifetime of the project (Years) | 20 |

Table 10: Results of Investment Analysis

|  | 22 x 500kW 1 Vocsidizer |
| --- | --- |
| Project IRR without CERs | 5.07% |
| Project IRR with CERs | 20.11% |

See Annex 3 for complete financial analysis and assumptions.

The analysis is based on the feasibility report and financial analysis completed by YCIG. In the absence of the CDM, the only revenue is from electricity and heat production. The results show that the project is not profitable in the absence of the CDM, with an IRR that falls clearly below the above discussed hurdle rate. With CER revenue, the project’s IRR is raised to 20.11%, which is significantly above the hurdle rate. It should also be noted that the analysis does not include additional costs from underground methane extraction and capture activities that are incurred to guarantee the consistency and stability of CMM flows and methane concentrations.

Sub-step 2d. Sensitivity analysis.

Table 11: Results of Sensitivity Analysis

| Parameters | -10% | -5% | 0 | 5% | 10% |
| --- | --- | --- | --- | --- | --- |
| Investment | 6.58 | 5.79 | 5.07 | 4.39 | 3.77 |
| Operational costs | 5.87 | 5.47 | 5.07 | 4.66 | 4.25 |
| Electricity and heat price | 2.77 | 3.94 | 5.07 | 6.15 | 7.21 |
| Gas engine operating hours | 2.81 | 3.96 | 5.07 | 6.14 | 7.17 |

The table above summarizes the results of the sensitivity analysis carried out for the project. The most sensitive parameter is the electricity and heat price. However, even a 10% increase in the electricity tariff does not raise the IRR above the hurdle rate of 11.8%. It can therefore be concluded that the results of the investment analysis are robust.

Step 3. Barrier analysis (optional)

No barrier analysis has been chosen for this additionality test.

Step 4. Common Practice Analysis

Sub-step 4a. Analyze other activities similar to the proposed project activity

The similarity of other activities can be assessed on the basis of two main factors: a) the geological conditions of the coal seam and the quality of the extracted CMM; b) the technology used in the project.

Concerning the geological conditions and the quality of CMM, other mine groups in Henan province provide a suitable basis for comparison. Various academic references show that coal permeability (measured in mD) is an important factor in determining the quality of extracted CMM using pre-mining drainage techniques. Low coal permeability prevents methane trapped in the coal seam from easily migrating through the coal deposits. Onsite visits to the four major coal groups in Henan showed that coal permeability was comparatively low (i.e. far below 1mD):

Table 12: Coal permeability at major mine groups in Henan Province

| Mine group | Coal permeability (mD) |
| --- | --- |
| Pingdingshan Coal (Group) Co., Ltd. | 0.0019 |
| Zhengzhou Coal (Group) Co., Ltd. | 0.008 |
| Yima Coal Industry (Group) Co., Ltd. | 0.05 |
| Hebi Coal Industry (Group) Co., Ltd. | 0.03-0.045 |

In part due to these geological circumstances, CMM extracted in Henan Province is consistently below 30%. This demonstrates that other coal groups in the province face the same challenges as YCIG in extracting high quality CMM with sufficient methane concentrations for large-scale utilization. An analysis of Henan confirms that none of the other major coal groups has a history of utilizing significant amounts of CMM for commercial purposes.

Concerning the technology used in the project, MEGTEC, the manufacturer of the Vocsidizer has confirmed that there is no ongoing or planned project in China that will use the Vocsidizer without benefiting from the CDM. The same has been confirmed by Shengdong concerning their gas engine model able to utilize lower-concentration CMM. This model has only been only the market since 2006, and it has been sold exclusively to projects operating within the CDM. Indeed, one of its most important markets has been Henan Province, where lower-concentration engines have been sold to seven coalmine groups to be used in CDM project activities.

Sub-step 4b. Discuss any similar options that are occurring

Based on the above analysis, it can be concluded that no other projects using the same technology and facing the same geological conditions as YCIG are occurring.

B.6.	 Emission reductions:

B.6.1. Explanation of methodological choices:

1. Project emissions

Based on Methodology ACM0008, the following formulae are used to calculate project emissions.

PEy=PEME+PEMD+PEUM	(1)

Where:

PEy		Project emissions in year y (tCO2e)

PEME 		Project emissions from energy use to capture and use methane (tCO2e)

PEMD		Project emissions from methane destroyed (tCO2e)

PEUM		Project emissions from un-combusted methane (tCO2e)

PEME: Emissions from additional energy use to capture and use methane

Additional energy use for methane capture will consist only of electricity consumption (gas pumps, water pumps, Vocsidizer VAM fans).

PEME=CONSELEC,PJ´EFELEC	(2)

Where:

CONSELEC,PJ	Additional electricity consumption for capture and use of methane (MWh)

EFELEC                       Emission factor of Central China Electricity Grid  (tCO2e/MWh)

PEMD: Emissions from methane destroyed

PEMD=(MDELEC+MDHEAT)´(CEFCH4+r´CEFNMHC)	(3)

- Where:

MDELEC		Methane destroyed through power generation (tCH4)

MDHEAT	Methane destroyed through heat generation (tCH4)

CEFCH4		Carbon emission factor for combusted methane (2.75 tCO2e/tCH4)

r		r = PCNMHC / PCCH4 (relative proportion of NMHC compared to methane)

CEFNMHC	Carbon emission factor for combusted non-methane hydrocarbons (tCO2eq/tNMHC)

PCCH4		Concentration (in mass) of methane in extracted gas (%) measured on wet basis

PCNMHC		NMHC concentration (in mass) in extracted as (%)

MDELEC=MMELEC´EffELEC	(4)

MDHEAT=MMHEAT´EffHEAT	(5)

Where:

MMELEC	Methane measured sent to power plant (tCH4)

EffELEC 		Efficiency of methane destruction in power plant (set at 99.5% from IPCC)

MMHEAT	Methane measured sent to Vocsidizer (tCH4)

EffHEAT		Efficiency of methane oxidation in Vocsidizer (97% as per manufacturer’s specifications)

PEUM: Emissions from un-combusted methane

PEUM=GWPCH4´iMMi´(1−Effi) 	(6)

Where:

i		Use of methane (power generation, heat generation)

MMi		Methane measured sent to use i (tCH4)

Effi		Efficiency of methane destruction in use i (%)

2. Baseline emissions

Based on Methodology ACM0008, the following formulae are used to calculate baseline emissions.

BEy=BEMD,y+BEMR,y+BEUse,y	(7)

Where:

BEy 		Baseline emissions in year y (tCO2e)

BEMD,y 	Baseline emissions from destruction of methane in the baseline scenario in year y (tCO2e)

BEMR,y	Baseline emissions from release of methane into the atmosphere in year y that is avoided by the project activity (tCO2e)

BEUse,y	Baseline emissions from the production of power or heat replaced by the project activity in year y (tCO2e)

BEMD,y: Baseline emissions from destruction of methane in the baseline scenario

There is no methane destruction in the baseline scenario, so emissions for this part of the formula are zero.

BEMR,y : Baseline emissions from release of methane into the atmosphere

As discussed in section B.4, for the purposes of the project no distinction is made between pre- and post-mining CMM. Therefore, the formula reads as follows:

BEMR,y=GWPCH4´i(CMMPJi,y−CMMBLi,y)	(8)

Where:

CMMPJi,y	Pre-mining CMM captured, sent to and destroyed by use i in the project activity in year y (tCH4)

CMMBLi,y 	Pre-mining CMM that would have been captured, sent to and destroyed by use i in the

baseline scenario in year y (tCH4)

GWPCH4 	Global warming potential of methane (21 tCO2e/tCH4)

Note that CMMBLi,y is zero, as there is no baseline utilization of CMM.

BEUse,y: Baseline emissions from the production of power and heat replaced by the project activity

PBEUse,y=GENy´EFELEC+HEATy´EFHEAT	(9)

Where:

PBEUse,y 	Potential total baseline emissions from the production of power or heat replaced by the project activity in year y (tCO2e)

GENy 		Electricity generated by project activity in year y (MWh)

EFELEC 		Emissions factor of electricity (Central China Electricity Grid) replaced by project

(tCO2/MWh)

HEATy 		Heat generation by project activity in year y (GJ)

EFHEAT 		Emissions factor for heat production replaced by project activity (tCO2/GJ)

EFELEC: Grid emission factor

As per ACM0008, the methodology ACM0002 is used to calculate the grid emission factor. For the proposed project, this emission factor is based on the Central China Electricity Grid and is calculated as follows:

EFELEC,y=wOM´EFOM,y+wBM´EFBM,y	(10)

Where:

EFOM,y 		Emission factor of the grid’s operating margin (tCO2e/MWh)

EFBM,y 		Emission factor of the grid’s build margin (tCO2e/MWh)

wOM		0.5

wBM		0.5

EFOM,y: Operating margin (OM) emission factor

ACM0002 states that the dispatch data analysis should be the first choice to calculate the OM. In China, this information is a commercial secret, and is not available to the public. Therefore, an alternative method will be used.

The Simple OM method may be used where low-cost/must run resources constitute less than 50% of total grid generation over an average of the five most recent years. This is the case for the Central China Electricity Grid. Therefore the simple OM method is used.

The Simple OM emission factor is calculated as the generation-weighted average emissions per electricity unit (tCO2/MWh) of all generating sources serving the system, not including low-operating cost and must-run power plants:

EFOM,y=i,jFi,j,y⋅COEFi,jjGENj,y	(11)

Where:

Fi ,j,		Amount of fuel i (in a mass or volume unit) consumed by relevant power sources j in

year(s) y

j	Power sources delivering electricity to the grid, not including low-operating cost and must-run power plants, and including imports to the grid

COEFi,j y 	CO2 emission coefficient of fuel i (tCO2 / mass or volume unit of the fuel), taking into

account the carbon content of the fuels used by relevant power sources j and the percent oxidation of the fuel in year(s) y

GENj,y		Electricity (MWh) delivered to the grid by source j

The CO2 emission coefficient COEFi of fuel i can be calculated as follows:

COEFi=NCVi´EFCO2,i´OXIDi´4412	(12)

Where:

NCVi 		Net calorific value per mass or volume unit of a fuel i

OXIDi 		Oxidation factor of the fuel

EFCO2,i 		CO2 emission factor per unit of energy of the fuel i.

EFBM,y: Build margin (BM) emission factor

Based on Methodology ACM0002, the BM can be calculated as the generation-weighted average emission factor (tCO2/MWh) of a sample of power plants m, as follows:

EFBM,y=i.mFi.m,y⋅COEFi.mmGENm,y	(13)

Where Fi,m,y, COEFi,m and GENm,y are analogous to the variables described for the simple OM method above for plants m.

As per ACM0002, Option 1 was selected to identify the sample group m to calculate the BM ex ante. According to this option, the sample group consists of either the five power plants that have been built most recently, or the power plant capacity additions in the electricity system that comprise 20% of the system generation (MWh) and that have been built most recently.

However, because information on electricity generation and fuel consumption of individual plants is regarded as a commercial secret in China, the CDM Executive Board has allowed for a deviation from ACM0002 in China. This deviation allows for the following steps to be used in order to calculate the BM under Option 1:

- Use of capacity additions for estimating the build margin emission factor for grid electricity.

- Use of weights estimated using installed capacity in place of annual electricity generation.

- Use the efficiency level of the best technology commercially available in the provincial/regional or national grid of China, as a conservative proxy, for each fuel type in estimating the fuel consumption to estimate the build margin (BM).

The BM is therefore calculated as follows:

EFBM,y=CAPThermalCAPTotal´EFAdv,Thermal	(14)

Where:

CAPTotal	Total capacity addition

CAPThermal	Fossil fuel-fired capacity addition

EFAdv,Thermal	CO2 emission factor of the best thermal power technology commercially available

EFHEAT: Emissions factor for heat production

ACM0008 uses the following formula to calculate the emission factor of heat that is displaced by the project:

EFheat,y=EFCO2,iEffheat´4412´1TJ1000GJ	(15)

Where:

EFheat,y 		Emissions factor for heat generation (tCO2/GJ)

EFCO2,i 		CO2 emissions factor of fuel used in heat generation (tC/TJ)

Effheat 		Boiler efficiency of the heat generation (%)

44/12 		Carbon to carbon dioxide conversion factor

1/1000 		TJ to GJ conversion factor

To select the boiler efficiency, Option B was chosen: assume a boiler efficiency of 100% based on the net calorific values as a conservative approach.

3. Leakage

Project leakage LEy is calculated from the following formula:

LEy=LEd,y+LEo,y	(16)

Where:

LEy		Leakage emissions in year y (tCO2e)

LEd,y	Leakage emissions due to displacement of other baseline thermal energy uses of methane in year y (tCO2e)

LEo,y	Leakage emissions due to other uncertainties in year y (tCO2e)

LEd,y: Leakage emissions due to displacement of other baseline thermal energy uses

Because there is no baseline thermal demand, this form of leakage can be excluded.

LEo,y: Leakage emissions due to other uncertainties

Three types of leakage due to other uncertainties are considered in ACM0008:

- CBM drainage from outside the de-stressed zone

- Impact of CDM project activity on coal production

- Impact of CDM project activity on coal prices and market dynamics

Leakage category a) can be excluded because the project does not involve any CBM capture.  Leakage category b) requires a discount factor to be applied if the project activity is CBM/CMM extraction and the baseline scenario is ventilation only. This does not apply to the current project because CMM extraction is already part of the baseline.  Leakage category c) may be further considered by the CDM Executive Board at a later date when more information concerning the price and market impacts of this project type is available. It is therefore not relevant at this point in time.

As a result, leakage emissions due to uncertainties can be safely excluded from the project.

B.6.2. Data and parameters that are available at validation:

| Data / Parameter: | Density of methane |
| --- | --- |
| Data unit: | kg/m3 |
| Description: |  |
| Source of data used: | 2006 IPCC Guidelines for National Greenhouse Gas Inventories |
| Value applied: | 0.67 |
| Justification of the choice of data or description of measurement methods and procedures actually applied : | Actual density will be measured by pressure transmitters and temperature sensors connected to the flow of both CMM and VAM to the project equipment. |
| Any comment: | Applies at atmospheric pressure and 20C |

| Data / Parameter: | EFCO2,i |
| --- | --- |
| Data unit: | tC/TJ |
| Description: | CO2 emissions factor of fuel used in heat generation |
| Source of data used: | 2006 IPCC Guidelines for National Greenhouse Gas Inventories |
| Value applied: | 25.8 |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | IPCC reference |
| Any comment: | Used to calculate the emission factor of coal in coal boilers which is replaced by methane-based thermal energy. |

| Data / Parameter: | EffHEAT |
| --- | --- |
| Data unit: | % |
| Description: | Efficiency of methane oxidation in Vocsidizer |
| Source of data used: | Manufacturer’s specifications |
| Value applied: | 97% |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | The Vocsidizer is based on a technology that oxidizes CMM over a heated ceramic bed. Every few minutes, the direction of the flow of CMM over the bed changes, and the molecules which change direction will not be fully oxidized. This results in an oxidation efficiency of 97%. |
| Any comment: | Used to calculate the actual amount of methane that is oxidized by the Vocsidizer during the project activity. |

| Data / Parameter: | EffELEC |
| --- | --- |
| Data unit: | % |
| Description: | Efficiency of methane destruction in power plant |
| Source of data used: | As per ACM0008 (IPCC) |
| Value applied: | 99.5% |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | IPCC reference |
| Any comment: | Used to calculate efficiency of gas destruction power electricity production in gas engines. |

| Data / Parameter: | EFOM,y |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | Operating margin (OM) of Central China Electricity Grid |
| Source of data used: | China Electricity Yearbooks 2004-2006 |
| Value applied: | 1.2899 |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | Calculated according to ACM0002. See Annex 3. |
| Any comment: |  |

| Data / Parameter: | EFBM,y |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | Build Margin (BM) of Central China Electricity Grid |
| Source of data used: | China Electricity Yearbooks 2003-2006 |
| Value applied: | 0.6594 |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | Calculated according to ACM0002. See Annex 3. |
| Any comment: |  |

| Data / Parameter: | EFHEAT |
| --- | --- |
| Data unit: | tCO2/GJ |
| Description: | Emissions factor for heat production |
| Source of data used: | Calculated as per ACM0008 |
| Value applied: | 0.095 |
| Justification of the choice of data or description of measurement methods and procedures actually applied: | Based on EFCO2,I of 25.8 tC/TJ from IPCC. |
| Any comment: | Used to calculate emissions from coal boilers replaced by project’s heat production. |

B.6.3 Ex-ante calculation of emission reductions:

1. Project Emissions

PEy=PEME+PEMD+PEUM	(17)

PEME: Project emissions from energy use to capture and use methane

Table 13: Energy requirements for methane capture and utilization

| Mine | Vocsidizer (kW) | Water cycle pumps (kW) | Hot water pumps (kW) | Cold water pumps (kW) | Cooling water towers (kW) | Water to steam conversion (kW) | Water softener pumps (kW) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qianqiu | 0 | 45 | 10 | 0.5 | 11 | 11 | 0.8 |
| Gengcun | 0 | 67.5 | 15 | 0.75 | 16.5 | 16.5 | 1.2 |
| Xinan | 0 | 45 | 10 | 0.5 | 11 | 11 | 0.8 |
| Xinyi | 0 | 45 | 10 | 0.5 | 11 | 11 | 0.8 |
| Yian | 0 | 45 | 10 | 0.5 | 11 | 11 | 0.8 |
| Mengjin | 300 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | 300 | 247.5 | 55 | 2.75 | 60.5 | 60.5 | 4.4 |

Operating hours of gas engine ancillary equipment: 	7,000 hours/year

Operating hours of Vocsidizer ventilators: 	7,200 hours/year

EFELEC:	 0.9747 tCO2e/MWh

Vocsidizer electricity consumption

= (300 kW x 7,200 hrs)/1000 = 2,160 MWh/year

Gas engine ancillary equipment electricity consumption:

= [(247.5 + 55 + 2.75 + 60.5 + 60.5 + 4.4) x 7,000 hrs]/1000 = 3,014.55 MWh/year

CONSELEC,PJ = 2,160 + 3,014.55 = 5,174.55 MWh/year

PEME: 5,174.55 x 0.9747  = 5,043 tCO2e/year (differences in value due to rounding)

PEMD: Project emissions from methane destroyed

Operating hours of gas engines:	7,000 hours/year

Methane destruction per gas engine: 	101 m3 CH4/engine/hour

Number of gas engines installed: 	22

Operating hours of Vocsidizer: 	7,200 hours/year

Methane destruction of Vocsidizer unit: 	500 m3 CH4/Vocsidizer/hour

Density of methane: 		0.67 kg/m3

EffELEC: 		0.995

EffHEAT: 	0.97

CEFCH4: 	2.75

MMELEC = 22 x 7,000 x 101 x 0.67/1000 = 10,422 tCH4/year

MMHEAT = 7,200 x 500 x 0.67/1000 = 2,412 tCH4/year

MDELEC = 10,422 x 0.995 = 10,370 tCH4/year

MDHEAT = 2,412 x 0.97 = 2,340 tCH4/year

PEMD= (10,370 + 2,340) x 2.75 = 34,952 tCO2e/year (difference in value due to rounding)

PEUM: Project emissions from un-combusted methane

PEUM = (10,422 x [1-0.995] + 2,412 x [1-0.97]) x 21 = 2,614 tCO2e/year

Total Project Emissions  = 5,043 + 34,952 + 2,614 = 42,609 tCO2e/year

2. Baseline Emissions

BEy=BEMD,y+BEMR,y+BEUse,y  	(18)

BEMD,y: Baseline emissions from destruction of methane in the baseline scenario

Methane destruction in the baseline scenario is zero.

BEMR,y: Baseline emissions from release of methane into the atmosphere that is avoided by the project activity

Methane emissions avoided by gas engines: 	10,422 tCH4/year

Methane emissions avoided by Vocsidizer: 	2,412 tCH4/year

CMMPJi,y (total methane emissions avoided by project activity):	12,834 tCH4/year

GWPCH4:		21 tCO2e/tCH4

BEMR,y = 12,834 x 21 = 269,519 tCO2e/year (differences in value due to rounding)

BEUse,y: Baseline emissions from the production of power and heat

Gas engine estimated performance: 	0.35 MW

Gas engine waste heat generation: 	0.1145 GJ/hour

Operating hours of gas engines: 	7,000 hours/year

Vocsidizer heat generation: 	2.592 GJ/hour

Operating hours of Vocsidizers: 	7,200 hours/year

EFELEC: 	0.9747 tCO2e/MWh

EFHEAT: 	0.095 tCO2e/GJ

GENy (Electricity generated by project):

= 22 x 0.35 x 7,000 = 53,900 MWh/year

HEATENG (Heat generated by gas engines):

= 22 x 0.1451 x 7,000  = 17,640 GJ/year (difference in value due to rounding)

HEATVOCS (Heat generated by Vocsidizers):

= 2.592 x 7,200 = 18,662 GJ/year

BEUse,y = (53,900 x 0.9747) + (17,640 + 18,662) x 0.095 = 55,969  tCO2e/year

(differences in value due to rounding)

Total baseline emissions = 0 + 269,519+ 55,969  = 325,488 tCO2e/year

3. Leakage

As determined in section B.6.1, leakage emissions are zero.

4. Emission reductions

Table 14: Summary of Project Emission Reductions

ERy=BEy−PEy−LEy	(19)

ERy = 325,488 – 42,609 = 282,879 tCO2e/year

B.6.4 Summary of the ex-ante estimation of emission reductions:

Table 15: Total Emission Reductions

| Year | BEy:Total baseline emissions (tCO2e) | PEy: Total project emissions (tCO2e) | LEy: Total leakage emissions (tCO2e) | ERy: Total emission reductions (tCO2e) |
| --- | --- | --- | --- | --- |
| 2008 | 82,749 | 14,017 | 0 | 68,732 |
| 2009 | 325,488 | 42,609 | 0 | 282,879 |
| 2010 | 325,488 | 42,609 | 0 | 282,879 |
| 2011 | 325,488 | 42,609 | 0 | 282,879 |
| 2012 | 325,488 | 42,609 | 0 | 282,879 |
| 2013 | 325,488 | 42,609 | 0 | 282,879 |
| 2014 | 325,488 | 42,609 | 0 | 282,879 |
| 2015 | 325,488 | 42,609 | 0 | 282,879 |
| 2016 | 325,488 | 42,609 | 0 | 282,879 |
| 2017 | 325,488 | 42,609 | 0 | 282,879 |
| 2018 | 54,248 | 7,102 | 0 | 47,146 |
| Total (tCO2e) | 3,012,139 | 397,500 | 0 | 2,661,786 |

B.7 Application of the monitoring methodology and description of the monitoring plan:

B.7.1	Data and parameters monitored:

| Data / Parameter: | CONSELEC,PJ |
| --- | --- |
| Data unit: | MWh |
| Description: | Electricity consumed for capture and use of methane: includes Vocsidizer ventilation fans and ancillary equipment like water pumps for gas engines. |
| Source of data to be used: | Based on estimate from equipment supplier (see Table 13). |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 5,1754 MWh/year |
| Description of measurement methods and procedures to be applied: | Electricity consumption will be measured by electricity flow meters conforming to Chinese national standards. |
| QA/QC procedures to be applied: | Meters will be subject to regular maintenance and calibration according to the manufacturer’s specifications. |
| Any comment: | See details of monitoring plan. |

| Data / Parameter: | MMELEC |
| --- | --- |
| Data unit: | tCH4 |
| Description: | Methane sent to gas engines for power generation |
| Source of data to be used: | Based on engine manufacturer specifications |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 10,422 tCH4/year |
| Description of measurement methods and procedures to be applied: | In order to calculate the amount of methane sent to gas engines, the following parameters will be measured: - Concentration of methane in coalmine gas (measured by gas analyzer) - Volume of coalmine gas (measured by gas flow meter) - Pressure of coalmine gas (measured by pressure transmitter) - Temperature of coalmine gas (measured by temperature sensor) These parameters will be fed into a data logger that will calculate and record the mass of methane sent to gas engines. |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: | See details of monitoring plan. |

| Data / Parameter: | MDELEC |
| --- | --- |
| Data unit: | tCH4 |
| Description: | Methane destroyed by power generation |
| Source of data to be used: | Calculation based on MMELEC and EffELEC |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 10,370 tCH4/year |
| Description of measurement methods and procedures to be applied: | Methane destroyed by power generation will be calculated monthly using the following formula: MDELEC=MMELEC´EffELEC |
| QA/QC procedures to be applied: | The data will be archived electronically and all data will be kept till at least 2 years after the end of the crediting period. |
| Any comment: |  |

| Data / Parameter: | MMHEAT |
| --- | --- |
| Data unit: | tCH4 |
| Description: | Methane sent to Vocsidizers for heat generation |
| Source of data to be used: | Manufacturer’s estimate. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 2,412 tCH4/year |
| Description of measurement methods and procedures to be applied: | The weight of methane will be determined as above, measuring concentration, volume, pressure and temperature of the incoming ventilation air methane. |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: | See details of monitoring plan. |

| Data / Parameter: | MDHEAT |
| --- | --- |
| Data unit: | tCH4 |
| Description: | Methane destroyed by heat generation |
| Source of data to be used: | Calculation based on MMHEAT and EffHEAT |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 2,340 tCH4/year |
| Description of measurement methods and procedures to be applied: | Methane destroyed by power generation will be calculated monthly using the following formula: MDHEAT=MMHEAT´EffHEAT |
| QA/QC procedures to be applied: | The data will be archived electronically and all data will be kept till at least 2 years after the end of the crediting period. |
| Any comment: |  |

| Data / Parameter: | CEFNMHC |
| --- | --- |
| Data unit: | tCO2eq/tNMHC |
| Description: | Carbon emission factor of non-methane hydrocarbons |
| Source of data to be used: | Periodical gas analyses provided by YCIG |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | NMHC should account for less than 1% by volume of extracted coal mine gas and therefore emissions from combustion of NMHC have not been taken into account for the calculation of expected emission reductions (gas analysis provided to validator). |
| Description of measurement methods and procedures to be applied: | YCIG will take annual gas analysis samples to verify that NMHCs can be excluded. |
| QA/QC procedures to be applied: | Analyses will be taken at all mines included in the project and the results supervised and signed off by the CDM project manager. |
| Any comment: |  |

| Data / Parameter: | PCCH4 |
| --- | --- |
| Data unit: | % |
| Description: | Concentration (in mass) of methane in extracted gas |
| Source of data to be used: | Measured on a daily basis. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | The amount of pure methane is adopted for expected emission reduction calculation. |
| Description of measurement methods and procedures to be applied: | Methane concentration will be measured on a wet basis by concentration meters, optical and calorific. The data will be archived electronically and all data will be kept till at least 2 years after the end of the crediting period. |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: |  |

| Data / Parameter: | PCNMHC |
| --- | --- |
| Data unit: | % |
| Description: | Non methane hydrocarbons (NMHC) concentration (in mass) in coal mine gas |
| Source of data to be used: | To be obtained through annual analysis of captured gas. If NMHC concentration is less than 1%, it is not accounted. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | Not accounted. |
| Description of measurement methods and procedures to be applied: | Annual measurements of NMHC concentration by concentration meters (optical and calorific) to check out whether it is less than 1% to determine whether NMHC emissions can be excluded from project emission calculation. The data will be archived electronically and all data will be kept till at least 2 years after the end of the crediting period. |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: |  |

| Data / Parameter: | r |
| --- | --- |
| Data unit: | % |
| Description: | Relative proportion of NMHC compared to methane in captured gas |
| Source of data to be used: | Calculated from PCCH4 and PCNMHC : r = PCNMHC / PCCH4. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | Not accounted. |
| Description of measurement methods and procedures to be applied: | Annual calculation based on annual monitoring of PCNMHC to check out whether r is less than 1% to determine whether NMHC emissions can be excluded from project emission calculation. |
| QA/QC procedures to be applied: |  |
| Any comment: |  |

| Data / Parameter: | GENy |
| --- | --- |
| Data unit: | MWh |
| Description: | Electricity generated by project |
| Source of data to be used: | Manufacturer estimate |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 53,900 MWh/year |
| Description of measurement methods and procedures to be applied: | Electricity flow meters will measure electricity production from gas engines. They will conform to Chinese national standards. |
| QA/QC procedures to be applied: | Meters will be subject to regular maintenance and calibration according to the manufacturer’s specifications. |
| Any comment: | See details of monitoring plan. |

| Data / Parameter: | HEATENG |
| --- | --- |
| Data unit: | GJ |
| Description: | Heat generation from gas engines |
| Source of data to be used: | Manufacturer’s specifications. |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 17,640 GJ/year |
| Description of measurement methods and procedures to be applied: | - The amount of thermal energy produced will be determined by measuring the following parameters: - Volume of hot water sent to mine facilities (flow meter) - Temperature of water (temperature sensor) - These parameters will be directly fed into a data logger which will calculate the energy content of the hot water (in joules). |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: | It is expected that only 4 engines at Gengcun mine will be equipped with heat recovery equipment. More heat may be recovered at other sites in future, so the technical potential for heat production from the project is significantly higher than the value above. |

| Data / Parameter: | HEATVOCS |
| --- | --- |
| Data unit: | GJ |
| Description: | Heat generation from Vocsidizer |
| Source of data to be used: | Manufacturer’s specifications |
| Value of data applied for the purpose of calculating expected emission reductions in section B.5 | 18,662 GJ/year |
| Description of measurement methods and procedures to be applied: | - The amount of thermal energy produced will be determined by measuring the following parameters: - Volume of hot water sent to mine facilities (flow meter) - Temperature of water (temperature sensor) - These parameters will be directly fed into a data logger which will calculate the energy content of the hot water (in joules). |
| QA/QC procedures to be applied: | All meters and sensors will be subject to regular maintenance and calibration according to manufacturer’s specifications. |
| Any comment: | To be conservative, it has been assumed that only 30% of available heat from the Vocsidizer will actually be utilized. The above value represents a conservative estimate and not the Vocsidizer’s technical potential for heat production. The exact heat utilization rate depends on the proximity of end users and mine facilities. |

B.7.2 Description of the monitoring plan:

Monitoring equipment installation

The project utilizes two types of equipment for gas utilization: a) gas engines for the combustion of CMM; b) Vocsidizers for the oxidation of VAM. There are therefore two types of setup for the monitoring equipment.

Monitoring equipment will be installed in each mine and the type of setup will depend on the CMM and VAM utilization equipment installed in the mine. Those mines where the same utilization approach (gas engines with power and heat generation and/or Vocsidizer with heat generation) is implemented would adopt the same setup type of monitoring equipment.

This table summarizes for each mine the equipment for CMM and VAM utilization that will be installed.

Table 16: CMM and VAM utilization in each mine

| Mine | Gas engines with power and heat generation | Vocsidizers with heat generation |
| --- | --- | --- |
| Qianqiu | X |  |
| Gengcun | X |  |
| Xinan | X |  |
| Xinyi | X |  |
| Yian | X |  |
| Mengjin |  | X |

Consequently, the following monitoring equipment will be installed:

- Monitoring equipment for CMM utilization adapted to engines for power and heat generation: installed at Qianqiu, Gengcun, Xinan, Xinyi and Yian mines (all mines except Mengjin mine).

- Monitoring equipment for VAM utilization adapted to Vocsidizers for heat generation: installed only at Mengjin mine.

For each of the above-described project sites, a separate agreement will be signed between monitoring equipment supplier ABB Systems Shanghai and YCIG. YCIG will notify ABB of any additional gas utilization equipment that will be installed. This will allow ABB to plan ahead and deliver the monitoring instruments in time so that emission reductions can be measured as soon as the gas utilization equipment is operational on each site.

YCIG and ABB will also sign a comprehensive service agreement that will ensure all instruments are serviced and maintained properly.

Gas engines monitoring equipment to be installed at Qianqiu, Gengcun, Xinan, Xinyi and Yian mines

The following instruments will be installed at each site at which gas engines utilize CMM (Qianqiu, Gengcun, Xinan, Xinyi and Yian).

| Instrument name | Parameter measured |
| --- | --- |
| Gas analyzer | CH4 concentration in CMM |
| Gas flow meter | Flow rate of CMM (through differential pressure) |
| Temperature transmitter | Temperature of CMM |
| Pressure Transmitter | Pressure of CMM |
| Temperature Transmitter | Temperature of hot water from engines |
| Water Flow Transmitter | Flow rate of hot water |
| Power Meters | Electricity produced and consumed by engines operation |

Vocsidizer monitoring equipment to be installed at Mengjin mine

The following instruments will be installed at Mengjin mine for monitoring of Vocsidizer’s operation.

| Instrument Name | Parameter Measured |
| --- | --- |
| Gas Analyzer | CH4 concentration in VAM |
| Flow meter | Flow rate of VAM (differential pressure) |
| Temperature Transmitter | Temperature of VAM |
| Pressure Transmitter | Pressure of VAM |
| Temperature Transmitter | Temperature of hot water from Vocsidizer |
| Flow Transmitter | Water Flow Rate |
| Power Meter | Electricity consumed for Vocsidizer operation |

Figure 4: Monitoring diagram for gas engines and Vocsidizers

Organization

YCIG has designated a CDM project manager who is in charge of the overall management of the project including the monitoring plan (see contact details in Annex 1). The managing site engineers of each site where equipment is installed report directly to him. He is also the direct contact point for the verification DOE and the monitoring equipment supplier.

For each mine, the individual managing site engineer is responsible for the operation and maintenance of the equipment (gas engines or Vocsidizers) and is supported by a staff of engineers and workers. As a complete set of monitoring equipment is installed at each equipment site, each managing site engineer will be responsible for the monitoring equipment, the data collection and QA/QC procedures for his/her site.

The equipment supplier will be in frequent contact with the managing site engineer and will conduct periodic visits (see Quality Control below). During these visits, there will be regular information exchange and a learning process to ensure that the managing site engineer and his team are familiar with the operation and maintenance of the equipment.

Figure 5: Organizational structure of monitoring team

QA/QC

Each coal mine will adopt the proper monitoring and QA/QC procedures according to its CMM and VAM utilization.

For each mine, specific individuals and QA/QC auditor (the managing site engineer) for project monitoring are appointed. Each managing site engineer is responsible for the training of site’s staff working on monitoring. Managing site engineers will be responsible for monitoring equipment operation, maintenance and calibration of the mine they are responsible for. They will check data quality by verifying if there are any values that can be considered to be outside the usual measurement range and by cross-checking all data available. In case inconsistency is detected, the managing site engineer will inform the CDM Project Manager who will take a decision on how to rectify the situation. Managing site engineers will also be in charge of data recording.

The CDM Project Manager is responsible for the training of all the managing site engineers with the technical assistance of the equipment supplier, for project monitoring management and for the QA/QC procedures and the consolidation of all measures. The CDM Project Manager will also check that data are recorded properly in each mine.

Technical services

YCIG will sign a service agreement with the selected equipment supplier. This agreement will guarantee a) that all equipment operates safely and reliably through regular maintenance visits in all mines where monitoring equipment is installed; b) that all equipment is correctly calibrated according to the manufacturer’s recommendations; c) that any malfunctioning or damaged equipment can be repaired or replaced quickly.

Data collection and storage

All measured parameters in each mine are sent to a data logger that records data on a continuous basis and computes the required values (such as mass of methane, or energy content of hot water). The records will also indicate the time and date of each data measurement in order to verify any irregularities and double check them with other data such as engine running hours. Records will exist electronically and as paper copies and will be stored for two years after the end of the crediting period as required by ACM0008. Each managing site engineer will be in charge of data records for his/her mine. In case of equipment malfunction, this will be registered by the data logger and will be taken into account in the monitoring report submitted to the relevant DOE. For each monitoring report submitted to the DOE, data will be aggregated from the individual sites under the supervision of the CDM project manager.

Measurements to determine CEFNMHC, PCNMHC and calculate r will be taken annually through manual procedures at all mines included in the project. These gas analyses will be included in the periodic monitoring reports and will be supervised by the CDM project manager and the managing site engineers.

Training

Each managing site engineers and his/her team will receive training on the functioning and maintenance of the site-specific monitoring equipment during the initial installation that will be supervised by the equipment supplier. Specific attention will be given to issues that could affect the CER calculations, such as equipment calibration. Additional information exchange and training will take place during each visit of the supplier’s technical team. Any technical questions that arise can be quickly addressed by contacting the supplier, with which there will be regular contact. Furthermore, the CDM consultant will organize a training session with the YCIG CDM manager and his staff to discuss the monitoring plan and the relevant requirements in detail.

Equipment specifications and calibration

All equipment will correspond to Chinese national standards. Data measurement will take place continuously and be transmitted  for each mine to the data logger as described above. The equipment will have an accuracy value of no less than ±3%. All instruments have possible measurement ranges that are adequate for the projects, and the specific ranges will be programmed according to project conditions. The most sensitive instrument for calibration is the gas analyzer which will either be self-calibrating, or be calibrated regularly as recommended by the supplier. This will be carried out by the supplier’s technicians themselves, or, after sufficient training is received, by YCIG’s staff.  Other equipment will be calibrated manually according to national standards and the supplier’s specifications.

B.8 Date of completion of the application of the baseline study and monitoring methodology and the name of the responsible person(s)/entity(ies)

Date of completion of the application of the methodology to the project activity: 02/11/07

| Name | Project Participant (Yes/No) |
| --- | --- |
| ECO-CARBONE 15, ave de Segur 75007 Paris France Tel: +33 (0)1 53 59 32 58 Fax: +33 (0)1 53 59 94 46 e-mail: michael.pollan@eco-carbone.com | No |

SECTION C. 	Duration of the project activity / crediting period

C.1	Duration of the project activity:

C.1.1.	Starting date of the project activity:

First set of gas engines were installed on 01/12/2006.

C.1.2.	Expected operational lifetime of the project activity:

20 years

C.2	Choice of the crediting period and related information:

C.2.1.	Renewable crediting period

C.2.1.1.	 	Starting date of the first crediting period:

>>

C.2.1.2.		Length of the first crediting period:

>>

C.2.2.	Fixed crediting period:

C.2.2.1.		Starting date:

01/04/2008

C.2.2.2.		Length:

10 years

SECTION D. 	Environmental impacts

D.1.	Documentation on the analysis of the environmental impacts, including transboundary impacts:

As per the relevant guidelines, the Environmental Impact Assessment Report for the project was prepared by the Henan Research Institute for Environmental Protection, and was reviewed and appraised by experts from the Henan Environmental Protection Bureau in January 2007. It was approved with the following conclusions:

Project construction period

The project’s main construction activities involve the construction of the workshop for the generator sets, and equipment installation in these workshops. Possible pollution caused by construction is dust and noise from equipment and material transport. No direct influence on the environment is anticipated, however, as the project sites at all mines are YCIG-owned industrial areas and abut farming land. No new land needs to be converted for the project.

Water

The main source of water consumption is cooling water required by gas engines. However, no new water needs to be added to this process as the volume of available and processed coalmine water at each mine is sufficient. Dispensed cooling water will be used for irrigation purposes in order to reduce dust. No new pollutants are added to the wastewater by the project.

Local air quality

The combustion of CMM in gas engines produces N2, CO2, and some NOx. NOx is a regulated pollutant, and its production depends largely on the gas combustion temperature. As gas combustion will take place at temperatures below 1,000 C, NOx production is expected to be limited to about 50 mg/Nm3. This is safely below the regulatory limit for coal-fired plants which, according to GB13223-2003, is set at 80 mg/Nm3.

Noise

Noise will be produced mainly from gas engines, generators, exhaust releasers and water pumps. Without any noise protection measures, it is expected to range from 67-85 dB on the project site itself. With noise protection measures (such as mufflers on exhaust pipes and workshop sound insulation), it can be reduced to a level of about 61-75 dB on the project site. Along a 50m radius, noise protection measures are necessary to comply with noise standards of 50dB and 60dB (during day and night respectively), and these will be implemented by the project. However, as there are no sensitivity points within a 200 m radius (schools, hospitals, residences), the project will not inflict any noise pollution on the surrounding environment.

D.2.	If environmental impacts are considered significant by the project participants or the host Party, please provide conclusions and all references to support documentation of an environmental impact assessment undertaken in accordance with the procedures as required by the host Party:

None of the above-identified environmental impacts was considered to be significant by the environmental impact assessment.

SECTION E. 	Stakeholders’ comments

E.1.	Brief description of how comments by local stakeholders have been invited and compiled:

YCIG carried out two types of stakeholder consultations. The first was a general meeting at its offices on March 21, 2006, in order to present and discuss the project. Participants (total of 18) at this meeting included project engineers, mine employees, representatives of local worker councils, representatives of local villagers, and mine representatives. Participants were invited to the meeting by telephone.

A second stakeholder consultation was carried out by questionnaire within the framework of the project’s environmental impact assessment. One hundred of these questionnaires were sent out to local residents, mine workers, mine engineers, and local farmers. The questionnaire included a description of the project and several questions regarding its potential impact on the local environment, the economy, and the general opinion about the project. Questionnaires were returned and the results summarized in the environmental impact assessment.

E.2.	Summary of the comments received:

The meeting on March 21, 2006 began with a general presentation of the project by YCIG. The current methane extraction situation was explained, including the need to address the safety issue connected to methane. An overview of different technologies that might be applied to the project was also given. Finally, the project itself was explained in detail, including the technology involved, the investment necessary, and other factors. The meeting was then opened for general discussion and questions by stakeholders.

Comments received were all positive. Participants emphasized the importance of combining environmental protection with increased profitability for the mine, as well as the fact that a previously unused energy resource was now being successfully harnessed. The stakeholder consultation process itself was also appreciated by participants. Some hopes were expressed concerning the project’s positive impact on local jobs. In conclusion, all participants expressed their support for the project.

All hundred questionnaires were returned and summarized in the environmental impact assessment. The summary shows that 99/100 respondents supported the project, and one person had no opinion. Ninety respondents were either familiar, or knew a little bit about the project itself. In response to the question of what the project’s main benefits were, 64% named an improvement of the local economy, and 60% cited environmental protection. The main concern raised in terms of the project’s potential negative impacts was noise, with respondents requesting that measures be taken to limit possible noise pollution.

E.3.	Report on how due account was taken of any comments received:

The only negative comments received concerned the possibility of noise pollution. As described in section D.1 above, all necessary measures will be taken to maintain noise emissions below the legally required levels. This will be done by installing noise reduction equipment such as mufflers on engine exhaust pipes and sound insulation in engine workshops.

Annex 1

# CONTACT INFORMATION ON PARTICIPANTS IN THE PROJECT ACTIVITY

| Organization: | Yima Coal Industry (Group) Co., Ltd. |
| --- | --- |
| Street/P.O.Box: | 6, Qianqiutonglu |
| Building: |  |
| City: | Yima |
| State/Region: | Henan |
| Postfix/ZIP: | 472300 |
| Country: | China |
| Telephone: |  |
| FAX: |  |
| E-Mail: |  |
| URL: |  |
| Represented by: |  |
| Title: | Director |
| Salutation: | Mr. |
| Last Name: | Pei |
| Middle Name: |  |
| First Name: | Changhe |
| Department: | Gas drainage and ventilation air |
| Mobile: | +86 139 3980 5738 |
| Direct FAX: |  |
| Direct tel: | +86 398 589 8301 |
| Personal E-Mail: | peichanghe@yimeijt.com, lsy161@sohu.com |

| Organization: | EDF TRADING Limited |
| --- | --- |
| Street/P.O.Box: | 71 High Holborn |
| Building: |  |
| City: | London |
| State/Region: |  |
| Postfix/ZIP: | WC1V 6ED |
| Country: | United Kingdom |
| Telephone: | +44 207 061 4000 |
| FAX: | +44 207 061 5000 |
| E-Mail: |  |
| URL: | www.edftrading.com |
| Represented by: |  |
| Title: | Vice Head of Origination and Corporate Development |
| Salutation: | Mr. |
| Last Name: | JOUBERT |
| Middle Name: |  |
| First Name: | François |
| Department: |  |
| Mobile: |  |
| Direct FAX: | +44 207 061 5208 |
| Direct tel: | +44 207 061 4208 |
| Personal E-Mail: | francois.joubert@edftrading.com |

Annex 2

# INFORMATION REGARDING PUBLIC FUNDING

No public funding was provided for this project activity.

Annex 3

BASELINE INFORMATION

Table 17: Project Financial Analysis

1. Calculation of Operating Margin (OM) of Central China Electricity Grid

Table 18: Electricity generation of the Central China Power Grid in 2003

| Province | Power Generation | Power Generation | Self-power consumption rate | Power supply |
| --- | --- | --- | --- | --- |
|  | （108kWh) | （MWh) | （%） | （MWh) |
| Jiangxi | 271.65 | 27,165,000 | 6.43 | 25,418,291 |
| Henan | 955.18 | 95,518,000 | 7.68 | 88,182,218 |
| Hubei | 395.32 | 39,532,000 | 3.81 | 38,025,831 |
| Hunan | 295.01 | 29,501,000 | 4.58 | 28,149,854 |
| Chongqing | 163.41 | 16,341,000 | 8.97 | 14,875,212 |
| Sichuan | 327.82 | 32,782,000 | 4.41 | 31,336,314 |
| Total |  |  |  | 225,987,719 |

Source: China Electricity Yearbook 2004

Table 19: Calculation of simple OM emission factor of the Central China Power Grid in 2003

| Fuel Type | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total | Emission Factor | Oxidation rate | Average net calorific value | CO2 emissions（tCO2e） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | （tc/TJ） | （%） | （MJ/t,km3 | K=G*H*I*J*44/12/10000（mass unit） |
|  |  | A | B | C | D | E | F | G=A+B+C+D+E+F | H | I | J | K=G*H*I*J*44/12/1000 (volume unit) |
| Raw coal | Mtons | 1427.41 | 5504.94 | 2072.44 | 1646.47 | 769.47 | 2430.93 | 13851.66 | 25.8 | 100 | 20908 | 273,971,540 |
| Cleaned Coal | Mtons |  |  |  |  |  |  | 0 | 25.8 | 100 | 26344 | 0 |
| Other Washed Coal | Mtons | 2.03 | 39.63 |  |  | 106.12 |  | 147.78 | 25.8 | 100 | 8363 | 1,169,146 |
| Coke | Mtons |  |  |  | 1.22 |  |  | 1.22 | 25.8 | 100 | 28435 | 32,817 |
| Coke Oven Gas | 109m3 |  |  | 0.93 |  |  |  | 0.93 | 12.1 | 100 | 16726 | 69,013 |
| Other Coke Gas | 109m3 |  |  |  |  |  |  | 0 | 12.1 | 100 | 5227 | 0 |
| Crude Oil | Mtons |  | 0.5 | 0.24 |  |  | 1.2 | 1.94 | 20 | 100 | 41816 | 59,490 |
| Gasoline | Mtons |  |  |  |  |  |  | 0 | 18.9 | 100 | 43070 | 0 |
| Diesel Oil | Mtons | 0.52 | 2.54 | 0.69 | 1.21 | 0.77 |  | 5.73 | 20.2 | 100 | 42652 | 181,016 |
| Fuel oil | Mtons | 0.42 | 0.25 | 2.17 | 0.54 | 0.28 | 1.2 | 4.86 | 21.1 | 100 | 41816 | 157,229 |
| LPG | Mtons |  |  |  |  |  |  | 0 | 17.2 | 100 | 50179 | 0 |
| Refinery Gas | Mtons | 1.76 | 6.53 |  | 0.66 |  |  | 8.95 | 18.2 | 100 | 46055 | 275,070 |
| Natural gas | 10 9 m 3 |  |  |  |  | 0.04 | 2.2 | 2.24 | 15.3 | 100 | 38931 | 489,223 |
| Other petroleum products | Mtons |  |  |  |  |  |  | 0 | 20 | 100 | 38369 | 0 |
| Other coking products | Mtons |  |  |  |  |  |  | 0 | 25.8 | 100 | 28435 | 0 |
| Other energy | Mtons standard coal |  | 11.04 |  |  | 16.2 |  | 27.24 | 0 | 100 | 0 | 0 |
|  |  |  |  |  |  |  |  |  |  |  | Total | 276,404,544 |
| Total emissions of Central China Power Grid (tCO2e) |  |  |  |  |  |  |  |  |  |  |  | 276,404,544 |
| Total thermal power supply of the Central China Power grid (MWh) |  |  |  |  |  |  |  |  |  |  |  | 225,987,719 |
| OM of the Central China Power grid (tCO2e/MWh) |  |  |  |  |  |  |  |  |  |  |  | 1.2231 |

Source: China Electricity Yearbook 2004

Table 20: Electricity generation of the Central China Power Grid in 2004

| Province | Power Generation | Power Generation | Self-power consumption rate | Power supply |
| --- | --- | --- | --- | --- |
|  | （108kWh) | （MWh) | （%） | （MWh) |
| Jiangxi | 301.27 | 30,127,000 | 7.04 | 28,006,059 |
| Henan | 1093.52 | 109,352,000 | 8.19 | 100,396,071 |
| Hubei | 430.34 | 43,034,000 | 6.58 | 40,202,363 |
| Hunan | 371.86 | 37,186,000 | 7.47 | 34,408,206 |
| Chongqing | 165.2 | 16,520,000 | 11.06 | 14,692,888 |
| Sichuan | 346.27 | 34,627,000 | 9.41 | 31,368,599 |
| Total |  |  |  | 249,074,186 |

Source: China Electricity Yearbook 2005

Table 21: Calculation of simple OM emission factor of the Central China Power Grid in 2004

| Fuel Type | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total | Emission Factor | Oxidation rate | Average net calorific value | CO2 emissions（tCO2e） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | （tc/TJ） | （%） | （MJ/t,km3） | K=G*H*I*J*44/12/10000（mass unit） |
|  |  | A | B | C | D | E | F | G=A+B+C+D+E+F | H | I | J | K=G*H*I*J*44/12/1000 (volume unit) |
| Raw coal | Mtons | 1863.8 | 6948.5 | 2510.5 | 2197.9 | 875.5 | 2747.9 | 17144.1 | 25.8 | 100 | 20,908 | 339,092,605 |
| Cleaned Coal | Mtons |  | 2.34 |  |  |  |  | 2.34 | 25.8 | 100 | 26,344 | 58,316 |
| Other Washed Coal | Mtons | 48.93 | 104.22 |  |  | 89.72 |  | 242.87 | 25.8 | 100 | 8,363 | 1,921,441 |
| Coke | Mtons |  | 109.61 |  |  |  |  | 109.61 | 25.8 | 100 | 28,435 | 2,948,455 |
| Coke Oven Gas | 109m3 |  |  | 1.68 |  | 0.34 |  | 2.02 | 12.1 | 100 | 16,726 | 149,900 |
| Other Coke Gas | 109m3 |  |  |  |  | 2.61 |  | 2.61 | 12.1 | 100 | 5,227 | 60,527 |
| Crude Oil | Mtons |  | 0.86 | 0.22 |  |  |  | 1.08 | 20 | 100 | 41,816 | 33,118 |
| Gasoline | Mtons |  | 0.06 |  |  | 0.01 |  | 0.07 | 18.9 | 100 | 43,070 | 2,089 |
| Diesel Oil | Mtons | 0.02 | 3.86 | 1.7 | 1.72 | 1.14 |  | 8.44 | 20.2 | 100 | 42,652 | 266,627 |
| Fuel oil | Mtons | 1.09 | 0.19 | 9.55 | 1.38 | 0.48 | 1.68 | 14.37 | 21.1 | 100 | 41,816 | 464,893 |
| LPG | Mtons |  |  |  |  |  |  | 0 | 17.2 | 100 | 50,179 | 0 |
| Refinery Gas | Mtons | 3.52 | 2.27 |  |  |  |  | 5.79 | 18.2 | 100 | 46,055 | 177,950 |
| Natural gas | 10 9 m 3 |  |  |  |  |  | 2.27 | 2.27 | 15.3 | 100 | 38,931 | 495,775 |
| Other petroleum products | Mtons |  |  |  |  |  |  | 0 | 20 | 100 | 38,369 | 0 |
| Other coking products | Mtons |  |  |  |  |  |  | 0 | 25.8 | 100 | 28,435 | 0 |
| Other energy | Mtons standard coal |  | 16.92 |  | 15.2 | 20.95 |  | 53.07 | 0 | 100 | 0 | 0 |
|  |  |  |  |  |  |  |  |  |  |  | Total | 345,671,697 |
| Total emissions of Central China Power Grid (tCO2e) |  |  |  |  |  |  |  |  |  |  |  | 345,671,697 |
| Total thermal power supply of the Central China Power grid (MWh) |  |  |  |  |  |  |  |  |  |  |  | 249,074,186 |
| OM of the Central China Power grid (tCO2e/MWh) |  |  |  |  |  |  |  |  |  |  |  | 1.3878 |

Source: China Electricity Yearbook 2005

Table 22: Electricity generation of the Central China Power Grid in 2005

| Province | Power Generation | Power Generation | Self-power consumption rate | Power supply |
| --- | --- | --- | --- | --- |
|  | （108kWh) | （MWh) | （%） | （MWh) |
| Jiangxi | 300 | 30,000,000 | 6.48 | 28,056,000 |
| Henan | 1315.9 | 131,590,000 | 7.32 | 121,957,612 |
| Hubei | 477 | 47,700,000 | 2.51 | 46,502,730 |
| Hunan | 399 | 39,900,000 | 5 | 37,905,000 |
| Chongqing | 175.84 | 17,584,000 | 8.05 | 16,168,488 |
| Sichuan | 372.02 | 37,202,000 | 4.27 | 35,613,475 |
| Total |  |  |  | 286,203,305 |

Source: China Electricity Yearbook 2006

Table 23: Calculation of simple OM emission factor of the Central China Power Grid in 2005

| Fuel Type | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total | Emission Factor | Oxidation rate | Average net calorific value | CO2 emissions（tCO2e） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | （tc/TJ） | （%） | （MJ/t,km3） | K=G*H*I*J*44/12/10000（mass unit） |
|  |  | A | B | C | D | E | F | G=A+B+C+D+E+F | H | I | J | K=G*H*I*J*44/12/1000 (volume unit) |
| Raw coal | Mtons | 1869.29 | 7638.87 | 2732.15 | 1712.27 | 875.4 | 2999.77 | 17827.75 | 25.8 | 100 | 20,908 | 352,614,497 |
| Cleaned Coal | Mtons | 0.02 |  |  |  |  |  | 0.02 | 25.8 | 100 | 26,344 | 498 |
| Other Washed Coal | Mtons |  | 138.12 |  |  | 89.99 |  | 228.11 | 25.8 | 100 | 8,363 | 1,804,669 |
| Coke | Mtons |  | 25.95 |  | 105 |  |  | 130.95 | 25.8 | 100 | 28,435 | 3,522,491 |
| Coke Oven Gas | 109m3 |  |  | 1.15 |  | 0.36 |  | 1.51 | 12.1 | 100 | 16,726 | 112,054 |
| Other Coke Gas | 109m3 |  | 10.2 |  |  | 3.12 |  | 13.32 | 12.1 | 100 | 5,227 | 308,897 |
| Crude Oil | Mtons |  | 0.82 | 0.36 |  |  |  | 1.18 | 20 | 100 | 41,816 | 36,185 |
| Gasoline | Mtons |  | 0.02 |  |  | 0.02 |  | 0.04 | 18.9 | 100 | 43,070 | 1,194 |
| Diesel Oil | Mtons | 1.3 | 3.03 | 2.39 | 1.39 | 1.38 |  | 9.49 | 20.2 | 100 | 42,652 | 299,798 |
| Fuel oil | Mtons | 0.64 | 0.29 | 3.15 | 1.68 | 0.89 | 2.22 | 8.87 | 21.1 | 100 | 41,816 | 286,959 |
| LPG | Mtons |  |  |  |  |  |  | 0 | 17.2 | 100 | 50,179 | 0 |
| Refinery Gas | Mtons | 0.71 | 3.41 | 1.76 | 0.78 |  |  | 6.66 | 18.2 | 100 | 46,055 | 204,689 |
| Natural gas | 10 9 m 3 |  |  |  |  |  | 3 | 3 | 15.3 | 100 | 38,931 | 655,209 |
| Other petroleum products | Mtons |  |  |  |  |  |  | 0 | 20 | 100 | 38,369 | 0 |
| Other coking products | Mtons |  |  |  | 1.5 |  |  | 1.5 | 25.8 | 100 | 28,435 | 40,349 |
| Other energy | Mtons standard coal |  | 2.88 |  | 1.74 | 32.8 |  | 37.42 | 0 | 100 | 0 | 0 |
|  |  |  |  |  |  |  |  |  |  |  | Total | 359,887,488 |
| Total emissions of Central China Power Grid (tCO2e) |  |  |  |  |  |  |  |  |  |  |  | 359,887,488 |
| Total thermal power supply of the Central China Power grid (MWh) |  |  |  |  |  |  |  |  |  |  |  | 286,203,305 |
| OM of the Central China Power grid (tCO2e/MWh) |  |  |  |  |  |  |  |  |  |  |  | 1.2575 |

Source: China Electricity Yearbook 2006

EFOM,y=(276,404,544+345,671,697+359,887,488)(225,987,719+249,074,186+286,203,305)=1.2899tCO2eMWh

2. Calculation of Build Margin (BM) of Central China Electricity Grid

Table 24: Calculation of coal-fired, oil-fired and gas-fired emission factor

|  | Power supply efficiency | CO2 emission factor (tC/TJ) | Oxidation rate | Emission Factor (tCO2/MWh) |
| --- | --- | --- | --- | --- |
|  | A | B | C | D=3.6/A/1000*B*C*44/12 |
| EFCoal,Adv | 35.82% | 25.8 | 1 | 0.9508 |
| EFGas,Adv | 47.67% | 15.3 | 1 | 0.4237 |
| EFOil,Adv | 47.67% | 21.1 | 1 | 0.5843 |

Table 25: Installed Capacity of Central China Power Grid in 2005

| Installed capacity | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coal | MW | 5,906 | 26,268 | 9,526 | 7,212 | 3,760 | 7,496 | 60,167 |
| Hydro | MW | 3,019 | 2,540 | 8,089 | 7,905 | 1,893 | 14,960 | 38,405 |
| Nuclear | MW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other | MW | 0 | 0 | 0 | 0 | 24 | 0 | 24 |
| Total | MW | 8,925 | 28,808 | 17,615 | 15,117 | 5,676 | 22,456 | 98,596 |

Source: China Electricity Yearbook 2006

Table 26: Installed Capacity of Central China Power Grid in 2003

| Installed capacity | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coal | MW | 5,408 | 17,636 | 8,173 | 6,447 | 3,126 | 6,104 | 46,894 |
| Hydro | MW | 2,307 | 2,438 | 7,337 | 6,603 | 1,330 | 12,342 | 32,357 |
| Nuclear | MW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other | MW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | MW | 7,715 | 20,074 | 15,511 | 13,050 | 4,456 | 18,446 | 79,251 |

Source: China Electricity Yearbook 2004

Table 27: Installed Capacity of Central China Power Grid in 2002

| Installed capacity | Unit | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coal | MW | 5,129 | 15,905 | 8,148 | 4,976 | 3,005 | 6,142 | 43,303 |
| Hydro | MW | 2,197 | 2,438 | 7,214 | 6,135 | 1,196 | 11,855 | 31,035 |
| Nuclear | MW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Other | MW | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total | MW | 7,326 | 18,343 | 15,362 | 11,111 | 4,200 | 17,997 | 74,338 |

Source: China Electricity Yearbook 2003

Table 28: Ratio of CO2 emissions among solid fuel, liquid fuel and gas fuel

|  |  | Jiangxi | Henan | Hubei | Hunan | Chongqing | Sichuan | Total | Average Low Calorific Value | Emission Factor (tC/TJ) | Oxidation rate | CO2 emissions (tCO2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel Type | Unit | A | B | C | D | E | F | G=A+…+F | H | I | J | K=G*H*I*J*44/12/100 |
| Raw coal | Mtons | 1869.29 | 7638.87 | 2732.15 | 1712.27 | 875.4 | 2999.77 | 17827.75 | 20908 | 25.8 | 1.00 | 352,614,497 |
| Cleaned coal | Mtons | 0.02 | 0 | 0 | 0 | 0 | 0 | 0.02 | 26344 | 25.8 | 1.00 | 498 |
| Other washed coal | Mtons | 0 | 138.12 | 0 | 0 | 89.99 | 0 | 228.11 | 8363 | 25.8 | 1.00 | 1,804,669 |
| Coke | Mtons | 0 | 25.95 | 0 | 106.5 | 0 | 0 | 132.45 | 28435 | 25.8 | 1.00 | 3,562,840 |
| Sub-total |  |  |  |  |  |  |  |  |  |  |  | 357,982,504 |
| Crude oil | Mtons | 0 | 0.82 | 0.36 | 0 | 0 | 0 | 1.18 | 41816 | 20 | 1.00 | 36,185 |
| Gasoline | Mtons | 0 | 0.02 | 0 | 0 | 0.02 | 0 | 0.04 | 43070 | 18.9 | 1.00 | 1,194 |
| Kerosene | Mtons | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 43070 | 19.6 | 1.00 | 0 |
| Diesel oil | Mtons | 1.3 | 3.03 | 2.39 | 1.39 | 1.38 | 0 | 9.49 | 42652 | 20.2 | 1.00 | 299,798 |
| Fuel oil | Mtons | 0.64 | 0.29 | 3.15 | 1.68 | 0.89 | 2.22 | 8.87 | 41816 | 21.1 | 1.00 | 286,959 |
| Other petroleum products | Mtons | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 38369 | 20 | 1.00 | 0 |
| Sub-total |  |  |  |  |  |  |  |  |  |  |  | 624,136 |
| Natural gas | 107m3 | 0 | 0 | 0 | 0 | 0 | 30 | 30 | 38931 | 15.3 | 1.00 | 655,209 |
| Coke oven gas | 107m3 | 0 | 0 | 11.5 | 0 | 3.6 | 0 | 15.1 | 16726 | 12.1 | 1.00 | 112,054 |
| Other coke gas | 107m3 | 0 | 102 | 0 | 0 | 31.2 | 0 | 133.2 | 5227 | 12.1 | 1.00 | 308,897 |
| LPG | Mtons | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 50179 | 17.2 | 1.00 | 0 |
| Refinery gas | Mtons | 0.71 | 3.41 | 1.76 | 0.78 | 0 | 0 | 6.66 | 46055 | 18.2 | 1.00 | 204,689 |
| Sub-total |  |  |  |  |  |  |  |  |  |  |  | 1,280,848 |
| Total |  |  |  |  |  |  |  |  |  |  |  | 359,887,488 |

λCoal= 99.47%; λOil= 0.17%; λGas= 0.36%

EFThermal=λCoal´EFCoal,Adv+λOil´EFOil,Adv´λGas´EFGas,Adv=0.9486

Table 29: Calculation of BM of Central China Electricity Grid

|  | Installed capacity in 2002 | Installed capacity in 2003 | Installed capacity in 2005 | New capacity additions 2002-2005 | Share of capacity additions |
| --- | --- | --- | --- | --- | --- |
|  | A | B | C | D=C-A |  |
| Coal (MW) | 43,303 | 46,894 | 60,167 | 16,864 | 69.52% |
| Hydro (MW) | 31,035 | 32,357 | 38,405 | 7,371 | 30.38% |
| Nuclear (MW) | 0 | 0 | 0 | 0 | 0.00% |
| Wind (MW) | 0 | 0 | 24 | 24 | 0.10% |
| Total (MW) | 74,338 | 79,251 | 98,596 | 24,259 | 100.00% |
| Share of installed capacity in 2005 | 75.40% | 80.38% | 100.00% |  |  |

EFBM,y=CAPThermalCAPTotal´EFAdv,Thermal		(20)

EFBM = 0.9486 x 69.52% = 0.6594 tCO2e/MWh

3. Calculation of combined margin emission factor of China Central Electricity Grid

EFELEC = 0.5 x 1.2899 + 0.5 x 0.6594 = 0.9747 tCO2e/MWh

Evidence that CDM was considered prior to the start of the project activity:

Annex 4

MONITORING INFORMATION

- No additional information. See monitoring plan above.

Based on a preliminary site assessment, Mengjin is the most promising site for the installation of the Vocsidizer unit. However, it is possible that other sites within the already defined project boundaries may be chosen for installation if their conditions are more favourable. Choosing another location would not affect the estimated CERs or other aspects of this PDD.

Mazumder et al (2002). CO2 Injection in Coal Seams for Sequestration and Enhanced Methane Recovery. Delft University of Technology, Netherlands. Available at: http://www.nwo.nl/nwohome.nsf/pages/NWOP_5WCGTL/$file/TransitionPreSaikat21nov02.pdf

Observed during visits to Hebi, Yima, Zhengzhou, Jiaozuo and Pingdingshan coalmine groups in 2005, 2006 and 2007.

In addition to the present project, NDRC and provincial DRC approval has been given to coalmine methane projects developed by by Pingdingshan Coal (Group) Co., Ltd., Zhengzhou Coal Industry (Group) Co., Ltd., and Hebi Coal Industry (Group) Co., Ltd. in Henan province. All of these projects employ the Shengdong engine technology for low-concentration CMM utilization.

Information obtained by MEGTEC. Pilot projects have been carried out in the UK, the United States and more recently, Australia. With the exception of the Australian site, VAM volumes were significantly smaller than those likely to be processed in the present project.

Westlake A, Creedy D P, Hu Yuhong. CDM, a Trigger for Coal Mine Methane Emission Reductions in China, 2003. Available at: http://www.coalinfo.net.cn/coalbed/meeting/2203/papers/coal-mining/005.pdf

Vrolijk, C. et al. Enhancing Coal Mine Methane Utilisation in China. Report No. COAL R298 DTI/Pub URN05/1816, 2005. Available at: www.berr.gov.uk/files/file29223.pdf

China Coal Information Institute (CCII), New Development of CMM Projects in China, The 5th International Symposium on CBM/CMM in China, Beijing, China, November 30 – December 2, 2005; cited in Jiangxi Fengcheng Mining Administration CMM Utilization Project.

Ibid.

The five most important mine groups in Henan province are Pingdingshan, Hebi, Zhengzhou, Yima and Jiaozuo Coal Group Industries. All five groups are currently preparing CMM utilization projects under the CDM (information obtained from onsite visits in 2005).

Information obtained from visits to all five major mine groups in 2005.

Kester, G.W. et al. (1999). Capital Budgeting Practices in the Asia-Pacific Region: Australia, Hong Kong, Indonesia, Malaysia, Philippines, and Singapore. Financial Practice and Education.

Available at: http://207.36.165.114/JAF/fpess993.pdf

Cited in Huainan Panyi and Xieqiao Coal Mine Methane Utilization Project, version 4, 2006; p. 17.

Cited in ibid., p. 17.

Lagman, A.S.; Aylward, B. (2000). Survey of Multilateral Bank Practice on Financial and Economic Analysis of Large Dams. Secretariat of the World Commission on Dams, Available at: http://www.wca-infonet.org/servlet/BinaryDownloaderServlet?filename=1066822636382_bank.pdf

Brennand, T.P. Wind Energy in China: Policy Options for Development, in Energy for Sustainable Development, Volume V. No. 4, December 2001. Available at: http://www.ieiglobal.org/ESDVol5No4/windpolicy.pdf

Estache, A.; Pinglo, M.E. Are Returns To Private Infrastructure In Developing Countries Consistent With Risks Since The Asian Crisis? World Bank Policy Research Working Paper 3373, August 2004. Available at: http://unpan1.un.org/intradoc/groups/public/documents/APCITY/UNPAN019815.pdf

Although the project incorporates six different mines, the financial return is only calculated for the project as a whole. All six mines belong to YCIG, and the investment decision is made with reference to the project as a whole. This is also confirmed by the fact that there is only one feasibility report approved by Chinese authorities.

See: Westlake A, Creedy D P, Hu Yuhong. CDM, a Trigger for Coal Mine Methane Emission Reductions in China, 2003. Available at: http://www.coalinfo.net.cn/coalbed/meeting/2203/papers/coal-mining/005.pdf

See: Vrolijk, C. et al. Enhancing Coal Mine Methane Utilisation in China. Report No. COAL R298 DTI/Pub URN05/1816, 2005. Available at: www.berr.gov.uk/files/file29223.pdf

Mazumder et al (2002). CO2 Injection in Coal Seams for Sequestration and Enhanced Methane Recovery. Delft University of Technology, Netherlands. Available at: http://www.nwo.nl/nwohome.nsf/pages/NWOP_5WCGTL/$file/TransitionPreSaikat21nov02.pdf

Site visits to Pingdingshan, Zhengzhou, Yima and Hebi coal groups in 2005 and 2006.

Visits to Yima, Zhengzhou, Hebi, Pingdingshan, and Jiaozuo coalmine groups in 2005 and 2006.

Visits to Yima, Zhengzhou, Hebi, Pingdingshan, and Jiaozuo coalmine groups in 2005 and 2006.

http://cdm.unfccc.int/UserManagement/FileStorage/AM_CLAR_QEJWJEF3CFBP1OZAK6V5YXPQKK7WYJ

These values are based on an estimate provided by the equipment suppliers for this type of installation. The above estimate is conservative because it assumes that all gas engine ancillary equipment will be operational from the beginning of the project, but in reality it will be phased in over at least a year.