VALIDATION REPORT

Nava Bharat Ventures Ltd

NAVA BHARAT RE BAGASSE PROJECT

SGS Climate Change Programme

SGS United Kingdom Ltd

SGS House

217-221 London Road

Camberley Surrey

GU15 3EY

United Kingdom

| Date of issue: |  | Project No.: |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01/08/2007 |  | CDM.Val0967 |  |  |  |
| Project title |  | Organisational unit: |  |  |  |
| Nava Bharat RE Bagasse Project |  | SGS Climate Change Programme |  |  |  |
| Revision number |  | Client: |  |  |  |
| 01 |  | Nava Bharat Ventures Ltd |  |  |  |
| Summary SGS India Pvt. Ltd., an affiliate of SGS United Kingdom Ltd. has made a validation of the CDM project activity “Nava Bharat RE Bagasse Project” by Nava Bharat Ventures Ltd, on the basis of UNFCCC criteria for the CDM, as well as criteria given to provide for consistent project operations, monitoring and reporting. UNFCCC criteria refer to Article 12 of the Kyoto Protocol, the CDM rules and modalities and the subsequent decisions by the CDM Executive Board, as well as the host country criteria. The scope of validation is the independent and objective review of the project design document, baseline study and monitoring plan and other relevant document of the project. The information in this document is reviewed against the criteria defined in the Marrakech Accords (Decision 17) and the Kyoto Protocol (Article 12) and subsequent guidance from the CDM Executive Board. The validation is not meant to provide any consulting towards the Client. However, stated requests for clarifications plan and/or corrective actions may provide input for improvement of the project design document (PDD). The overall validation process, from Contract Review to Validation Report & Opinion, was conducted using internal procedures (UK.PP.12 issue 3 dated 19/01/2007). The first output of the validation process is a list of Corrective Actions Requests and New Information Requests (CAR and NIR), presented in Annex 2 of this document. Taking into account this output, the project proponent revised its project design document. In summary, it is SGS’s opinion that the proposed CDM project activity correctly applies the baseline and monitoring methodology as mentioned in approved methodology adopted for the proposed project activity and meets the relevant UNFCCC requirements for the CDM and the relevant host country criteria. |  |  |  |  |  |
| Subject.: Subject Group: |  |  |  |  |  |
| CDM validation |  |  |  | Indexing terms |  |
| Work carried out by |  |  |  |  |  |
| Mr. Sanjeev Kumar - Team Leader Mr. Nikunj Agarwal – Local Assessor |  |  |  |  |  |
| Technical review |  |  |  |  |  |
| Mr. – Jochen Gross |  |  |  | No distribution without permission from the Client or responsible organisational unit |  |
| Authorized signatory |  |  |  |  |  |
| Siddharth Yadav |  |  |  | Limited distribution |  |
| Date of final decision: 16/10/07 | Number of pages: |  |  |  |  |
|  | 44 |  |  | Unrestricted distribution |  |

Abbreviations

| CAR | Corrective Action Request |
| --- | --- |
| CDM | Clean Development Mechanism |
| CEA | Central Electricity Authority |
| CER | Certified Emission Reductions |
| CO2 | Carbon Dioxide |
| DNA | Designated National Authority |
| DOE | Designated Operational Entity |
| DR | Document Review |
| EIA | Environment Impact Assessment |
| GHG | Green House Gas(es) |
| I | Interview |
| IPCC | Intergovernmental Panel on Climate Change |
| ISHC | International Stakeholder Consultation |
| kWh | Kilo watt hour |
| MNES | Ministry of Non Conventional Energy Sources |
| MoEF | Ministry of Environment and Forest |
| MoV | Means of Verification |
| MP | Monitoring Plan |
| MWh | Mega watt hour |
| MT | Metric Tonne |
| NIR | New Information Request |
| PDD | Project Design Document |
| PPA | Power Purchase Agreement |
| UNFCCC | United Nations Framework Convention for Climate Change |

DG		  Deisel Generator

# Table of content

1.	Introduction	6

1.1	Objective	6

1.2	Scope	6

1.3	GHG Project Description	6

1.4	The names and roles of the validation team members	7

2.	Methodology	7

2.1	Review of CDM-PDD and additional documentation	8

2.2	Use of the validation protocol	8

2.3	Findings	8

2.4	Internal quality control	9

3.	Determination Findings	10

3.1	Participation requirements	10

3.2	Baseline selection and additionality	10

3.3	Application of Baseline methodology and calculation of emission factors	11

3.4	Application of Monitoring methodology and Monitoring Plan	11

3.5	Project design	12

3.6	Environmental Impacts	13

3.7	Local stakeholder comments	13

4.	Comments by Parties, Stakeholders and NGOs	13

4.1	Description of how and when the PDD was made publicly available	14

4.2	Compilation of all comments received	14

4.3	Explanation of how comments have been taken into account	15

5.	Validation opinion	20

6.	List of persons interviewed	21

7.	Document references	22

Annex 1: Local assessment

Annex 2: Validation Protocol

Annex 3: Overview of findings

Annex 4: Statement of Competency of Validation Team Members

# Introduction

## Objective

Nava Bharat Ventures Ltd has commissioned SGS to perform the validation of the project: “Nava Bharat RE Bagasse Project” with regard to the relevant requirements for CDM project activities. The purpose of a validation is to have an independent third party assess the project design. In particular, the project's baseline, the monitoring plan (MP) and the project’s compliance with relevant UNFCCC and host country criteria are validated in order to confirm that the project design as documented is sound and reasonable and meets the stated requirements and identified criteria. Validation is seen as necessary to provide assurance to stakeholders of the quality of the project and its intended generation of Certified Emission Reduction (CER). UNFCCC criteria refer to the Kyoto Protocol criteria and the CDM rules and modalities and related decisions by the COP/MOP and the CDM Executive Board.

## Scope

The scope of the validation is defined as an independent and objective review of the project design document, the project’s baseline study and monitoring plan and other relevant documents. The information in these documents is reviewed against Kyoto Protocol requirements, UNFCCC rules and associated interpretations. SGS has employed a risk-based approach in the validation, focusing on the identification of significant risks for project implementation and the generation of CERs.

The validation is not meant to provide any consulting towards the Client. However, stated requests for clarifications and/or corrective actions may provide input for improvement of the project design.

## GHG Project Description

The project activity consists of the installation of 9MW double extraction cum condensing turbine generator.  The steam for operation of the turbine will be provided by the existing Thermax 43 kg/cm2 pressure, 410±15oC temperature, 64 tonnes per hour capacity boiler.  The boiler and hence turbine generator will be powered by the combustion of bagasse, a co-product of the sugar production process and will therefore be a renewable carbon neutral source of electricity. The export of surplus electricity to the regional grid will thus lead to a reduction in greenhouse gas emission through the substitution of the predominantly fossil fuel electricity generation in the grid.

Baseline Scenario:

Under the baseline scenario, the project activity thus displaces equivalent amount of electricity from grid which is predominantly generated from thermal (fossil fuel based) power plants.

With Project Scenario:

The project activity uses biomass as fuel for generation of power, which in turn contributes to conservation of fossil fuel, a non-renewable natural resource and also reduces GHG emissions.

Leakage:

As per the methodology AMS.I.D. ; The energy generating equipment(s) is not transferred from another activity, thus no leakage is to be considered.

Environmental & Social Impacts:

According to local assessor, there is no negative environmental and social impact expected due to the project activity.

## The names and roles of the validation team members

| Name | Affiliate | Role |
| --- | --- | --- |
| Sanjeev Kumar | SGS India | Team Leader / Lead Auditor |
| Nikunj Agarwal | SGS India | Local Assessor |
| Jochen Gross | SGS Germany | Technical reviewer |

Statement of Competency of the team members are attached at Annex IV

# Methodology

## Review of CDM-PDD and additional documentation

The validation is performed primarily as a document review of the publicly available project documents. The assessment is performed by trained assessors using a validation protocol.

A site visit is usually required to verify assumptions in the baseline. Additional information can be required to complete the validation, which may be obtained from public sources or through telephone and face-to-face interviews with key stakeholders (including the project developers and Government and NGO representatives in the host country). These may be undertaken by the local SGS affiliate. The results of this local assessment are summarized in Annex 1 to this report.

## Use of the validation protocol

The validation protocol used for the assessment is partly based on the templates of the IETA / World Bank Validation and Verification Manual and partly on the experience of SGS with the validation of CDM projects. It serves the following purposes:

- it organises, details and clarifies the requirements the project is expected to meet; and

- it documents both how a particular requirement has been validated and the result of the validation.

The validation protocol consists of several tables. The different columns in these tables are described below.

| Checklist Question | Means of verification (MoV) | Comment | Draft and/or Final Conclusion |
| --- | --- | --- | --- |
| The various requirements are linked to checklist questions the project should meet. | Explains how conformance with the checklist question is investigated. Examples of means of verification are document review (DR) or interview (I). N/A means not applicable. | The section is used to elaborate and discuss the checklist question and/or the conformance to the question. It is further used to explain the conclusions reached. | This is either acceptable based on evidence provided (Y), or a Corrective Action Request (CAR) due to non-compliance with the checklist question (See below). New Information Request (NIR) is used when the validation team has identified a need for further clarification. |

The completed validation protocol for this project is attached as Annex 2 to this report

## Findings

As an outcome of the validation process, the team can raise different types of findings

In general, where insufficient or inaccurate information is available and clarification or new information is required the Assessor shall raise a New Information Request (NIR) specifying what additional information is required.

Where a non-conformance arises the Assessor shall raise a Corrective Action Request (CAR). A CAR  is issued, where:

- mistakes have been made with a direct influence on project results;

- validation protocol requirements have not been met; or

- there is a risk that the project would not be accepted as a CDM project or that emission reductions will not be verified.

The validation process may be halted until this information has been made available to the assessors’ satisfaction. Failure to address a NIR may result in a CAR. Information or clarifications provided as a result of an NIR may also lead to a CAR.

Observations may be raised which are for the benefit of future projects and future verification or validation actors. These have no impact upon the completion of the validation or verification activity.

Corrective Action Requests and New Information Requests are raised in the draft validation protocol and detailed in a separate form (Annex 3). In this form, the Project Developer is given the opportunity to “close” outstanding CARs and respond to NIRs and Observations.

## Internal quality control

Following the completion of the assessment process and a recommendation by the Assessment team, all documentation will be forwarded to a Technical Reviewer. The task of the Technical Reviewer is to check that all procedures have been followed and all conclusions are justified. The Technical Reviewer will either accept or reject the recommendation made by the assessment team.

# Determination Findings

## Participation requirements

The host Party for this project is India. India has ratified the Kyoto protocol on 26th Aug 2002. A Letter of Approval from DNA was not submitted by the project proponent. CAR (1) was raised asking project proponent to submit the Letter of approval from Indian DNA. The project proponent provided the letter dated 2nd April 2007; issued by the Indian DNA (reference number 4/2/2007-CCC) has been provided by the client which was verified from the original copy.

Project Proponent has identified U.K. as a Project Participant country and U.K. has ratified the Kyoto protocol on 31 May 2002 and the letter of approval has not been submitted by the U.K. DNA, CAR (1) was raised asking project proponent to submit the Letter of approval from DNA. The project proponent provided the letter dated 2nd May 2007; issued by the U.K. DNA (reference number AL/38/2007) has been provided by the client which was verified from the original copy.

Hence CAR (01) was closed out.

## Baseline selection and additionality

The baseline selected by the project proponent was the most likely baseline scenario. Now, the project has applied baseline as mentioned in the small scale methodology AMS. I.D. version 10 dated 23rd December 2006 as per the CDM project activities. The baseline scenario was selected from the methodology AMS. I.D version 10.

The project proponent has considered the carbon intensive fossil fuel based thermal power generation scenario of the southern regional grid as the most like baseline scenario for the project activity.

The project proponent has adopted the financial barrier supporting with Institutional barriers for the project activity. In order to get all the related documents on the basis of which the project was shown additional, CAR (10) was raised. In response of this CAR project proponent provide IRR calculation sheet along with supporting document for additionality.

The financial risks relate to the pricing of the main input, bagasse and the only output (aside from carbon credit revenues) electricity. Project Proponent has presented a financial analysis of the project with and without carbon credit revenues.

The project IRR without CDM revenue is 10.94%, and it increases from 10.94% to 15.67% with the inclusion of carbon credit revenues. The IRR with CDM cross the prime lending rate of bank 12.75% - 13.25% as verified with http://www.rbi.org.in/home.aspx#. This IRR with CDM assumes a final price of carbon credits of 10 Euro per tonne based on current discounted spot prices trading in the European market. Project proponent assumes a bagasse price of Rs 950/mt and a season length of 150 days and off-season of 75 days.  These assumptions are conservative given that bagasse prices last year were over Rs 1100/mt and the factory operated for 154 days.  Other assumptions are standard, such as operation, maintenance and salaries, which we assume will be 2.5% of the investment costs.

Other Barriers includes risks in the pricing of bagasse which is a substantial barrier to the project activity.  The factory has the option to sell bagasse on the open market, and bagasse prices have exhibited volatility in the recent past (as indicated above the actual price of bagasse in the year 2004/5 was Rs 1250/tonne).  High opportunity values for bagasse will make the project activity unviable.  Allied to the volatility of bagasse prices is the availability of bagasse.  The supply of bagasse to the project activity depends on the sugarcane yield which in turn depends on the rainfall and therefore if the supply of sugarcane is reduced it will impact the availability of bagasse and the factory will not be able to run for the expected 75 days in the off-season and season generation will be shortened, both of these factors will impact power generation and the financial viability of the project

The project proponent is claiming credits for seven years crediting period from date of registration.

Based on the findings above, it was concluded that the project activity was not a likely baseline scenario and hence additional to any that would occur in absence of project activity.

## Application of Baseline methodology and calculation of emission factors

The proposed CDM project activity is the power generation using biomass and uses baseline methodology as described under AMS. I.D. version 10 dated 23rd December 2006 as per small scale CDM project activities.

The detailed calculation worksheet for baseline emission factor was not clear and a CAR 8 was raised asking for a full reference. The project developer provided all the calculation excel sheets to verify all data and basis of the baseline emission factor calculations. All the supportive calculations regarding baseline (OM, BM and CM) and emission reductions calculation have been compiled together in supporting document folder. It was checked by the local assessor and found that the emission factors calculation was still not according to the methodology and local assessor again request the project proponent to recalculate the emission factor, the project proponent then decided that emission factor will be determined ex-post and for the purposes of calculations an EF value 0.86 tCO2e/MWh will be used.

The emission factor value 0.86 tCO2e/MWh used for the calculation of emission reduction in the PDD was conservative and near to the value published by CEA, hence this value of emission factor was accepted to DOE and DOE agreed that the project proponent will be determined emission factor ex-post for further crediting period. Hence this CAR 8 was closed out.

## Application of Monitoring methodology and Monitoring Plan

The present CDM project activity uses monitoring methodology as described in AMS. I.D. version 10 dated 23rd December 2006 as per small scale CDM project activities.

NIR11 was raised such as to get evidence about the surplus availability of bagasse and leakage. In response of this NIR project proponent replies that the project activity does not result in transfer of equipment and also there is surplus bagasse available at the project activity site.

It has been checked during site visit that energy generating equipment is not transferred from another activity and also the existing equipment is not transferred to another activity, the surplus availability of bagasse has been checked by verifying the past years of bagasse records which confirms that bagasse is not required from outside for this project activity, and if in future bagasse will be purchased from outside than the emission from the same will be deducted from final emission reduction, hence this NIR was closed out.

CAR13 was raised as there was no information about the quantity, quality and calorific value of the auxiliary fuel used in the boiler during boiler start up. In response of this CAR project proponent replies that the start up operation of the boiler relies on bagasse that has been stored from the previous season.  However fossil fuel consumption will be monitored.

No fossil fuel is being used during boiler start up operation; the same has been checked during site visit and also verified the same by communicating with boiler personnel and operator, however the consumption of any fossil fuel will be monitored and is now included in monitoring plan of the revised PDD, so CAR13 was closed out.

NIR 14 was raised as the monitoring plan mentioned in the PDD version 01, was not clear on the use of main meter & check meter, number of meters available for measuring power generated, auxiliary consumption, import and export of electricity from the grid system,. On the top of that nothing was mentioned about quantity, quality  and calorific value of auxiliary fuel used for calculation of the project emission, calorific value of bagasse used, leakage monitoring. The project proponent in their response clarifies that there will be no auxiliary fuel used in the boiler, however the consumption of auxiliary fuel will be monitored and is included in the monitoring plan of the revised PDD, Leakage is also not considered for the project activity, the calorific value of bagasse may be taken from the laboratory reports at the sugar factory, but we do not feel it is significant for the monitoring of the project activity. The import of power from the grid may occur in periods of emergency or during the off-season but this would be the case in the baseline and therefore is not included in the determination of the emission reductions. The DG sets will only be run in the off-season or during emergencies. The emissions from the DG sets are not considered because DG set comes under use only during the emergency situation and cannot be attributed to just the project activity, Further, the DG set is operated rarely and therefore emissions from this are not significant.

As per the site visit made by local assessor it was checked and confirmed by interviewing the boiler operator that Auxiliary fuel is not used in the boiler, however the consumption of auxiliary fuel will be monitored and is included in the monitoring plan of the revised PDD. The data for calorific value of bagasse can be taken by the laboratory as it is determined in the in-house laboratory. Since the project activity does not result in a transfer of equipment so the leakage is not considered and there is surplus bagasse available at the project activity site, so there is no need to purchase bagasse from outside sources, hence, the project emission due to transportation of bagasse can be ignored. The import of power from the grid is the case during periods of emergency or during the off-season but this would be the case in the baseline and therefore is not included in the determination of the emission reductions. The metering at the plant consists of an energy meter for export located on the panels in the power plant and energy meters at the sub-station. The same has been crosschecked during site visit. A detailed monitoring plan has now been included in the PDD which covers the monitoring requirements, roles and responsibilities, data collection and storage and the calibration of meters, all these are acceptable to DOE and hence NIR14 was closed out.

## Project design

The Project Design Document (PDD) was designed as per version 03 of guidelines laid for preparing PDD of small scale CDM project activity, hence the format of the present PDD was checked against it.

NIR 05 was raised asking project proponent to provide any documentary evidence that the present project technology will not be substituted or replaced by more efficient technologies during the crediting period. Project proponent has assured that project technology will not be substituted or replaced by more efficient technology during the crediting period and the letter of undertaking for the same has also been obtained from the project proponent. This was accepted and hence NIR05 was closed out.

NIR 06 was raised as there was no detail of extensive initial training for the project Activity in the PDD. In response of this NIR project proponent replies that the training of personnel at the plant was undertaken at the time of installation and was provided by the turbine supplier.

Project Proponent has submitted the Letter from the turbine supplier to the DOE which states that the training for maintenance and operation of turbine has been given to the staff and the same will be provided regularly when required. The same was accepted to DOE and hence the NIR6 was closed out.

The starting date of the project activity was not clear in the PDD version 01 and CAR 07 was raised, in response to this CAR, the project proponent provided a copy of purchase order document forwarded to the equipment supplier and explained that the project start date had been configured in accordance with the purchase order date for project equipment. The letter of intent for the purchase of the TG set has been submitted to the validator which was checked for the starting date of the project activity and 29/05/20004 was the date found on which the letter of intent for the purchase of TG set was issued. Hence 29/05/20004 was accepted as the starting date of the project activity and the same has been corrected in the revised PDD, hence this CAR was closed out.

The project boundary given in the PDD version 01 was not clear and hence NIR 12 was raised for the same. The project proponent made required corrections in the project boundary and provided a logical schematic diagram and the same are included in the revised PDD, this was also verified during site visit by the local assessor and hence NIR 12 was closed out.

## Environmental Impacts

NIR 09 was raised as there was no information regarding the EIA study has been provided with the version 01 of the PDD.

The compliance with local environmental regulations, The EIA Notification, 1994 and the revisions made there under, EIA requirement for the project activity was checked and also project proponent submitted consent to operate and establish from Andhra Pradesh State Pollution Control Board, a local authority responsible for giving Environmental Clearance. Though EIA is not required for this kind of project activity, however the project proponent have undertaken a Rapid EIA study, which have not revealed any negative environmental impact to be occurred by the project activity. The project proponent had mentioned the positive environmental impacts due to project activity, under section D in the PDD. This was also checked from the copy of report which was given by the project proponent to the local assessor. These were in compliance and even during local stakeholder consultation carried out by local assessor no negative comment was reported. Thus, NIR 09 was closed out.

## Local stakeholder comments

The local stakeholder consultation process mentioned in the PDD version 01 does not provid any clear impression about the stakeholders were consulted for project activity, thus a CAR 02 was raised asking  a full reference of governmental and non-governmental parties involved during the LSC meeting for the project activity.

The local stakeholders consulted for the project activity are the designated representatives of the local community, and Andhra Pradesh State Pollution Control Board. In response to the CAR 02, the project proponent has provided the copies of complete attendance list of the local stakeholder consultation meeting, NOC from Andhra Pradesh State Pollution Control Board, and NOC from local village community, the copies of the document were duly verified with the original copies and thus CAR 02 was closed out.

NIR 03 & NIR 04 were raised as the PDD version 01 was not clear about minutes of the LSC meeting and the summary of comments received by stakeholder consultation process.

The LSC meeting was organised on 14th November 2006 at the plant site and the public notice for the invitation of comments towards the project activity has been published in the local news paper on 16th October 2006 to invite interested person to attend the meeting and express their opinion no negative comment was appeared during consultation process. To ensure that the stakeholder process was conducted properly, the copies of paper advertisement for invitation to the LSC meeting, NOCs from State Pollution Control Board and PPA with Andhra Pradesh State Electricity Board were obtained and verified. The project participant also submitted one NOC from local village community to the validator. This was concluded that the LSC process was transparent and no adverse comment was raised on project activity. Thus, NIR 03 and 04 were closed out.

# Comments by Parties, Stakeholders and NGOs

In accordance with sub-paragraphs 40 (b) and (c) of the CDM modalities and procedures, the project design document of a proposed CDM project activity shall be made publicly available and the DOE shall invite comments on the validation requirements from Parties, stakeholders and UNFCCC accredited non-governmental organizations and make them publicly available. This chapter describes this process for this project.

## Description of how and when the PDD was made publicly available

The PDD and the monitoring plan for this project were made available on the UNFCCC website http://cdm.unfccc.int/Projects/Validation/DB/DHX7F9F99KKAFI11A8RWXMN2IU33T2/view.html from 22 February 2007 to 23 March 2007 and Comments were invited through the UNFCCC CDM homepage.

## Compilation of all comments received

| Comment number | Date received | Submitter | Comment |
| --- | --- | --- | --- |
| 1 | 13/03/07 | Name: Anamika Chatterjee City: Bangalore Country: India Organisation: Shiva Energy | Bagasse Based Power generation projects for export of power to grid are a common practice in India, particularly in South India. The project proponent is a late comer into the field. In fact, many plants around North East Andhra (eg. GMR) have already invested and operating similar projects, which are not registered/withdrawn under CDM. As per the 2005-06 SREB report, Andhra has a staggering 397 MW (1642 MUs) generated from bagasse cogen + biomass only. Surely, all 397 MW may not have gone for CDM. Further, Additionality of the project appears to be woefully weak. Just good English does not make good additionality. Hope UNFCCC takes note of this point and only genuine projects are registered under CDM. . |

The project was up loaded for International stakeholder consultation (ISHC) for a period of 30 days and received one comment.

## Explanation of how comments have been taken into account

Date: 13/03/07					Raised by: Anamika Chatterjee

| Comment | Issue | Ref |
| --- | --- | --- |
| 1.1 | Bagasse Based Power generation projects for export of power to grid are a common practice in India, particularly in South India. The project proponent is a late comer into the field. In fact, many plants around North East Andhra (eg. GMR) have already invested and operating similar projects, which are not registered/withdrawn under CDM. As per the 2005-06 SREB report, Andhra has a staggering 397 MW (1642 MUs) generated from bagasse cogen + biomass only. Surely, all 397 MW may not have gone for CDM. Further, Additionality of the project appears to be woefully weak. Just good English does not make good additionality. Hope UNFCCC takes note of this point and only genuine projects are registered under CDM. | 3.2 |
| Date: 20th April 2007 [Response from project developer] There are certain barriers to the establishment of bagasse based cogeneration plants. In the state of TN, for example, among 12 sugar factories that export electricity to the grid, there are 6 plants selling their bagasse and burning solely coal as a fuel. Of the remaining 6, three belong to the Thiru Arooran group, which developed a methodology (AM0007) specifically for bagasse based plants burning fossil fuels during the off-season, in association with the Prototype Carbon Fund and Winrock International India. The remaining three are small-scale projects in the co-operative sector. This highlights one of the most significant barriers to development of such plants, namely unavailability of biomass residues for cogeneration. In the state of Andhra Pradesh (AP), there are 37 sugar mills, 12 of which employ bagasse cogeneration plants with capacities varying from 1.5KW to 161.1MW, totaling to around 263MW. Amongst these, only the Ganpati co-generation project at Medak, Andhra Pradesh (Commissioning date: 29/04/2001) has been registered as a CDM project. There is another project, the16 MW Bagasse based cogeneration plant by GMR Industries Ltd (Commissioning date: 13/02/2000), but this has now been withdrawn from the CDM process due to modifications within the plant set-up. Thus, it is fair to say that the project activity is not common practice within the state. Biomass based plants registered as CDM in AP In the state of AP, as on January 2007, there are 14 biomass based power plants registered as CDM, leading to an approximate total capacity of 80MW. The SREB report 05-06 quotes a generation figure of 397MW, not just from bagasse cogeneration and biomass, but also from Isolated Gas (IG) wells. According to the January 2007 SRPC report, the total installed capacity for biomass based projects and biomass based cogeneration in AP is approximately 198 (178.5+20.43) MW, with the remaining 199MW distributed among bagasse based cogeneration(151.5MW), Municipal waste based power projects (12.6MW), Industrial waste based power projects (8.5MW) and Isolated Gas Wells (27.04MW) . Thus, it would be fair to say that biomass based plants exporting electricity to the grid are not common practice within the state. Tariff related barriers The guidelines on tariff rates for Non-conventional energy (NCE) projects were outlined by the Ministry of Non-Conventional Energy Sources (MNES), India. The MNES guidelines assume 1994-95 as the base year for energy purchase. For this year the tariff was set at 2.25 rupees / Unit. This was then to be escalated at a minimum rate of 5% per annum for 10 years. This leads to an approximate tariff rate of 3.6 rupees/unit for the year 2006-07. However, as per the Andhra Pradesh Electricity Regulatory Commission (APERC) Order R.P.No.84 / 2003 in O.P.No.1075 / 2000 dated 20th March 2004, the tariff rates for NCE projects have been revised and will be divided into two parts – fixed and variable. The fixed cost is based on the year of commissioning and it will come down gradually over a period of 10 years, whereas variable cost will go up by 5% every year up to a control period of 5 years. This leads to an approximate tariff rate of 2.75 rupees/unit for the year 2006-07. If we look at the Nava Bharat Project in particular, the price in the PPA is broken into two parts i.e. 9MW = 2MW + 7MW. For the 2MW part, the price that will be paid to the project in 2007-2008 is Rs 2.53/kWh. The price for the 7MW part for 2007-2008 will be Rs 2.71/kWh. Clearly, the export to the grid does not give good financial returns to the project proponent. Thus, there are significant barriers to the implementation of the proposed project activity due to the downward revision in tariff rates by the APERC. Another barrier is the order preventing sale of power generated by NCE sources to 3rd parties, as per the Regulatory Commission of Andhra Pradesh. Thus, the proposed project activity can only sell power to APTRANSCO and hence has to comply with their policies. |  |  |
| Date: 2nd May 2007[Nikunj Agarwal] - there are 6 plants selling their bagasse and burning solely coal as a fuel Are they using coal as a fuel in the boiler for cogeneration and exporting electricity to the grid. - three belong to the Thiru Arooran group, which developed a methodology (AM0007) specifically for bagasse based plants burning fossil fuels during the off-season, in association with the Prototype Carbon Fund and Winrock International India Please provide documentary evidence for the same. - The remaining three are small-scale projects in the co-operative sector Are these three going for CDM. - Thus, it is fair to say that the project activity is not common practice within the state. Do you want to say that only one sugar industries has gone for CDM and has been registered out of 12 sugar industries doing cogeneration. Biomass based plants registered as CDM in AP - there are 14 biomass based power plants registered as CDM What does it mean while in the upperlines you are saying that there are only one bagasse based cogeneration power plant. - http://www.srpc.kar.nic.in/AR_05-06/ANN6.pdf The same is not being seen by opening the link. Tariff related barriers - For the 2MW part, the price that will be paid to the project in 2007-2008 is Rs 2.53/kWh.Why there is variation in price of electricity and why PPA is for 2MW & 7 MW separately. - Clearly, the export to the grid does not give good financial returns to the project proponent How can you say export to the grid is not a good financial returns to the project proponent, is there any better option if you are not exporting the power to the grid. - Please provide the documentary evidence for another barrier is the order preventing sale of power generated by NCE sources to 3rd parties, as per the Regulatory Commission of Andhra Pradesh [Acceptance and close out] Open |  |  |
| Date: 22nd May 2007 [Response from project developer] Comment no 1: Yes, all the boilers within these 6 projects burn solely coal in their boilers. (This is probably because coal fired boilers are cheaper and easier to maintain as compared to co-fired boilers) Comment no 2: http://cdm.unfccc.int/UserManagement/FileStorage/CDMWF_AM_374220993 is the link to AM007, the meth proposed by Thiru Arooran grp and approved by UNFCCC. In case you could also refer to RSCL PDD http://cdm.unfccc.int/UserManagement/FileStorage/CDMWF_AM_374220993 where we mentioned this projects. Comment no 3: The three projects mentioned above are: - Subramaniya Siva Co-Op (1.2MW) - Sakthi Sugars Ltd (2MW) - MR Krishnamurthy Co-Op (7.5MW) As per our knowledge, none of these three projects have been undertaken/proposed as CDM. Comment no 4: Yes, amongst the 12 cogen plants, only 3 have undergone CDM, one of which is our project. Comment no 5: Biomass based plants differ from bagasse cogen plants. These projects are based on other biomass residues such as rice husk.. Comment no 6: I have checked the link and it works fine. Comment no 7: This is because the PPA if offering a lower price for the 2MW part as this is based on older price as explained during site visit and also explained in PDD. Comment no 8: The export to the grid is not a good financial return compared to the investments in the project activity because the price offered is low. The other alternative would have been to continue the current situation and not increase the power generation at the site. The increase in power generation is additional and the surplus is exported to the grid. This additional export has been undertaken only because the revenues will be increased by the inclusion of revenues from the sales of CERs. Comment no 9: Enclosed. APERC order 2004. |  |  |
| Date: 29th June 2007[Nikunj Agarwal] - The explanation given by project proponent is found satisfactory and hence this point was closed out. - The same has been checked with the links provided by the project proponent and found ok; hence this point can be closed out. - These three projects - Subramaniya Siva Co-Op (1.2MW) - Sakthi Sugars Ltd (2MW) - MR Krishnamurthy Co-Op (7.5MW) have been checked with UNFCCC websites and is not found on the website. Hence this point was closed out. - Due to this clarification we can say that project activity is not a common practice. Hence this point was closed out. - Since Biomass based plants differ from bagasse cogen plants and in fact these biomass projects are based on other biomass residues such as rice husk and not on bagasse hence the explanation given by the project proponent is satisfactory and this point was closed out, - The same has been checked with the links and hence this point was closed out. 7. The same has been explained in the PDD and hence this point was closed out. 8. The explanation is found satisfactory and hence this point was closed out. 9. APERC order 2004 has been checked for the barrier related to the sale of power generated and is found satisfactory, hence this point was closed out. Acceptance and close out] OK, Closed Out[Sanjeev Kumar] |  |  |

# 5.	Validation opinion

SGS has performed a validation of the project: “Nava Bharat RE Bagasse Project”, by Nava Bharat Ventures Ltd. The Validation was performed on the basis of the UNFCCC criteria and host country criteria, as well as criteria given to provide for consistent project operations, monitoring and reporting. Using a risk based approach, the review of the project design documentation and the subsequent follow-up interviews have provided SGS with sufficient evidence to determine the fulfilment of the stated criteria. In our opinion, the project meets all relevant UNFCCC requirements for the CDM and all relevant host country criteria. The project will hence be recommended by SGS for registration with the UNFCCC.

Using a risk based approach, the review of the project design documentation and the subsequent follow-up interviews have provided SGS with sufficient evidence to determine the fulfilment of the stated criteria. In our opinion, the project meets all relevant UNFCCC requirements for the CDM and all relevant host country criteria. The project will hence be recommended by SGS for registration with the UNFCCC.

SGS has received confirmation by the host Party that the project activity assists it in achieving sustainable development.

By utilizing the bagasse from the existing sugar manufacturing process for cogeneration of electricity, the project results in reductions of greenhouse gas emissions that are real, measurable and give long-term benefits to the mitigation of climate change. A review of the financial barrier supporting with Institutional barriers demonstrates that the proposed project activity is not a likely baseline scenario. Emission reductions attributable to the project are hence additional to any that would occur in the absence of the project activity.

The validation is based on the information made available to SGS and the engagement conditions detailed in the report. The validation has been performed using a risk based approach as described above. The only purpose of this report is its use during the registration process as part of the CDM project cycle. Hence SGS can not be held liable by any party for decisions made or not made based on the validation opinion, which will go beyond that purpose.

The DOE declares herewith that in undertaking the validation of this proposed CDM project activity it has no financial interest related to the proposed CDM project activity and that undertaking such a validation does not constitute a conflict of interest which is incompatible with the role of a DOE under the CDM.

# 6.	List of persons interviewed

| Date | Name | Position | Short description of subject discussed |
| --- | --- | --- | --- |
| 20/3/2007 | Mr. N. Prabhakar - Vice President | Project Proponent | About the description of the project. |
| 20/3/2007 | Mr. N. Subhash Chandra – Asst. General manager (Commercial) | Project Proponent | About the technology and additionality of the project. |
| 21/3/2007 | Mr. M. Ramakrishna – Manager – Power Plant | Project Proponent | About the technology of the project activity and operation and monitoring. |
| 21/3/2007 | Mr. Valluri Venkata Rao | Local Resident | Local Stake Holder Consultation |
| 21/3/2007 | Mr. Santosh Kumar Singh | Consultant | About the CARs & NIRs and additionality |

# Document references

Category 1 Documents (documents provided by the Client that relate directly to the GHG components of the project, (i.e. the CDM Project Design Document, confirmation by the host Party on contribution to sustainable development and written approval of voluntary participation from the designated national authority):

| /1/ | Letter of Approval from Indian DNA |
| --- | --- |
| /2/ /3/ | Letter of Approval from U.K. DNA PDD version 1 dated 29th December 2006 (Webhosted) |
| /4/ | PDD version 2 dated 27th March 2007 |
| /5/ /6/ /7/ 8 | PDD version 3 dated 25th July 2007 PDD version 4 dated 25th July 2007 PDD version 5 dated 8th August 2007 PDD version 6 dated 16th October 2007(Present) |
| /9/ /10/ | IRR Calculation Sheet Modalities of communication |

Category 2 Documents (background documents used to check project assumptions and confirm the validity of information given in the Category 1 documents and in validation interviews):

| /1/ | Board resolution document |
| --- | --- |
| /2/ | Proof of Additionality |
| /3/ | Local Stakeholder Consultation meeting minutes |
| /4/ | NOC from State Pollution Control Board |
| /5/ | Proof of project start date |
| /6/ | Supporting document ISHC |
| /7/ | Undertaking document for non-replacement of he implemented technology during the entire span of crediting period. |
| /8/ | Bagasse balance for 2005-2006 and 2006-2007. |
| /9/ | PPA between Andhra Pradesh State Electricity Board and Nava Bharat Ferro Alloys Limited for power evacuation to the state grid system. |
| /10/ | AMS. I.D. version 10 dated 23rd December 2006 |

Annex 1: Local Assessment

| CHECKLIST QUESTION | Ref. | MoV* | COMMENTS | Draft Concl | Final Concl |
| --- | --- | --- | --- | --- | --- |
| - Invitation for LSC meeting was sent to participate and communicate suggestions regarding the project activity. Documents are required to verify the same. | PDD G.1 |  | Public notice published in the local news paper for declaration and invitation to the LSC meeting has been validated and Letter of No Objection and appreciation provided by Gram Panchayat for NOC on project activity have been obtained to verify the transparency in consultation process. | Y | Y |
| - The regulatory approval (consent to establish and operate the project) from the State Pollution Control Board is required to verify that local/legal requirements have been met. |  |  | The copy of consent to operate letter from State Pollution Control Board has been obtained and checked with original consent. It was found to be OK | Y | Y |
| - Local stakeholders’ comments are required to be verified for any adverse comment. - MoM of stakeholder consultation meeting. | PDD G.1 and G.2 |  | Letter of No Objection and appreciation from Gram Panchayat have been obtained. There is no adverse comment found in the letter. MOM also received and found to be satisfying. | Y | Y |
| - It is required to be checked whether the project technology used is likely to be substituted by other or more efficient technologies within the project period. | PDD |  | Project proponent submitted an undertaking that the project activity will not be substituted by other or more efficient technologies within the entire project crediting period. | Y | Y |
| - MoM of board meeting in which CDM was considered for the project activity. To be verified during site visit. | PDD |  | Project proponent submitted the MOM of board meeting which were also verified by seeing the original copy and also interviewing the Executive President. | Y | Y |
| - Availability of Bagasse is to be checked, it should be 25% more than the rated consumption | PDD |  | Project Proponent Submitted the estimated bagasse balance for the year 2006-07 and also submitted the record of bagasse production for the last two years. | Y | Y |
| - It is required to be checked that if there are any possibility of bagasse import from outside the project boundary. | PDD |  | The Bagasse balance sheets for previous and current year have been checked and it was found to be satisfactory in terms of availability of surplus bagasse at the project site, thus nullifying the possibility of any leakage calculation. | Y | Y |
| - The possibility of anykind of project emission to be checked. | PDD |  | As the project is a biomass based co-generation project and through checking and interviewing the respective personnel, it has been found that the boiler operation does not involve combustion of any fossil fuel at any stage, thus nullifying the requirement of any project emission calculation, however the monitoring of the same has been included in the monitoring plan and any emission due to the fossil fuel will be deducted from the emission reduction. | Y | Y |
| - It is required to be checked that the Small scale project activity is not a debundled component of a larger project. | PDD |  | It has been checked during site visit, that the project activity is not a debundled component of a larger project. | Y | Y |
| - Is Project Proponent also using fossil fuel in the boiler during the initial start up. | PDD |  | It has been verified during the site visit by interviewing the boiler operator and the personnel in boiler house and along with that the bagasse consumption from the record and fuel steam ratio has been checked. However the monitoring of the same has been included in the monitoring plan and any emission due to the fossil fuel will be deducted from the emission reduction. | Y | Y |
| - Roles and responsibility for the implementation of monitoring plan is to be cross checked with the plant personnel. | PDD |  | The roles and responsibility in accordance with the monitoring plan is incorporated in the revised version of the PDD. | Y | Y |
| - Proof of additionality, i.e barrier due to the first of its kind in the entire State, to be checked | PDD |  | The documentary evidence of financial barrier supporting with Institutional barriers has been checked and all the facts and figures have been incorporated in the revised version of the PDD. | Y | Y |
| - EIA and NOC from the concerned authority for the project activity | PDD |  | The factory has a Environment management plan in place and prepares Environment audit reports and obtained NOC from State Pollution Control Board. | Y | Y |
| - Calculation spreadsheet for baseline and project emission reductions during project crediting period. | PDD |  | The excel spreadsheet for baseline and emission reduction calculation was obtained and checked for the relevant information in PDD. | Y | Y |
| - Is the assumed crediting time clearly defined and reasonable (renewable crediting period of max. two x 7 years or fixed crediting period of max. 10 years)? | S.V./PDD |  | Renewable Crediting period of 7 years is selected for the project activity. | Y | Y |

Annex 2 Validation Protocol

# Table 1	Participation Requirements for Clean Development Mechanism (CDM) Project Activities (Ref PDD, Letters of Approval and UNFCCC website)

| REQUIREMENT | MoV | Ref | Comment | Draft finding | Concl |
| --- | --- | --- | --- | --- | --- |
| - 1.1 The project shall assist Parties included in Annex I in achieving compliance with part of their emission reduction commitment under Art. 3 and be entered into voluntarily. | PDD | DR | The project activity is likely to contribute to sustainable development. Letter of approval from Host Country (U.K.) Designated National Authority (DNA) to be submitted by the project proponent | CAR 1 | Y |
| - 1.2 The project shall assist non-Annex I Parties in achieving sustainable development and shall have obtained confirmation by the host country thereof, and be entered into voluntarily. | PDD | DR | The project activity is likely to contribute to sustainable development. Letter of approval from Host Country (India) Designated National Authority (DNA) to be submitted by the project proponent | CAR 1 | Y |
| - 1.3 All Parties (listed in Section A3 of the PDD) have ratified the Kyoto protocol and are allowed to participate in CDM projects. | PDD/UNFCCC Web-site | DR/ UNFCCC Web-site | Project is bilateral and India has ratified the protocol on 26th August 2002 and is allowed to participate. The web link is http://unfccc.int/parties_and_observers/parties/items/2109.php U.K. has ratified the protocol on 31st May 2002 and is allowed to participate. The web link is http://maindb.unfccc.int/public/country.pl?country=GB | Y | Y |
| - 1.4 The project results in reductions of GHG emissions or increases in sequestration when compared to the baseline; and the project can be reasonably shown to be different from the baseline scenario. | PDD | DR/I | The purpose of the CDM project activity is to utilize the bagasse available in the sugar plant for effective generation of electricity for use in Nava Bharat sugar plant, and to supply to the Southern grid. This power generation using bagasse meets the energy demand and will replace same amount of electricity from the Southern regional grid which is dominated by fossil fuel based power plants. Thus the project activity will reduce the GHG emissions. | Y | Y |
| - 1.5 Parties, stakeholders and UNFCCC accredited NGOs shall have been invited to comment on the validation requirements for minimum 30 days (45 days for AR projects), and the project design document and comments have been made publicly available. | PDD | DR/UNFCCC Web-site | Yes, the project is listed on UNFCCC website from 22nd February 2007 to 23rd March 2007. http://cdm.unfccc.int/Projects/Validation/DB/DHX7F9F99KKAFI11A8RWXMN2IU33T2/view.html The project was also listed on SGS climate change website from 22nd February 2007 to 23rd March 2007. http://www.sgsqualitynetwork.com/tradeassurance/ccp/projects/project.php?id=216 Number of comments received - 1 | Y | Y |
| - 1.6 The project has correctly completed a Project Design Document, using the current version and exactly following the guidance. | PDD | DR | Project has used current version of PDD applicable and followed the guidelines, except pending closure of some CARs/ NIRs. | Site Visit | Y |
| - 1.7 The project shall not make use of Official Development Assistance (ODA), nor result in the diversion of such ODA. | PDD | DR | No ODA has identified in PDD. Records to be checked during Site visit. | Pending Site Visit | Y |
| - 1.8 For AR projects, the host country shall have issued a communication providing a single definition of minimum tree cover, minimum land area value and minimum tree height. Has such a letter been issued and are the definitions consistently applied throughout the PDD? | PDD | DR | Not relevant as the project is not an AR project. | Not Applicable | Y |
| - 1.9 Does the project meet the additional requirements detailed in: - Table 9 for SSC projects - Table 10 for AR projects - Table 11 for AR SSC projects | PDD | DR | This is an SSC project which comes under category AMS I-D and hence table 9 is applicable. | Y | Y |
| - 1.10 Is the current version of the PDD complete and does it clearly reflect all the information presented during the validation assessment? | PDD | DR | The version of PDD used by project proponent present all the information, except pending closure of some CARs/ NIRs. | Pending closure of CARs/ NIRs | Y |
| - 1.11 Does the PDD use accurate and reliable information that can be verified in an objective manner? | PDD | DR | The PDD uses reliable information and that can be verified in an objective manner. Parameters and default values used for calculation need to be checked. All the pending CAR/NIR need to be closed. | Site visit All pending CAR/NIR need to be closed. | Y |

# Table 7	COMMENTS BY LOCAL SAKEHOLDERS (Ref PDD Section G)

| CHECKLIST QUESTION | Ref. | MoV* | COMMENTS | Draft Concl | Final Concl |
| --- | --- | --- | --- | --- | --- |
| - 7.1 Have relevant stakeholders been consulted? | PDD | DR | The national stakeholder review mentioned in PDD is not clear is not complete. Please clarify which governmental and non-governmental parties are consulted for project activity. | CAR2 | Y |
| - Have appropriate media been used to invite comments by local stakeholders? | PDD | DR | According to the PDD the Project proponent placed an advertisement in paper, the same has to be checked during site visit. | Pending Site Visit | Y |
| - If a stakeholder consultation process is required by regulations/laws in the host country, has the stakeholder consultation process been carried out in accordance with such regulations/laws? | PDD | DR | Stakeholder consultation process is not required as per regulation/laws in host country. However the project participant has consulted the local stakeholders as a requirement for CDM project. MoM of the meeting is to be provided by the Project Proponent. | NIR3 | Y |
| - Is a summary of the stakeholder comments received provided? | PDD | DR | Summary of comments not mentioned in PDD. | NIR4 | Y |
| - Has due account been taken of any stakeholder comments received? | PDD | DR | No adverse comment identified in the PDD. Same has to be verified during site visit. | Site Visit | Y |

# Table 8	Other requirements

| CHECKLIST QUESTION | Ref. | MoV* | COMMENTS | Draft Concl | Final Concl |
| --- | --- | --- | --- | --- | --- |
| - 8.1 Project Design Document |  |  |  |  |  |
| 8.1.1 Editorial issues: does the project correctly apply the PDD template and has the document been completed without modifying/adding headings or logo, format or font. | PDD | DR | The PDD template for version 03 has been applied correctly. | OK | Y |
| 8.1.2 Substantive issues: does the PDD address all the specific requirements under each header. If requirements are not applicable / not relevant, this must be stated and justified. | PDD | DR | Yes, the PDD address all the specific requirements under each header. | Y | Y |
| 8.2 Technology to be Employed |  |  |  |  |  |
| - 8.2.1 Does the project design engineering reflect current good practices? | PDD | DR | The project reflects current good practice for project design engineering. | Site visit | Y |
| - 8.2.2 Does the project use state of the art technology or would the technology result in a significantly better performance than any commonly used technologies in the host country? | PDD | DR | The project uses the technology that would result in better performance. Technical specifications of the Turbines need to be checked during site visit and obtain the copy of the same. | Site visit | Y |
| - Is the project technology likely to be substituted by other or more efficient technologies within the project period? | PDD | DR | Proof for the technology will not be changed during the project period needs to be submitted by the project proponent. | NIR 5 | Y |
| - Does the project require extensive initial training and maintenance efforts in order to work as presumed during the project period? | PDD | DR | The PDD mentions that the individual supplier of the equipment will train the staff in charge at the Nava Bharat Sugar factory after commissioning to operate and maintain the equipment. Documentary evidence is required to support the claim. | NIR 6 | Y |
| - Duration of the Project/ Crediting Period |  |  |  |  |  |
| - Are the project’s starting date and operational lifetime clearly defined and reasonable? | PDD | DR | Project activity starting date is defined in the PDD as 12/08/2004. Evidence for the same is required. | CAR 7 | Y |
| 8.3.2 Is the assumed crediting time clearly defined and reasonable (renewable crediting period of max. two x 7 years or fixed crediting period of max. 10 years)? | PDD | DR | Fixed crediting period of 7 years is selected for the project activity and it is reasonable | Site Visit | Y . |
| - 8.3.3 Does the project’s operational lifetime exceed the crediting period. | PDD | DR | The project’s operational life time is expected to be 20 years which exceeds the crediting period of 7 years. | Y | Y |

# Table 9	Additional requirements for SSC project activities only

| CHECKLIST QUESTION | Ref. | MoV* | COMMENTS | Draft Concl | Final Concl |
| --- | --- | --- | --- | --- | --- |
| - Does the project qualify as a small scale CDM project activity as defined in paragraph 6 (c) of decision 17/CP.7 on the modalities and procedures for the CDM? | PDD | DR | The project activity is 9 MW bagasse based power plant and it qualify as a small scale CDM project activity as defined in paragraph 6 (c) of decision 17/CP.7 as the installed capacity of the power plant is less than 15 MW the limit set for the small scale projects. | Y | Y |
| - The project conforms to one of the categories listed in Appendix B to Annex II to Decision 21/CP8. | PDD | DR | Yes, the project activity falls under Type (i) D. Renewable energy projects-energy generation for a system | Y | Y |
| - The small scale project activity is not a debundled component of a larger project activity? d be entered into voluntarily 000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 | PDD | DR | Small scale project activity is not a debundled component of a larger project. | Y | Y |
| - PDD has been prepared in accordance with appendix A of Annex II to Decision 21/CP8 | PDD | DR | The CDM - SSC - PDD (version 3) template is followed. | Pending closure of CARs/ NIRs | Y |
| - The project uses a simplified baseline and monitoring methodology specified in Appendix B. If not, they may propose changes to the meths or a new SSC project category | PDD | DR | Yes, The project uses a simplified baseline and monitoring methodology specified in Appendix B. | Y | Y |
| - Are the emission reductions determined in accordance with the methodology described? | PDD | DR | Provide calculation spreadsheet for emission reduction calculation. (excel sheet) Provide OM, BM calculation sheet. Please provide the Proof for OM & BM data. | CAR 8 | Y |
| - Is there any bundling of SSC activities into one PDD? If so, does the monitoring plan consider sampling of activities? Refer to para 19 of Annex II. Also, note bundling provisions in SSC Briefing Note and SSC meths I C / I D and III D and Para 22e of Appendix B. | PDD | DR | There is no bundling of SSC activities into one PDD. | Y | Y |
| - Is EIA required by host party? If not, none is required irrespective of SHC. If yes, has one been performed consistent with local requirements? | PDD | DR | Information on EIA requirement is missing in the PDD. | NIR9 | Y |
| - The project results in emission reductions that are additional in accordance with the following requirements: y host Party to be y host Party. ted and biomass based power plant.by districts. The same to be confiermed 000000000000000000 - (Para 26) The project is additional if emissions are reduced below those in the absence of the project. - (Para 27) Simplified baseline can be used; if not, baseline proposed shall cover all gases, sectors and sources listed in Annex A to the KP - (Para 28) One or more barriers as detailed in attachment A to Appendix B to Annex II will be used to demonstrate that the project would not proceed without the CDM YI nt)?ou do this remotely over the internet (I have VPN p? ut to stop it from re-installing microsoft 000000000000000000000000 reductions that area additional in accordance with the following requirements: 7 accordingly. es that are listed on our website | PDD | DR | Provide Supporting Document for Additionality. - IRR Calculation Sheet is to be submitted. - PPA copy to be provided along with justification how it is acting as a barrier. - How Risks in the pricing of bagasse are a substantial barrier to the project activity, please justify | CAR 10 | Y |
| - Leakage is calculated according to the provisions of the SSC methodologies in Appendix B. | PDD | DR | Basis of calculation for leakage emission on account of transportation of biomass need to be provided. Evidence needs to be checked for surplus availability of bagasse in the region where the project activity is installed. | NIR 11 | Y |
| - The project boundary shall be constructed in accordance with the requirements of the SSC meths in Appendix B. | PDD | DR | Project boundary is not clear in PDD. | NIR 12 | Y |
| - The Monitoring plan shall be consistent with the requirements of the SSC methodology in Appendix B and shall provide for the collection and archiving of data needed to determine project emissions, baseline emissions and leakage. | PDD | DR | During boiler start up operation the quantity, quality and calorific value of the fossil fuel that will be fired is not clear in monitoring plan of PDD. Project Emission is taken as zero, please justify. | CAR 13 | Y |
| - The monitoring plan shall present good monitoring practice appropriate to the circumstances of the project activity. | PDD | DR | Monitoring plan for the present project activity dose not include following parameters 1. Quantity and quality of auxiliary fuel used 2. Calorific value of bagasse used 3. Calorific value of auxiliary fuel used 4. Leakage monitoring 5. Power imported from grid 6. No clarity on the use of main meter & check meter. PDD does not give number of meters available for measuring power generated, auxiliary consumption, import and export in PDD. | NIR 14 | Y |
| - If project activities are bundled, separate monitoring plan shall be prepared for each of the activities or an overall plan reflecting good monitoring practice will be prepared, consistent with the above requirements. | PDD | DR | The SSC project is not a bundled project activity. | Y | Y |

Annex 3

FINDINGS OVERVIEW

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 1 | CAR | Provide Host Country Approval from DNA. | 1.2 |
| Date: 27/3/2007 The project has received HCA from India and the same has been provided to DOE. The HCA is awaited from UK . |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Ok, host country approval is awaited. [Acceptance and close out]Open |  |  |  |
| Date: 15th May 2007 The host country has been send to the DOE for both U.K. and India. |  |  |  |
| Date: [2007-05-21] [Comments from Local Assessor] Letter of approval from Indian DNA (reference number 4/2/2007-CCC) dated 2nd April 2007 has been received and checked with the original copy and found satisfactorily, also the Letter of approval from U.K. DNA (reference number AL/38/2007) dated 2nd May 2007 has been received and checked with the original copy and found satisfactorily Hence CAR 01 can be closed out. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 2 | CAR | Please clarify which governmental and non-governmental parties are consulted for project activity. | 7.1 |
| Date: 27/3/2007 Governmental Parties: Indian DNA – Ministry of Environment and Forest State Pollution control Board Non-governmental parties Local Stakeholders which include local farmers supplying cane to the sugar factory General Public in the area by the publication of notice in the news paper |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] The same has been cross checked during stake holder consultation during site visit, hence CAR 2 was closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 3 | NIR | MoM of the meeting with Local stake holder consultation is to be provided by the Project Proponent. | 7.3 |
| Date: 27/3/2007 The minutes of the local stakeholder consultation have been provided during the validation and the same have been incorporated in the PDD. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] The same has been checked and verified at site visit during local stake holder consultation, this NIR can be closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 4 | NIR | Please provide the summary of comments received by stakeholder consultation. | 7.4 |
| Date: 27/3/2007 Main comments received are: Can the electricity be supplied round the year Can the electricity be made available to the local community Adverse effect on the farmers |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] The same has been incorporated in the revised PDD and has been verified during site visit, the NIR can be closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 5 | NIR | Provide evidence that project technology will not be substituted by other or more efficient technologies during the crediting period. | 8.2.3 |
| Date: 27/3/2007 Currently NBVL does not plan to substitute the technologies in the project by more efficient technologies. However, this will be checked during the annual verification. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Please provide the letter of undertaking for the same from the Project Participant. [Acceptance and close out]Open |  |  |  |
| Date: 11/4/2007 A letter will be provided from the project proponent stating the same. |  |  |  |
| Date: [2007-04-27] [Comments from Local Assessor] The letter of undertaking from project proponent has been obtained and checked and found satisfactory, hence this NIR can be closed. [Acceptance and close out]Ok, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 6 | NIR | Please provide the details of extensive initial training for the project Activity. | 8.2.4 |
| Date: 27/3/2007 The operators in the power plant have been provided necessary training for the successful operation. The details regarding this have been provided in the form of a document during the validation. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Training for the Project activity was provided and the document for training has been submitted to the validator, hence NIR6 can be closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 7 | CAR | Provide an evidence for the starting date of project activity. | 8.3.1 |
| Date: 27/3/2007 The starting date of the project activity has been given as the date in the letter of intent for the purchase of the TG set. A copy of this letter has been provided at the time of validation. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] The letter of intent for the purchase of the TG set has been submitted to the validator which was checked for the starting date of the project activity and 29/05/20004 was the date found on which the letter of intent for the purchase of TG set was issued. Hence 29/05/20004 was accepted as the starting date of the project activity and the same has been corrected in the revised PDD, hence this CAR was closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 8 | CAR | Provide calculation spreadsheet for grid emission factor calculation. Provide yearly emission reductions calculation spreadsheet for baseline and project emissions. Evidence to be provided for claim of reduction in carbon emissions from project activity. Provide OM, BM calculation sheet. | 9.6 |
| Date: 27/3/2007 These have been provided and sent to DOE by email. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Calculation spreadsheet for grid emission factor has been received and checked, Please clarify why the grid emission factor used by you is high as per CEA data, please use the grid emission factor as per CEA guidelines for the calculation of emission reductions. [Acceptance and close out]Open |  |  |  |
| Date: 11/4/2007 We have not used the CEF calculated by CEA due to the following reasons: There are many discrepancies within the CEF calculated by the CEA. For example, the emission factors for Gas/Lignite stations are calculated based on station heat rates, but there is no detailed outline on how these heat rates lead to emission factors. Again, the use of GCV to calculate emission factors deviates from ACM0002, in the sense that the meth asks for NCV to be used for such calculations. Another reason our emission factors for gas/lignite stations differ from CEA values is that we have calculated these based on annually available CEA data, whereas the CEA has obtained fuel consumption data individually (something not available on the website earlier). Another interesting discrepancy is the lack of consistency among CEA data. Often, the stations whose generation value goes up (based on CEA generation data) (capacity expansion) will not be listed among the stations with capacity expansion within the CEA report itself. The data calculated by Agrinergy has been accepted by validators earlier and it is based on CEA data, alongside data from individual station websites/regional grid websites. It follows the guidance within ACM0002 and does not deviate from expected results. We feel that the CEA has grossly underestimated CEFs. The Chinese DNA has calculated these emission factors and has values mostly greater than 1. The fact is, China has more hydro projects than India, which should lead to a higher CEF in India, if calculated correctly by the CEA. |  |  |  |
| Date: [2007-05-30] [Comments from Local Assessor] Project proponent provides the emission factor calculation sheet for baseline emission factor which was checked thoroughly by the DOE, but still DOE was not satisfied with the calculation so EF= 0.9266 tCO2/MWh calculated by project proponent is not accepted to DOE. Please provide further justification. [Acceptance and close out]Open |  |  |  |
| Date: 25/7/2007 The emission factor will be determined ex-post and for the purposes of our calculations an EF value 0.86 tCO2e/MWh is used. |  |  |  |
| Date: [2007-07-25] [Comments from Local Assessor] The emission factor will be determined ex-post and for the purpose of calculations emission factor value 0.86 tCO2e/MWh is used which is near to the value published by CEA, which is acceptable to DOE, hence this CAR was closed out. [Acceptance and close out]Ok, closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 9 | NIR | There is no information about EIA in the PDD. Provide a copy of consent to operate and establish from Andhra Pradesh State Pollution Control Board. | 9.8 |
| Date: 27/3/2007 The setting up of the project activity does not need EIA as no new boiler is installed. However the factory has an Environment monitoring plan and it prepares Environment Audit Report. The details regarding these have been given to the DOE during the validation |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Environment monitoring plan has been obtained from the project proponent, according to the Environmental Audit report there is no adverse effect of project activity on air, water etc. Please provide consent to operate for cogeneration plant from pollution control board. [Acceptance and close out]Open |  |  |  |
| Date: 11/4/2007 The consent to operate was provided during the site visit, however the same is sent by email with this report for your reference. |  |  |  |
| Date: [2007-04-27] [Comments from Local Assessor] The consent to operate has been submitted by the project proponent and the same has been found satisfactory, hence this NIR can be closed. [Acceptance and close out]Ok, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 10 | CAR | Provide Evidence for Additionality : - IRR Calculation Sheet is to be submitted. - PPA copy to be provided along with justification how it is acting as a barrier. - How Risks in the pricing of bagasse are a substantial barrier to the project activity, please justify | 9.9 |
| Date: 27/3/2007 The copies have been provided during validation site visit. However, the additionality has been further substantiated in terms of barriers to the project activity. This has been outlined in the PDD now and discussed during the site visit with Mr Chandra (NBVL). |  |  |  |
| Date: [] [Comments from Local Assessor] Please justify how PPA will act as a barrier. IRR calculation sheet is not transparent. Please justify. [Acceptance and close out]Open |  |  |  |
| Date: 11/4/2007 The barriers related to PPA have been detailed in section B.5 in the PDD. A copy of the PPA was also provided during the site visit. Revised excel sheet for IRR is attached. |  |  |  |
| Date: [2007-05-30] [Comments from Local Assessor] The barriers related to PPA are explained in the revised PDD in section B.5 IRR calculation sheet is checked and IRR calculation has been rephrased and found consistent; hence CAR 10 was closed out. [Acceptance and close out]Ok, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 11 | NIR | Evidence for leakage emissions need to check. Evidence needs to be checked for surplus availability of bagasse in the project activity region. | 9.10 |
| Date: 27/3/2007 As there is no transfer of equipments, the leakage is not considered in line with the methodology. The factory has a bagasse surplus. This can be demonstrated from the factory records during the annual verification. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] During site visit it has been verified that no leakage is to be considered. Previous records for the bagasse availability has been checked and verified that factory has surplus availability of bagasse and no need to purchase bagasse from outside, the NIR can be closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 12 | NIR | Project boundary is not clear in PDD. | 9.11 |
| Date: 27/3/2007 This has been mentioned in the PDD now. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] The project boundary is now clear in the revised PDD. Hence the NIR can be closed. [Acceptance and close out]OK, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 13 | CAR | During boiler start up operation the quantity, quality and calorific value of the fossil fuel that will be fired is not clear in monitoring plan of PDD. Project Emission is taken as zero, please justify. | 9.12 |
| Date: 27/3/2007 There is no use of fossil fuel in the boilers; therefore the parameters relating this have not been taken into consideration. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] No fossil fuel is used in the boilers; this has been verified during site visit, please provide the letter from boiler supplier that there is no provision for use of fossil fuel in the boiler, or include consumption of fossil fuel in the monitoring plan of the PDD. Please justify why the project emission is taken as zero. [Acceptance and close out]Open |  |  |  |
| Date: 25/7/2007 The project emissions have been taken as zero as there will not be any import of bagasse from outside and therefore emissions related to transport are ruled out. The consumption of fossil fuel if used will be monitored and is now included in the monitoring plan. |  |  |  |
| Date: [2007-07-25] [Comments from Local Assessor] The availability of bagasse for previous years has been checked and there was no need of import of bagasse from outside and therefore project emissions can be neglected, the use of fossil fuel and its calorific value will be monitored and is now included in the monitored plan of the PDD. Hence this CAR can be closed. [Acceptance and close out]Ok, Closed Out[Sanjeev Kumar] |  |  |  |

Date: 16th March 2007		Raised by: Nikunj Agarwal

| No. | Type | Issue | Ref |
| --- | --- | --- | --- |
| 14 | NIR | Monitoring plan for the present project activity dose not include following parameters 1. Quantity and quality of auxiliary fuel used 2. Calorific value of bagasse used 3. Calorific value of auxiliary fuel used 4. Leakage monitoring 5. Power imported from grid 6. No clarity on the use of main meter & check meter. PDD does not give number of meters available for measuring power generated, auxiliary consumption, import and export in PDD. | 9.13 |
| Date: 27/3/2007 - Only bagasse is fired in the boiler. - This is not required as a monitoring parameter but during annual energy balance this will be taken from the laboratory reports at the sugar factory. - Not applicable as no other type of fuel is fired in the boiler. - Leakage monitoring is not required as per AMS1D, as there is no transferring of equipment. If needed this can be checked during annual verification. - Power imports from the grid are part of the baseline and therefore not accounted for in the project activity. In the absence of project activity, the imports would still be there and therefore cannot be attributed to the project activity. |  |  |  |
| Date: [2007-04-02] [Comments from Local Assessor] Is there any D.G. running for the project activity. Please clarify Calorific value of the bagasse has been checked with lab analysis report. No other fuel other than bagasse is used in the boiler; the same has been cross check during site visit, the monitoring of any other fuel if used will be monitored and monitoring of the same is now included in monitoring plan of the PDD. No leakage is required as per the methodology AMS I D. Since the import of electricity was during the baseline also so no need of monitoring the import of electricity, please give the detail of meters in the PDD. [Acceptance and close out]Open |  |  |  |
| Date: 11/4/2007 Meter details have been provided in Annex 4 in the attached PDD. There is one DG set of 125kVA capacity which is operated only in emergencies/blackout. The emissions from the DG sets are not considered because DG set comes under the baseline as the emergency situation cannot be attributed to just the project activity. Further, the DG set is operated rarely and therefore emissions from this are not significant. |  |  |  |
| Date: [2007-04-27] [Comments from Local Assessor] Since the D.G Sets are operating during emergencies and the same has been verified during site visit and therefore the emissions from D.G. can be neglected, the meters details are incorporated in the revised PDD in Annex 4, so this NIR can be closed. [Acceptance and close out]Ok, Closed Out[Sanjeev Kumar] |  |  |  |

Annex 4

Statement of Competence

Name: Sanjeev Kumar				SGS Affiliate: SGS India Pvt. Ltd.

Status

-	Product Co-ordinator

-	Operations Co-ordinator

- Technical Reviewer

- Expert

Validation	      Verification

- 	Local Assessor

- Lead Assessor

- 	Assessor

/Trainee Lead Assessor

Scopes of Expertise

  - Energy Industries (renewable / non-renewable)

  - Energy Distribution

  - Energy Demand

  - Manufacturing

- Chemical Industry

- Construction

- Transport

- Mining/Mineral Production

- Metal Production

- Fugitive Emissions from Fuels (solid,oil and gas)

- Fugitive Emissions from Production and

Consumption of Halocarbons and Sulphur Hexafluoride

- Solvent Use

- Waste Handling and Disposal

- Afforestation and Reforestation

- Agriculture

Approved Member of Staff by Siddharth Yadav 	Date: 16th May 2007

Statement of Competence

Name: Nikunj Agarwal				SGS Affiliate: SGS India Pvt. Ltd.

Status

-	Product Co-ordinator

-	Operations Co-ordinator

- Technical Reviewer

- Expert

Validation	      Verification

- 	Local Assessor

- Lead Assessor

- 	Assessor

/ Trainee Lead Assessor

Scopes of Expertise

  - Energy Industries (renewable / non-renewable)

  - Energy Distribution

  - Energy Demand

  - Manufacturing

5    Chemical Industry

6    Construction

7     Transport

8     Mining/Mineral Production

9     Metal Production

10    Fugitive Emissions from Fuels (solid,oil and gas)

11    Fugitive Emissions from Production and

Consumption of Halocarbons and Sulphur Hexafluoride

- Solvent Use

- Waste Handling and Disposal

- Afforestation and Reforestation

- Agriculture

Approved Member of Staff by Marco van der Linden 	Date: 03-04-07

Statement of Competence

Name: Jochen Gross				SGS Affiliate: Germany

Status

-	Product Co-ordinator

-	Operations Co-ordinator

- Technical Reviewer

- Expert

Validation	      Verification

- 	Local Assessor

- Lead Assessor

- 	Assessor

/ Trainee Lead Assessor

Scopes of Expertise

  - Energy Industries (renewable / non-renewable)

  - Energy Distribution

  - Energy Demand

  - Manufacturing

- Chemical Industry

- Construction

- Transport

- Mining/Mineral Production

- Metal Production

- Fugitive Emissions from Fuels (solid,oil and gas)

- Fugitive Emissions from Production and

Consumption of Halocarbons and Sulphur Hexafluoride

- Solvent Use

- Waste Handling and Disposal

- Afforestation and Reforestation

- Agriculture

Approved Member of Staff by Siddharth Yadav 	Date: 16th May 2007

Invoices for selling of baggase.

“The economics of sugar in India are more complicated than those of sugar industries in many other countries. Both area and production of sugarcane fluctuate considerably from year to year. This is due to variations in climatic conditions, the vulnerability of areas cultivated under rainfed conditions, fluctuations in prices of gur and khandsari, and changes in returns from competing crops.” Source: FAO.

“List of Cane Sugar Factories and Distilleries, Season 2003-04”, published by the Sugar Technologists’ Association of India

http://cdm.unfccc.int/Projects/DB/SGS-UKL1146080365.67/view.html

http://cdm.unfccc.int/Projects/DB/SGS-UKL1154937278.97/view.html

http://www.srpc.kar.nic.in/AR_05-06/ANN6.pdf

http://www.srpc.kar.nic.in/january07/january07.pdf

http://www.ercap.org/OtherOrders/Order_RP_84_2003.doc