- TABLE FOR COMMENTS

- Name of submitter: ______Edwin Cogho_____________

- Affiliated organization of the submitter (if any): ____TASC___________

- Contact email of submitter: _____ edwin@tasc.je_______________

| 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| # | Para No./ Annex / Figure / Table | Line Number | Type of comment ge = general te = technical ed = editorial | Comment (including justification for change) | Proposed change (including proposed text) | Assessment of comment (to be completed by UNFCCC secretariat) |
| 1 | General | N/A | ge | Country Approval While we welcome the increased sophistication of fNRB estimation, the MoFuSS model presented is complex. Neither the model nor the data inputs have been adequately validated by Designated National Authorities (DNAs) due to the short timeframe provided for analysis. | The determination of the fNRB deserves critical scientific consensus before final values are released. The current 5-week review period provides little room for sufficient stakeholder engagement and inadequate time for DNAs to assess and provide comprehensive feedback on the input and results accuracy. We urge the CDM Executive Board to delay the implementation of the new fNRB estimates until a broader scientific consensus is achieved. This will ensure the integrity and accuracy of the environmental claims, aligning with ISO standard ISO 5725-1:1994, which emphasizes accuracy over conservativeness in scientific guidelines. |  |
| 2 | General | N/A | ge | Independent Validation We have concerns with respect to the MoFuSS tool’s use in the determination of the fNRB without independent validation or approval from a broad selection of experts in the biomass, forestry and geo-imaging industries. While the MoFuSS tool has undergone peer review, the data inputs for fNRB computations have not, driving wide variance between the latest submission and the October defaults. We note with concern that there is a limited availability of individuals or organizations with the required combination of statistical, computing and forestry expertise that this model and its outputs require to assess. | In the absence of a governing framework that can review and certify the outputs of the model in real time, we observe a risk in adoption of values as presented in its current iteration, but also in future iterations of the tool or the underlying definitions of fNRB following these consultations. We recommend that assumptions from global datasets are validated by ground truthed studies and approved by Host Country governments. |  |
| 3 | General | N/A | ge | Timeline for Validation The authors have referenced plans for external validation in the coming months. However, in our view the timelines and the funding for conducting such a review must be consistent with the respective workload, complexity, and aligned with the UNFCCC process, ensuring a broad participation of stakeholders. Further, after the external validation is completed, a proper process should be established to enable improvements in the model and the use of input data as per availability of reliable sources. It has also been mentioned that there will be an open-access cloud-based version of the model, which will allow anyone to run it for an area of interest (country, project area, etc) and adjust parameters, without needing to download software or understand the underlying code. However, the timeline provided by the UNFCCC does not seem to be appropriate for such a development, as there would be no time to have this online tool ready before the Meth Panel meeting in September. | We suggest to the CDM/UNFCCC that a clear process is established for validating the work before it gets approved as well as for managing future developments and updates. The process would be communicated to relevant stakeholders, with details including timelines, funding, and tools for the external validation/calibration of the latest version of MoFuSS model and its results. In particular, please advise on opportunities for stakeholder engagement on this issue after September 2024. What happens to this feedback when the transition from the CDM to the Article 6.4 SB happens? |  |
| 4 | General | N/A | ge | Validation of sub-national defaults The intention is that project developers can use an open-access tool to generate sub-national defaults. We welcome this, but question what framework will be provided to VVBs to validate the numbers generated by project developers based on project or sub-national boundaries. | Please develop guidelines for VVBs to validate MoFuSS derived sub-national or project fNRB values. |  |
| 5 | General | N/A | ge | Open Source & Replication It is mentioned in the report that “MoFuSS is an open-source freeware in constant development and that there is no restriction to access the code”. However, the current structure of the online repository means finding one’s way around the folders to find the correct scripts already takes a few days. Further, as a large model, MoFuSS requires large computing power, and each run takes a lot of time. As such, it will be very difficult to run the model and provide proper inputs during the time allocated for public inputs. | We suggest that UNFCCC allocates a longer review period particularly to allow for independent third-party testing of the model itself by a range of relevant stakeholders, rather than just a review of the published report and results thereof. |  |
| 6 | General | N/A | ge | Application As there are currently no interim solutions proposed to project developers (PDs) while the review and validation processes of the MoFuSS results are being completed, we anticipate that Project Developers will continue to use existing fNRB protocols. | Until the review and validation processes are completed, project developers will continue to use existing fNRB protocols. Any project developers who would like to voluntarily use the draft MoFuSS defaults should be allowed to do so, provided there is guidance for use of the standard deviation (SD) values, e.g., availability of evidence applicable to the project context could determine the use of upper or lower SDs. |  |
| 7 | General | N/A | ge | fNRB as a marginal variable Members of the PD Forum have been exploring a marginal approach to fNRB which we believe warrants further exploration. At present the MoFuSS model considers the non-renewability of the total harvest across a landscape, but we suggest that fNRB should instead consider the non-renewability of a reduction in harvest. This would bring cookstove carbon crediting in line with the emission reduction approach in energy efficiency projects where the methodology (e.g. AMS II.C) uses a marginal grid emission factor. The original definition of fNRB from 2012 required PDs to demonstrate a harvesting dynamic of increasing biomass scarcity (CDM SSC WG 35, Annex 20). This implicitly considered fNRB as a marginal variable. However, in 2017, fNRB was redefined as a fraction of all the wood stock available, without a corresponding update being made to the ER calculations to account for the new meaning. This definition change could explain the gap between the original CDM defaults (~90%) and the new defaults generated by MoFuSS (~30-50%). Further research is needed to explore the implications of a marginal fNRB. We understand that MoFuSS was designed to compare baseline and intervention scenarios, and that it is therefore well-suited to generating marginal defaults. We recommend that this work is commissioned, and that MoFuSS be used to generate marginal defaults, while addressing some of the concerns raised below. | We strongly recommend that the CDM EB assess the marginal approach to fNRB, with a view to bringing cookstove methodologies in line with the approach to grid emission factors in AMS II C. We recommend that this work is conducted as part of the review of these numbers, and before the conclusion of this workstream at the Executive Board in November. Funding should be provided for MoFuSS to be re-run in an ‘intervention’ scenario, for 2020-2030, based on a Paris-aligned clean cooking adoption curves. Only fNRB defaults that are calculated based on the delta between baseline and intervention scenario should be published. Baseline fNRBs should not be published, and the timeline for adopting new variables should be extended. The benefits of a more scientifically accurate approach to fNRB outweigh the delay. |  |
| 8 | General | N/A | ge | Results The current report does not provide any suitable reviewing advice for carbon credit buyers, developers and investors to illustrate that these numbers are effectively desk based figures that are subject to material changes when new inputs and assumptions are used (which are often generated from utilising local knowledge). Instead, the UNFCCC / the authors need to make clear that these figures can, and fundamentally should, change when the MoFuSS inputs and assumptions are updated with better and more contextual understanding of the underlying country and region. | As part of any ‘official’ communication of such figures there needs to be a disclaimer that these figures are derived from desk-based research and subject to material change when inputs and assumptions are revised to adhere to local knowledge. Ideally the report should also clearly list in a table where generic assumptions have been made so that the average carbon market participant, who is not an academic in nature, can better understand why there are likely to be material differences in model outputs when improved inputs are used. |  |
| 9 | General | N/A | ge | Local Data Inputs It is clear from reviewing the latest report that there are several local / national variations that need to be considered and researched to build an accurate understanding of fNRB values. For these numbers to become de facto defaults, we recommend the UNFCCC commissions local or regional studies to use localised inputs and assumptions for accurate fNRB values. Only once local inputs and assumptions have been used in the MoFuSS model should there be ‘default’ values approved by the UNFCCC. In the interim period existing fNRB protocols should continue to apply. | Local / national variations need to be included in the results before they become de facto default values. |  |
| 10 | 1. Executive Summary Para 1 | 1 | ge | Executive Summary The introductory paragraph notes that MoFuSS was “initially developed to estimate CO2 emission reductions from traditional woodfuel harvest and use, comparing business-as-usual with intervention scenarios” | Given the wide implications of the redefinition, we recommend that the UNFCCC consider further research into a marginal fNRB. We understand that this could be achieved with the current MoFuSS tool by comparing a baseline and implementation scenarios. |  |
| 11 | Table ES1 Para 3 | 3 | ge | Summarised Results We note with concern that standard deviations are high, bringing the accurateness of the model and values into question. | Please provide a clearer explanation for how project developers should interpret the high Standard Deviations. For example, availability of evidence applicable to the project context could determine the use of upper or lower SDs. In particular, please provide direction on the SD tolerances that are feasible for developers given that the tool can generate such wide SD ranges. |  |
| 12 | 1.4 Uncertainty Para 10 | 10 | te | Uncertainty The resulting standard deviations of the default values is a cause for concern in terms of the robustness of these results. The low number of simulations (30) while varying only one parameter seems to be too low to enable acceptable results. | We suggest running the simulation while varying all parameters simultaneously for a minimum of 1,000 times. |  |
| 13 | 1.5 Validation and next steps, Para 11-12 | 11 - 12 | ge | Validation and Next Steps It is stated by these paragraphs that the data/values have not yet been validated. Furthermore, the authors indicate that they will be conducting a series of validation studies in the coming year. We strongly recommend that the UNFCCC allows/commissions these validations to take place before the values are released. | Delay release of the fNRB results until they have been validated. We request clarification from the CDM that these are provisional estimates, and that further research is required to garner broader scientific consensus on the quantification approaches and definitions. |  |
| 14 | 2.14 Calculating fNRB, Para 14 | 14 | ge | Calculating fNRB The document defines that "real emission reductions are only attributable to the fraction of harvested wood that would not have regenerated naturally." However, the fraction of the forest that is included in the calculations should be adjusted to account for the probability that marginal forest areas are tapped first for fuelwood. | Allow the possibility to discount total forest area to account for marginal harvest |  |
| 15 | 2.1 What is fNRB?, Para 15 | 15 | ed | “CO2” should be “CO2” | Please be consistent throughout the document with chemical formulae. |  |
| 16 | 2.4 Reassessing fNRB, Para 20 - 25 | 20 - 25 | ge | Reassessing fNRB It is acknowledged that MoFuSS "requires some expertise to run," and it is well known that further development is required to enable PDs to replace default values with project-specific values. Default values are in many cases inaccurate and are derived from datasets that are "all 10 or more years old" per the document. So, it is important that PDs can assess accurate, ground-truthed values and implement them. | MoFuSS derived values should only be implemented after development work is complete and it is possible for PDs accurately define the inputs. |  |
| 17 | 2.4 Key assumptions in MoFuSS, para 22 | 22 | te | Key Assumptions in MoFuSS MoFuSS relies on several dozen parameters to model land cover change associated with woodfuel harvesting Based on our request to delay the publication of these numbers before strict validation by experts. We suggest the following modification | Values for fNRB provide preliminary results and will only be applicable for use upon a complete validation and verification of the data sets support the MoFuSS tool |  |
| 18 | 2.5 Biomass stocks, para 23 | 23 | te | Data Inputs: Biomass Stocks Global Forest Watch data is referenced several times as a source of information on forest change. It is not entirely clear the extent to which this data is relied on, yet analysis of a cluster of countries has raised a concern in relation to revised fNRB numbers. As an example, when reviewing GFW data for several countries from the database, it was noted that ‘annual tree cover loss by dominant driver’ between 2001-2023 is as follows for 6 countries within the EAC: Rwanda; 12%. DRC; 0.23%. Uganda; 2.8%. Tanzania; 0.64%. Burundi; 0.85%. Kenya; 3.6%. Whilst we understand that fNRB is calculated under a separate equation, from an overall assessment perspective, it is difficult to understand how Rwanda can have the lowest fNRB within this region yet has the highest rate of permanent deforestation occurring because of tree cover loss. | The comparison of Global Forest Watch data with regards to the relationship between overall tree cover loss and annual tree cover loss by dominant driver (resulting in permanent deforestation) suggests a reassessment of the fNRB value. This has been evidence for Rwanda and it is likely needed for other countries. |  |
| 19 | 2.5 Biomass stocks, Para 23 | 23 | te | Data Inputs: Biomass Stocks Globally calibrated remotely sensed products are usually measured with considerable error with forest / non-forest classifications especially in off the shelf products that are being used by the research team. Two specific papers that detail these can be found here: Kinnebrew, E., Ochoa-Brito, J. I., French, M., Mills-Novoa, M., Shoffner, E., & Siegel, K. (2022). Biases and limitations of Global Forest Change and author-generated land cover maps in detecting deforestation in the Amazon. PLOS ONE, 17(7), e0268970 Lindquist, E. J., & D’Annunzio, R. (2016). Assessing Global Forest Land-Use Change by Object-Based Image Analysis. Remote Sensing, 8(8), Article 8. https://doi.org/10.3390/rs8080678 | More detail is required about how the authors considered these well understood, and written about, data issues to calculate accurate, and reliable, fNRB numbers. |  |
| 20 | 2.5 Biomass stocks, Para 23 | 23 | te | Data Inputs: Biomass Stocks The Global Forest Watch data has particularly come under scrutiny. It requires a very high threshold of change: pixels are 30 meters in size, and a pixel is classified as "deforested" only when it has almost been entirely cleared. So, if half the pixel has been cleared, it does not measure as deforestation. As an example, the data classifies all of Rwanda's plantations as forest which means that whole 30 metre pixels will need to be cleared before it is deemed as deforested. Rwanda is unlikely to clear 30 metres in one specific pixel due to the land constraints and instead are likely to be deforesting across numerous pixels but rarely 100% within a specific pixel and as such deforestation in Rwanda is likely to be significantly understated. Therefore, it is not a surprise that the GFC suggests that Rwanda is "stable" when Government documents suggested a significantly growing deforestation problem with citizens struggling to access biomass for their fuel needs. | Country specific data and locally produced spatial maps are needed to calculate an accurate fNRB figure. |  |
| 21 | 2.5 Biomass stocks, Para 23 | 23 | te | Data Inputs: Biomass Stocks As per the above comment, how does this misclassification, that is inherent to global remotely sensed products, impact the outcomes. | Add an additional section in the report detailing how these misclassifications would impact the ‘default’ fNRB figure |  |
| 22 | 2.5 Biomass stocks, Para 24 | 24 | ge | Data Inputs: Biomass Stocks The data sets used to map above ground biomass are global models from 2010. We understand the need to train the predictive model, but recommend that simulated biomass stocks for 2010-2024 are validated with contemporary satellite data, and ground truthing studies. We note the authors aims to “calibrate our models to observed changes that occurred over leading up to 2010,” and recommend the same approach is applied to the time period up to the present. We strongly recommend undertaking this exercise and presenting the findings in the report to show alignment and/or discrepancies. Given the severity of loss in the forest cover due to unsustainable harvesting of wood fuels and deforestation predominantly in SSA regions in the last decade, It is obvious that the biomass stocks available in 2020 will substantially differ from 2010, therefore the resulted fNRB values from the MoFuSS study considering 2010 biomass stocks will have highest degree of uncertainty unless proven otherwise through calibrations of model by tracking the changes occurred in the past. | Please add a section on how the model calibration for biomass stocks were completed and add calibration plots. In particular, the report should include a cross comparison between 2020 data generated by the model and real observed 2020 biomass stocks. This can help validate the predictions from the model. We strongly recommend the researchers use to most recent biomass stock maps or alternatively if the NASA vintage maps is still used a validation process is a must. UNFCCC shall ensure the model is fully calibrated to garner wide acceptability from the carbon / scientific community. |  |
| 24 | 2.6 Biomass growth functions, Para 33 | 33 | te | Biomass Growth Functions It is stated that MoFuSS can simulate future tree cover loss that might be caused by drivers unrelated to woodfuel demand, such as agricultural expansion, but we do not predict future degradation. It is further mentioned that in areas that are not affected by future tree loss, the simulation allows trees to grow to their full potential unless they are affected by woodfuel harvesting. We believe that the model does not adequately predict future degradation as it does not capture the acceleration of forest loss due to extreme weather events and agricultural pressure of smallholder farmers. Evidence suggests that these drivers will become more important with climate change leading to more frequent loss of fertile agricultural land, thereby increasing the pressure on rural farmers to clear further forest spaces for agriculture. Furthermore, rapid population growth rates in many countries further exacerbates deforestation due to clearing of land to build dwellings. | We recommend these aspects be reviewed and a clarification is provided on whether the model considers the impact of agriculture not only as a primary driver of deforestation (conversion of forest lands) but also as an activity that is likely to be implemented on lands that were previously deforested and as such would prevent regeneration on such lands by occupying them, thus impacting the amount of natural generation in fNRB calculations. |  |
| 31 | Accounting for non-energy wood demand Para 2.9 | 41 & 42 | Te | Non-Energy wood demand The para #41 and #42 justifies the exclusions of non-energy wood demand for applications like building materials and timber exports citing the reasons such as non-availability of the forest plantation maps, minimal inaccuracies of non-consideration of non-energy biomass consumption etc. However, the research team has not clarified that how they ensured that exclusion of forestry plantations from the 2010 NASA data of Global above Aboveground and Belowground Biomass stocks given the challenges in the accessing the forest plantation maps. If in case the MoFuSS does not exclude the forest plantations in its initial biomass stocks, then the consumption of non-energy wood demand should also be considered in the MoFuSS model. Further para # 42 quotes an example of South Africa, wherein despite with mature forestry industries the plantations are about 2% of country’s total area. This perspective is deceptive, since the plantation area constitutes around ~18% of South Africa’s total forest area (Natural + Plantation). Also, there are countries in SSA such as Burundi, Uganda, Rwanda etc. having considerable share of forestry plantation. | We request clarification of the approach adopted for demarcation of natural forest and plantations in the 2010 NASA biomass map or else the non-energy wood demand to be accounted in the MoFuSS model. |  |
| 32 | 2.9 Accounting for non-energy wood demand and timber plantations, Para 43 | 43 | te | Residential Biomass Consumption Regionalized wood usage estimates are not appropriate in context where accurate government data exists. In addition, where project developers have submitted data from baseline KPTs, these should also be considered to avoid a standardised baseline biomass consumption figure being applied to all sub-Saharan Africa. | An additional round of published MoFuSS numbers is needed that must account for a combination of evidence from recent KPTs and other surveys, often commissioned by Governments themselves, at the individual country level when considering baseline biomass consumption by households. |  |
| 33 | 2.11 Quantifying household woodfuel consumption, Para 44, Table 3 | 44 | ed | Residential Biomass Consumption In the first row of the table, the Annual per capita woody biomass consumption unit is noted as kg. This should be in tonnes (t) | Change “Annual per capita consumption (kg)” to “Annual per capita consumption (t)” |  |
| 35 | 2.11 Quantifying household woodfuel consumption, Para 45 | 45 | te | Residential Biomass Consumption This paragraph references data submitted by the Project Developer Forum to both the UNFCCC methodology panel, and to the authors directly. This represented actual baseline biomass usage data determined by KPTs from 72 projects and 16 project developers. This data has not yet been included into the MoFuSS values. From one of the author’s analysis of the data of SSA, LAC and Asia all the actual KPT values are larger than the literature values used in the current study e.g. - SSA 0.71 vs. 0.40 - LAC 1.25 vs 1.10 - Asia 0.97 vs. 0.44 (East Asia) and 0.4 (South Asia)). It is clear that the actual ground truthed values differ greatly from the values used in the study and the current values could result in underestimation of the fNRB | Please clarify why the default baseline woody consumption values have remained low, below the level observed by the UN (Table 3) and by ongoing projects. |  |
| 41 | 3.1 Updated fNRB values for low- and middle-income countries, Para 80 | 80 | ge | Summarized Results Standard deviations are not only large for low fNRB values – in many of the cases the standard deviations are large for the larger values as well (Examples: Bangladesh fNRB = 39 StDev = 30; Malaysia fNRB = 34 StDev = 33). How can these values be accepted when the standard deviations are this large, especially in the cases of lower values with extreme standard deviations like Indonesia with and fNRB of 5 and a StDev of 100? Overall, the large standard deviations bring into question the validity of the model and default values. In all other aspects of cookstove and carbon projects standard deviations on parameters values like this will not be accepted by VVBs and Standard bodies. The same should apply for these values. | Please provide a clearer explanation for the high Standard Deviations for the modelling and the value in terms of accuracy of using these numbers in our estimates. |  |
| 42 | 3.1 Updated fNRB values for low- and middle-income countries, Table 5 | 82 | ge | Results The latest proposed fNRB numbers have drastically different ratio than the previous CDM defaults as well as the Q3 2023 MoFuSS output. | These variations in the computation and final default fNRB values highlights the need for further and broader scientific engagement before any determinations on the matter are concluded. |  |
| 43 | 3.4 How sensitive are MoFuSS fNRB results to input parameters?, Para 90 | 90 | te | Sensitivity As part of this update, five simulations were run while varying respective input parameters to determine their impact on the fNRB results. However, no simulation where woody consumption was varied was conducted, which would have been a good opportunity to see how much consumption affects the model outputs. For instance, it is stated in Paragraph 12 of Appendix 3 that there are several reasons for differences between fNRB values generated by WISDOM (2015) and MoFuSS. And that “while the underlying concepts of the WISDOM and MoFuSS models are similar, the input data vary substantially. For example, […], our estimates of woodfuel consumption are only moderately correlated with the estimates from the 2015 [WISDOM] study.” This potentially indicates that consumption data can have a significant impact on the fNRB results and consequently variation in this parameter should properly be considered. | We suggest that simulations where woody consumption is varied are also run (see related comment below), since this is one of the most important variables for the estimation of fNRB. |  |
| 44 | 3.4 How sensitive are MoFuSS fNRB results to input parameters?, Para 92 | 92 | ge | Sensitivity “Regarding the second factor, we are planning to improve MoFuSS to better accommodate the errors inherent in large spatial AGB maps; however, this is still a work in progress and was not prepared for this assessment”. The comment indicates that the MoFuSS values are still in preparation and not yet final. If this is going to be amended/changed what effect will that have on the fNRB values? A large affect would call into question the validity of the current values. | We recommend the UNFCC provides further funding to finalise the validation before the numbers are finalised. Provide more funding and time to the MoFuSS authors to complete the study and submit the most accurate and up to date values for public consultation. |  |
| 45 | 3.7 Addressing large differences between Oct 2023 and the current release, Para 105 | 105 | ge | Kenya It is stated that woodfuel demand in Kenya is projected to decrease between now and 2030. A study published in 2020 in Biomass and Bioenergy indicated that the woodfuel demand in Kenya was projected to increase from 26 million m3 to 40 million m3 per annum from 2007 to 2020 with an estimated supply of 31 million m3/year. Currently it is estimated that the demand in Kenya is 41.7 million m3. Some studies suggest that the demand for biomass energy will rise by 40% by 2040 in SSA. This all points to the demand for woodfuel increasing. Thus, on what basis was it established that the demand would decrease? | Substantiate how it was determined that woodfuel demand would decrease in Kenya, as it is clear from literature that it is not the case. |  |
| 46 | Appendix 2 Figure 22 | Figure 22 | te | Simulated deforestation patterns Here the authors acknowledge that “MoFuSS pattens result in unrealistic given the coarse resolution used in the study” The implications of the difference between deforestation predicted by MoFuSS and those that are observed is not clear. Is the implication that MoFuSS’s predictive capabilities are insufficient, or that the model needs to run at a higher resolution? | Please provide additional explanation of the causes and implications |  |
| 47 | Appendix 3 Tool 30 Revisions | Paragraph 4 | Ge | The document proposes changes to Tool30 in the Results section, yet the 4C CLEAR Methodology, which we understand will become the methodology for Article 6.4, scraps Tool30 altogether. | If the document is recommending use of Tool30, with MoFuSS used to calculate the inputs, state clearly that this is the case. If Tool30 is no longer recommended, state this clearlyBio |  |

https://doi.org/10.1016/j.biombioe.2020.105519

MECS Brief 001 -2022, Thie Biomass Challenge in Kenya

Smith, H., Jones, D., Vollmer, F., Baumert, S., Ryan, C., Woollen, E., Lisboa, S., Carvalho, M., Fisher, J., Luz, A., Grundy, I. & Patenaude, G. (2019). Urban energy transitions and rural income generation: Sustainable opportunities for rural development through charcoal production. World Development. 113: 237-245.