Inputs for “Top-down development of Standardized approaches for rural energy Supply (biogas)”

(a) Are the proposed specific default values for biogas generation reasonable and conservative?

The default values proposed here based on the study are deemed to be conservative.

On the other hand, the default value provides rather conservative emission reductions than what we know base on our knowledge at least for the case of China.

As shown in the attached spreadsheet, the baseline emissions would be 1.14 ton CO2/yr using the default value rather than 1.76 ton CO2/yr per household based on the Standards of Chinese Government, i.e., 35% lower.  Based on another calculation using the fossil fuel consumption, it is estimated to be 3.35 tCO2/yr, i.e., around 1/3 of that of the default value.  Therefore, we consider that the default value would be too conservative...  I hope you to apply the default value to other registered CDM projects/PoAs.

The default values can be extended to biogas digesters fed with poultry litters. Because the values are also believed to be conservative values for feeding of poultry litters as poultry litters has the higher B0 (0.24 m3 CH4/kg VS: Table 10A-9, IPCC 2006 Volume 4 chapter 10) than that of cattle and buffalo.

The values also can be extended to regions/countries where climate zone is temperate/cold (temperature 15-20) with some conditions like existence of heating measures for biogas digester such as a biogas digester in a greenhouse. This kind of measure has been widely practiced in China.

Moreover, the unit for the default value should be m3/m3.day rather than m3/m3.

(b) What kind of additional conditions or monitoring requirements should be included in the methodology to use the default values if any?

The study shows that the biogas generation rate depends mainly on the operation practice. Therefore, as mentioned in the study, the practical monitoring requirements should be to ensure that the biogas digesters are still operating.

(c) Are there other appropriate approaches for standardization? If any, please recommend providing justification on the proposed approach(es).

The efficiency of the conventional cookstove is better to be provided. In the above calculation, it is set as 0.2 as a conservative value.  The result is sensitive to this value.

It is also useful to provide per household coal consumption in the coal use areas as you try to provide that of the woody biomass.