PSB0009 Submission Grid emission factor of Sao Tome and Principe: Findings and resolution

| CL No. | Clarification (CL) | Reference to general provisions of guidelines on quality assurance and quality control of data used for sector-specific standardized baselines | Responses and corrective actions of DNA | Conclusion |
| --- | --- | --- | --- | --- |
| 1 | As per Table 3 “Emission Factor (tCO2/MWh) and CO2 emissions of each power plant”, emission factor of Bobo Forro 1 in 2010 is 3.3240 (tCO2/MWh), while emission factors of other diesel power plants are around 0.6 to 0.7 (tCO2/MWh). Please check the raw data and provide additional explanation. | Documentation provisions, data quality for use of secondary data, data traceability paragraph 11 (a) to (k) of the QA/QC Guidelines | The values used for estimating the emission factor of Bobo Forro 1 in 2010 can be found in the following document: Empresa de Água e Electricidade, Government of Sao Tome and Principe, Relatório e Contas – Exercício de 2009, 2010 The document – EMAE’s Annual Report for 2010 – has been duly audited as per the information gathered by the author. The author suggests using Option A2 of the Simple OM method to estimate the emission factor of the power plant in question, as a conservative approach. The first power units of Bobo Forro 1 date back to 2008. These were, namely: - MGO SACM - GESAN 1 - GESAN 2 - Caterpillar 1 - Caterpillar 2 The electricity supply from Bobo Forro 1 in 2010 came from one or more of these units. All of these units are no longer supplying electricity to the grid, and have been replaced by the group of 7 power units, whose installation started in 2011. As per the annual reports, the latter started to supply electricity to the grid in 2012. In 2011, no electricity was supplied from Bobo Forro 1. Applying Option A2 of the Simple OM method, we would obtain an emission factor of: EF, el = (0.0726 (tCO2/GJ) x 3.6 ) / 0.395 (-) = 0.6617 tCO2/MWh The default efficiency factor for the power plant is assumed to be: Oil > Open Cycle > New units (after 2000) > 39.5% | Closed |
| 2 | The calculation of the grid emission factor relies on the primary data from the Water and Electricity Company. A confirmation letter from the Water and Electricity Company indicating the power plants used for calculation and the authenticity of the data needs to be provided. | Documentation provisions, data quality for use of secondary data, data traceability paragraph 11 (a) to (k) of the QA/QC Guidelines | A confirmation letter from EMAE indicating the power plants used for the calculation and the authenticity of the data is submitted. | Closed |
| 3 (New on 19 Sep. 2014) | Please elaborate on the public consultation process conducted for the proposed standardized baseline (e.g. when, how, with whom, issues were raised, ways these issues have been addressed etc) | Transparency as per paragraph 15 (j) of the QA/QC guidelines: DNAs are expected to conduct a public consultation on the matters related to proposed standardized baselines and prepare a report that includes: (i) the objectives of public consultation; (ii) processes followed for public consultation; (iii) participants who attended the public consultation; (iv) a summary of the comments provided by stakeholders/experts; and (v) how comments were taken into account. | A QA/QC report has been developed in line with version 2.0 of the Guideline “Quality assurance and quality control of data used in the establishment of standardized baselines” and submitted together with this response. Please see file “Sao Tome_GEF SBL_QA-QC report_OCT 2015”. The QA/QC report address – inter alia - the issue of how transparency was ensured during the SBL development. |  |
| 4 (New on 19 Sep. 2014) | Table 1 “List of power plants connected to the main electricity system of the island of Sao Tome” indicates “Y” or “N” for the column “Connected”. Please clarify the meaning of “Connected” and also explain how it is considered in the calculation of OM. In particular, the following data is not clear: - “N” is shown for Bobô-Fôrro 1 in 2010, but the electricity generation data (192 MWh) in 2010 is used for OM calculation. - “Y” is shown for S. Tomé Pielstick in 2010 and “N” for the same unit in 2011 and 2012. However, it is not clear whether the 2010 data was considered in OM calculation and 2011-2012 was not considered. | Conservativeness as per paragraph 15 (h) of the QA/QC guidelines. | Few months after the receipt of this second request for clarification, electricity generation and fuel consumption data for the years 2013 and 2014 became available. The DNA, in consultation with the Regional Collaboration Centre of Lome’, decided to update the GEF calculation using 2012-2014 vintage data, which is in line with the Standard “Determining coverage of data and validity of standardized baselines” requiring the use of the most recent data available. The updated recalculation is submitted together with this response, including GEF calculation report and supporting evidence. Please see the following attached files: - GEF2012-2014_STP_updated.xlsx - Sao Tome_GEF SBL_calculation report_OCT 2015.pdf - EMAE RAPPORT 2010_excerto.pdf - EMAE RAPPORT 2011_excerto.pdf - EMAE RAPPORT 2012_excerto.pdf - EMAE RAPPORT 2013_excerto.pdf - EMAE RAPPORT 2014_excerto.pdf In occasion of the recalculation, and after consultation with EMAE, it has been clarified which plants are connected to the grid. Please see Table 1 in the file “Sao Tome_GEF SBL_calculation report_OCT 2015.pdf” and the worksheet “Electricity system” in the excel file “GEF2012-2014_STP_updated.xlsx”. |  |
| 5 (New on 19 Sep. 2014) | According to Table 2 “Table 2. Disaggregated data for all power units connected to the main electricity system of the island of São Tomé in São Tomé and Príncipe in 2012” of the spreadsheet, all five units of Santo Amaro plant (i.e. HIMSEN # 1 to # 5) have the same commissioning date. However, HIMSEN # 2 was included for BM calculation without any justification, although inclusion of any other units could also results in exceeding 20%. Inclusion of HIMSEN #5 which has the lowest emission factor seems to be most conservative. Please explain. | Conservativeness as per paragraph 15 (h) of the QA/QC guidelines. | As explained above, the DNA, in consultation with the Regional Collaboration Centre of Lome’, decided to update the GEF calculation using 2012-2014 vintage data. The calculation of the BM EF in the updated calculation considers Bobo Forro I and Santo Amaro plants as one single power unit, which is in line with the Tool requirements being made of several identical power units (i.e. with the same capacity, age and efficiency). Together, contributes to over 68% of electricity generation in 2014. That is because no disaggregated data of fuel consumption per unit is available. Therefore the update calculation indirectly addresses the issue raised. |  |